A  WINDOWS®  RALLY™  SPECIFICATION

PnP-X: Plug and Play Extensions for Windows

Abstract

This specification describes the Plug and Play Extensions (PnP-X) for the Windows Vista™ and Microsoft® Windows Server™ Code Name “Longhorn” operating systems. It provides requirements and guidelines for hardware manufacturers to create devices that are installable with PnP-X.

PnP-X is a key component of the Microsoft Windows® Rally™ set of technologies.

Version 1.0c – January 11, 2007

LICENSE NOTICE. Access to and viewing and implementation of the technology described in this document is granted under the Microsoft Windows Rally Program License Agreement (“License Agreement”). If you want a license from Microsoft to access, view or implement one or more Licensed Technologies, you must complete the designated information in the License Agreement and return a signed copy to Microsoft. The License Agreement is provided at the end of this document. If the License Agreement is not available with this document, you can download a copy from the Windows Rally Web site at http://www.microsoft.com/rally.

Disclaimer

This is a preliminary document and may be changed substantially prior to final commercial release of the software described herein.

The information contained in this document represents the current view of Microsoft Corporation on the issues discussed as of the date of publication. Because Microsoft must respond to changing market conditions, it should not be interpreted to be a commitment on the part of Microsoft, and Microsoft cannot guarantee the accuracy of any information presented after the date of publication.

This White Paper is for informational purposes only. MICROSOFT MAKES NO WARRANTIES, EXPRESS, IMPLIED OR STATUTORY, AS TO THE INFORMATION IN THIS DOCUMENT.

Complying with all applicable copyright laws is the responsibility of the user. Without limiting the rights under copyright, no part of this document may be reproduced, stored in or introduced into a retrieval system, or transmitted in any form or by any means (electronic, mechanical, photocopying, recording, or otherwise), or for any purpose, without the express written permission of Microsoft Corporation.

Microsoft may have patents, patent applications, trademarks, copyrights, or other intellectual property rights covering subject matter in this document. Except as expressly provided in any written license agreement from Microsoft, the furnishing of this document does not give you any license to these patents, trademarks, copyrights, or other intellectual property.

Unless otherwise noted, the example companies, organizations, products, domain names, e-mail addresses, logos, people, places and events depicted herein are fictitious, and no association with any real company, organization, product, domain name, email address, logo, person, place or event is intended or should be inferred.

© 2006-2007 Microsoft Corporation. All rights reserved.

Microsoft, Rally, Windows, Windows Mobile, Windows Server, and Windows Vista are either registered trademarks or trademarks of Microsoft Corporation in the United States and/or other countries.

The names of actual companies and products mentioned herein may be the trademarks of their respective owners.

The current version of this specification is maintained on the Web at: 
http://www.microsoft.com/rally

Revision History

| Date | Revision |
| --- | --- |
| 2005 | Portions previously published as “Introduction to PnP-X” |
| May 2006 | Revision as part of specifications for Windows Rally technologies. |
| July 2006 | Additional service detail. |
| December 2006 | Corrected WSD metadata elements. Added Network Explorer device discovery information. |
| January 2007 | Corrected SSDP and WSD namespace references. |

Contents

Introduction to PnP-X	4

PnP-X Services	4

Default Startup Type for PnP-X Services	5

Windows Vista	5

Windows Server	5

Network Explorer Device Discovery and PnP-X	5

Windows Vista	5

Windows Server	5

WSD Device Installation Process	5

Using PnP-X	6

Creating a PnP-X Device	6

Creating a WSD-Compliant Device	7

WSD Device Metadata Requirements	7

Sample INF File for WSD Devices	9

Sample Metadata Message for WSD Devices	11

Creating a SSDP-Compliant Device	12

SSDP Device Metadata Requirements	13

Sample SSDP Discovery Response Message	14

Sample INF File for SSDP Devices	15

Sample Device Description File for SSDP Devices	16

Querying Device Metadata	17

PnP-X Reference	18

PnP-X Category Definitions	18

pnpx-ssdp Schema Reference	20

pnpx-ssdp Schema Elements	20

X_compatibleId (X_PnpXType) Element	20

X_deviceCategory (X_PnpXType) Element	20

X_hardwareId (X_PnpXType) Element	21

X_PnpX Element	21

pnpx-ssdp Schema Complex Types	21

X_PnpXType Complex Type	22

pnpx-wsd Schema Reference	22

pnpx-wsd Schema Elements	23

CompatibleId Element	23

DeviceCategory Element	23

HardwareId Element	24

pnpx-wsd Schema Simple Types	24

CompatibleIdType Simple Type	24

DeviceCategoryType Simple Type	24

HardwareIdType Simple Type	25

References	25

# Introduction to PnP-X

This guide provides information about the Plug and Play Extensions (PnP-X) component of the Microsoft® Windows® Rally™ set of technologies. This document provides guidelines for creating devices that are installable with PnP-X for Windows Vista™.

PnP-X is an extension of Plug and Play (PnP) in Windows Vista that integrates network-connected devices into the Windows PnP subsystem. PnP-X allows network-connected devices to appear as devices inside Windows and provides an installation experience that is similar to attaching a physically connected device.

Just as PnP in Windows operates over a specific set of buses, PnP-X in Windows Vista has been defined to operate over network protocols for virtually connected devices. This includes:

- Universal Plug and Play (UPnP)–capable devices

- Device Profile for Web Services (DPWS))–capable devices (see http://go.microsoft.com/fwlink/?LinkID=59069)

For more information about Windows Rally technologies and other resources discussed here, see http://www.microsoft.com/rally.

PnP-X discovers devices attached to a virtual PnP-X network bus. The IP bus enumerator is used to discover PnP-X devices. This enumerator discovers DPWS-capable devices and UPnP devices connected to a LAN.

Device discovery takes place when Network Explorer is launched. The user must explicitly choose to install the device. The Found New Hardware Wizard appears if user intervention is required to install a device driver.

Devices appear in Device Manager after discovery and installation. The devices can then be enumerated for use by other Windows applications by calling the Function Discovery API to query the PnP subsystem. Each device is associated with a function instance. These function instances can be queried by using the IFunctionDiscovery::CreateInstanceQuery and the IFunctionDiscovery::CreateInstanceCollectionQuery methods.

See also on the MSDN Web site:

- Function Discovery

## PnP-X Services

PnP-X uses two services to enable device discovery: PnP installation and device presence management. These services are the following:

- IP Bus Enumerator. The IP Bus Enumerator (IPBusEnum) is a Windows service that handles the installation of PnP-X devices.

IPBusEnum monitors a list of devices currently present on the system. When a device is installed, IPBusEnum sends a request to the PnP component to create the device devnode. When a device is removed, IPBusEnum receives a notification message from the PnP component and then sends a request to remove the device from the list of present devices.

PnP-X devices can be installed only when the IPBusEnum service is running.

- Function Discovery Provider Host. The Function Discovery Provider Host (FDPHost) service hosts the discovery providers for PnP-X. This includes the built-in discovery providers for the Simple Services Discovery Protocol (SSDP) and Web Services on Devices (WSD) WS-Discovery protocol.

## Default Startup Type for PnP-X Services

The default start type for the PnP-X services differs for the Windows client and server operating systems.

#### Windows Vista

The IPBusEnum and FDPHost services are set to Manual start on the Windows Vista client operating systems. These services will start and stop automatically as the operating system needs them.

#### Windows Server

The IPBusEnum service is set to Disabled on the Microsoft Windows Server® operating systems. Server administrators wanting to enable PnP-X on Windows Server must start the IPBusEnum service. This can be done through the Services control panel application or on the command line via the following:

C:\>net start IPBusEnum

The FDPHost service is set to Manual start on the Windows Server operating systems. This service starts and stops automatically as the operating system needs it.

## Network Explorer Device Discovery and PnP-X

Network Explorer displays the list of discovered devices to the user. Which devices are discovered by default differs based on whether the operating system is Windows Vista or Windows Server.

#### Windows Vista

In Windows Vista, both WSD- and UPnP-based PnP-X devices are discovered and displayed in Network Explorer.

#### Windows Server

In Windows Server, only WSD-based PnP-X devices are discovered and displayed in Network Explorer. The UPnP discovery service (SSDPSrv) does not run by default in Windows Server. Server administrators wanting to enable discovery of UPnP-based PnP-X devices must start the SSDPSrv service. This can be done through the Control Panel Services application or on the command line via the following:

C:\>net start SSDPSrv

## WSD Device Installation Process

The following procedure describes the WSD device installation process:

1.	The user launches the Network Explorer.

2.	The Network Explorer queries the WSD Provider (and other providers) for a list of devices and displays the devices to the user.

3.	The user chooses to install the device.

4.	The IPBusEnum service is notified of the new device installation request and queries the WSD Provider for device metadata.

5.	The WSD Provider queries the device for metadata.

6.	The device sends a metadata message to the WSD Provider in response.

7.	The WSD Provider parses the metadata message and returns the results to IPBusEnum.

8.	Based on the metadata, IPBusEnum creates a physical device object (PDO) for the device and passes the PDO to the PnP component.

9.	PnP searches for the INF file associated with the device.

10.	Based on the data in the INF file, PnP attempts to select a compatible device driver.

11.	If a compatible device driver is available on the system, the driver is loaded and the device is installed. Otherwise, the Found New Hardware Wizard appears and the user must locate and install the driver manually.

See also in this document:

- Creating a WSD-Compliant Device

# Using PnP-X

Independent hardware vendors (IHVs) can create devices that are compatible with the PnP-X architecture.

Application developers can query PnP-X device metadata by using the Function Discovery API. For more information, see “Querying Device Metadata” later in this section

## Creating a PnP-X Device

A PnP-X device is a device that can be discovered by one of the following Function Discovery providers:

- The WSD Provider

- The SSDP Provider

- A custom provider developed to discover the PnP-X device

In most cases, the built-in WSD and SSDP providers can be used for device discovery, which simplifies development. If the device communicates by using a custom protocol, then a custom provider is required. For more information about writing a custom provider, see “Function Discovery Providers” at MSDN (http://msdn.microsoft.com).

The Web Services protocol offers two advantages. First, the WSD provider can discover devices on multiple subnets. Second, the WS device description document has more elements for a more detailed device description.

### Creating a WSD-Compliant Device

Device manufacturers must implement the following, in addition to base device functionality:

- Device Profile for Web Services (DPWS) support. To view the DPWS specification, see http://go.microsoft.com/fwlink/?LinkID=59069.

- pnpx-wsd schema support. Specifically, the device must read metadata from a SOAP message containing pnpx-wsd schema elements and create a set of hardware IDs and compatible IDs from the XML.

The device must send SOAP messages containing device metadata. The required elements are described in “WSD Device Metadata Requirements” in this document. This file includes both WSD and pnpx-wsd schema elements. The device must implement all WSD messages as described in the DPWS specification. These messages are required for device discovery.

WSD devices can be installed only if the HWID or Compatible ID specified in the device's INF file begins with the prefix UMB\. For a correctly formatted HWID, see “Sample INF File for WSD Devices” in this document.

WSD devices are installed silently (without prompting the user) if a signed driver package is installed on the computer performing device discovery. If no signed driver package is available, the user is prompted to install the device.

See also in this document:

- pnpx-wsd Schema Reference

#### WSD Device Metadata Requirements

Metadata is supplied in a metadata message. This message is a SOAP message that is sent by a WSD device that contains the DPWS elements. These elements represent the device metadata. The WSD Provider reads the metadata from the message. Metadata is stored in property keys (PKEYs), and can be queried as described in “Querying Device Metadata” in this document. For an example, see “Sample Metadata Message for WSD Devices” in this document.

For a complete description of the metadata elements listed below, see the DPWS specification at http://go.microsoft.com/fwlink/?LinkID=59069. The PKEY_PNPX_* property keys are described in “Key Definitions” on MSDN. Other property keys are defined in propkey.h.

The following table shows the namespace prefixes that are used by metadata elements.

| Prefix | Namespace |
| --- | --- |
| wsa | http://schemas.xmlsoap.org/ws/2004/08/addressing |
| wsd | http://schemas.xmlsoap.org/ws/2006/02/devprof |
| wsdisco | http://schemas.xmlsoap.org/ws/2005/04/discovery |
| pnpx | http://schemas.microsoft.com/windows/pnpx/2005/10 |

Most properties are persisted when the device is installed. A device's property store is cached from installation to installation, so the device metadata is refreshed only when the <wsdisco:MetadataVersion> element has changed. Only the <wsd:ThisModel/pnpx:DeviceCategory> element (and its corresponding PKEY, PKEY_PNPX_DeviceCategory) are not persisted.

##### Metadata Elements Required by PnP-X

The following table shows the metadata elements that must be included in the metadata message for device installation. The table also shows the mapping from metadata elements to PKEYs.

| Element | Device PKEY | Service PKEY |
| --- | --- | --- |
| wsa:Address | PKEY_PNPX_GlobalIdentity | PKEY_PNPX_GlobalIdentity |
|  | PKEY_PNPX_ID | n/a |
| wsd:ThisModel/pnpx:DeviceCategory | PKEY_PNPX_DeviceCategory | n/a |
| wsd:Hosted/pnpx:hardwareID | n/a | PKEY_PNPX_CompatibleTypes |
| wsd:Hosted/pnpx:compatibleID | n/a | PKEY_PNPX_CompatibleTypes |

The address (PKEY_PNPX_GlobalIdentity), the hardware identifier(s), and compatible identifier(s) (PKEY_PNPX_CompatibleTypes) are used by PnP to identify the device and to locate compatible drivers. The wsd:Hosted/pnpx:hardwareID and wsd:Hosted/pnpx:compatibleID elements are used to populate the PKEY_PNPX_CompatibleTypes key only if the device is installable, that is, if PKEY_PNPX_Installable is TRUE.

Note  The device identifier must be unique. Devices cannot share an identifier, even if the devices use different protocols. If a device implements more than one protocol, the device must have a unique identifier for each protocol.

##### Metadata Elements Required by DPWS

The following table shows the metadata elements required by the DPWS specification. The table also shows the mapping from metadata elements to PKEYs.

| Element | Device PKEY | Service PKEY |
| --- | --- | --- |
| wsdisco:MetadataVersion | PKEY_PNPX_MetadataVersion | PKEY_PNPX_MetadataVersion |
| wsd:ThisModel/wsd:Manufacturer | PKEY_PNPX_Manufacturer | PKEY_PNPX_Manufacturer |
|  | PKEY_Device_Manufacturer | PKEY_Device_Manufacturer |
|  | PKEY_PNPX_CompatibleTypes | PKEY_PNPX_CompatibleTypes |
| wsd:ThisModel/wsd:ManufacturerURL | PKEY_PNPX_ManufacturerUrl | PKEY_PNPX_ManufacturerUrl |
|  | PKEY_DriverPackage_VendorWebSite | PKEY_DriverPackage_VendorWebSite |
| wsd:ThisModel/wsd:ModelName | PKEY_PNPX_ModelName | PKEY_PNPX_ModelName |
|  | PKEY_Device_Model | PKEY_Device_Model |
|  | PKEY_Device_DeviceDesc | PKEY_Device_DeviceDesc |
|  | PKEY_PNPX_CompatibleTypes | PKEY_PNPX_CompatibleTypes |
| wsd:ThisModel/wsd:ModelNumber | PKEY_PNPX_ModelNumber | PKEY_PNPX_ModelNumber |
|  | PKEY_PNPX_CompatibleTypes | PKEY_PNPX_CompatibleTypes |
| wsd:ThisModel/wsd:ModelURL | PKEY_PNPX_ModelUrl | PKEY_PNPX_ModelUrl |
| wsd:ThisModel/wsd:PresentationURL | PKEY_PNPX_PresentationUrl | PKEY_PNPX_PresentationUrl |
| wsd:ThisDevice/wsd:FriendlyName | PKEY_PNPX_FriendlyName | PKEY_PNPX_FriendlyName |
|  | PKEY_Device_FriendlyName | PKEY_Device_FriendlyName |
| wsd:ThisDevice/wsd:FirmwareVersion | PKEY_PNPX_FirmwareVersion | PKEY_PNPX_FirmwareVersion |
|  | PKEY_Device_BIOSVersion | PKEY_Device_BIOSVersion |
| wsd:ThisDevice/wsd:SerialNumber | PKEY_PNPX_SerialNumber | PKEY_PNPX_SerialNumber |
| wsd:Hosted/wsd:ServiceId | n/a | PKEY_PNPX_ID |
| wsd:ServiceId | n/a | PKEY_PNPX_ServiceId |
| wsd:Hosted/wsd:ServiceAddress | n/a | PKEY_PNPX_ServiceAddress |
| wsd:Hosted/wsdp:Types | n/a | PKEY_PNPX_Types |

Note  The wsd:ThisModel/wsd:Manufacturer, wsd:ThisModel/wsd:ModelName, and wsd:ThisModel/wsd:ModelNumber elements are used to populate the PKEY_PNPX_CompatibleTypes key only if the device is not installable, that is, if PKEY_PNPX_Installable is FALSE.

Note  The total combined length of PKEY_PNPX_GlobalIdentity and PKEY_PNPX_ServiceId cannot exceed 167 characters (not including the NULL terminator).  If this limit is exceeded the installation of the device will fail.

##### Optional Metadata

The following table shows optional metadata elements that can be included in the metadata message. The table also shows the mapping from metadata elements to PKEYs.

| Element | Device PKEY | Service PKEY |
| --- | --- | --- |
| wsdisco:Types | PKEY_PNPX_Types | n/a |
|  | PKEY_PNPX_CompatibleTypes | n/a |
| wsdisco:Scopes | PKEY_PNPX_Scopes | PKEY_PNPX_Scopes |
| wsdisco:XAddrs | PKEY_PNPX_XAddrs | PKEY_PNPX_XAddrs |
|  | PKEY_Device_LocationInfo | PKEY_Device_LocationInfo |

Note  The wsdisco:Types element is used to populate the PKEY_PNPX_CompatibleTypes key only if the device is not installable, that is, if PKEY_PNPX_Installable is FALSE.

See also in this document:

- Querying Device Metadata

- pnpx-wsd Schema

#### Sample INF File for WSD Devices

The following INF file provides driver-matching information for the device using the hardware identifier PnPX_SampleService1_HWID.

Note  When adapting this sample for use with your device, you must maintain the UMB\ prefix on the device HWID or compatible ID. If you fail to do so, the device will not be recognized by PnP-X and therefore the device will not be installed.

;/*++

;

;Copyright )) Microsoft Corporation. All rights reserved.

;

;Module Name:

;

;    pnpxsample.inf

;

;Abstract:

;    Sample PnP-X INF file

;

;--*/

[Version]

Signature="$WINDOWS NT$"

Class=system

Class=PNPX

Provider=%MSFT%

ClassGuid={46162fd1-d6ed-4a13-8cae-273b9498c251}

DriverVer=4/8/2005,1.00.0000

[DestinationDirs]

DefaultDestDir = 12

[ClassInstall32]

Addreg=PNPXDevices_ClassReg

[PNPXDevices_ClassReg]

HKR,,,,%ClassName%

HKR,,SilentInstall,,1

HKR,,Icon,,"-52"

[Manufacturer]

%MSFT%=PnpxDevice, ntamd64, ntia64

[PnpxDevice]

; Model Device Description    Install Section       HWIDs, Compatible IDs

; -----------                    -------               --------

%PnpxDevice.DeviceDesc1%=PnpxDevice_Install, UMB\PnPX_SampleService1_HWID

[PnpxDevice.ntamd64]

%PnpxDevice.DeviceDesc1%=PnpxDevice_Install, UMB\PnPX_SampleService1_HWID

[PnpxDevice.ia64]

%PnpxDevice.DeviceDesc1%=PnpxDevice_Install, UMB\PnPX_SampleService1_HWID

[PnpxDevice_Install]

[PnpxDevice_Install.HW]

AddReg=PnpxDevice_Install.HW.AddReg

[PnpxDevice_Install.HW.AddReg]

HKR,,InterfaceGUIDs,0x10000, "{6639fe91-e2e7-4bed-a066-8ddfcf64049d}"

[PnpxDevice_Install.Services]         ; Setup the UMPass service for our device

AddService = UMPass, %SPSVCINST_ASSOCSERVICE%, UMPassService_Install

[UMPassService_Install]

DisplayName    = %umpass.SVCDESC%     ; Friendly Name of the Service

ServiceType    = 1                    ; SERVICE_KERNEL_DRIVER

StartType      = 3                    ; SERVICE_DEMAND_START

ErrorControl   = 1                    ; SERVICE_ERROR_NORMAL

ServiceBinary  = %12%\umpass.sys

LoadOrderGroup = Extended Base

[Strings]      ; Strings that are referenced throughout the INF

MSFT = "Microsoft"

ClassName = "PNP-X Devices"

SPSVCINST_ASSOCSERVICE= 0x00000002

umpass.SVCDESC = "Microsoft UMPass Driver"

PnpxDevice.DeviceDesc1 = "PNP-X Example Device INF"

#### Sample Metadata Message for WSD Devices

The following SOAP message specifies metadata for a device (printer/scanner) that offers two hosted services, identified by PnPX_SampleService1_HWID and PnPX_SampleService2_HWID.

<soap:Envelope

xmlns:soap="http://www.w3.org/2003/05/soap-envelope"

xmlns:wsa="http://schemas.xmlsoap.org/ws/2004/08/addressing"

xmlns:wsdisco="http://schemas.xmlsoap.org/ws/2005/04/discovery"

xmlns:wsx="http://schemas.xmlsoap.org/ws/2004/09/mex"

xmlns:wsd="http://schemas.xmlsoap.org/ws/2006/02/devprof"

xmlns:pnpx="http://schemas.microsoft.com/windows/pnpx/2005/10">

<soap:Header>

<!-- Place SOAP header information here.-->

</soap:Header>

<soap:Body>

<wsx:Metadata>

<wsx:MetadataSection

Dialect="http://schemas.xmlsoap.org/ws/2005/05/devprof/ThisDevice">

<wsd:ThisDevice>

<!-- Place ThisDevice metadata here.-->

</wsd:ThisDevice>

</wsx:MetadataSection>

<wsx:MetadataSection

Dialect="http://schemas.xmlsoap.org/ws/2005/05/devprof/ThisModel">

<wsd:ThisModel>

<!-- Place ThisModel metadata here.-->

<pnpx:DeviceCategory>

<!-- This device is in the Printers category -->

Printers Scanners

</pnpx:DeviceCategory>

</wsd:ThisModel>

</wsx:MetadataSection>

<wsx:MetadataSection

Dialect="http://schemas.xmlsoap.org/ws/2005/05/devprof/Relationship">

<wsd:Relationship

Type="http://schemas.xmlsoap.org/ws/2005/05/devprof/host">

<wsd:Hosted>

<!-- Place Hosted metadata for the

first service here.-->

<pnpx:HardwareId>

<!-- Place the Hardware ID for the

first service here.-->

PnPX_SampleService1_HWID

</pnpx:HardwareId>

<pnpx:CompatibleId>

<!-- Place the Compatible ID for the

first service here.-->

PnPX_SampleService1_CPID

</pnpx:CompatibleId>

</wsd:Hosted>

<wsd:Hosted>

<!-- Place Hosted metadata for the

second service here.-->

<pnpx:HardwareId>

<!-- Place the Hardware ID for the

second service here.-->

PnPX_SampleService2_HWID

</pnpx:HardwareId>

<pnpx:CompatibleId>

<!-- Place the Compatible ID for the

second service here.-->

PnPX_SampleService2_CPID

</pnpx:CompatibleId>

</wsd:Hosted>

<wsd:Hosted>

<!-- Place Hosted metadata for the

third service here.-->

<!-- This service will not be installed by

PnP-X because it does not contain a

Hardware ID or Compatible ID.-->

</wsd:Hosted>

</wsd:Relationship>

</wsx:MetadataSection>

</wsx:Metadata>

</soap:Body>

</soap:Envelope>

### Creating a SSDP-Compliant Device

Device manufacturers must implement the following, in addition to base device functionality:

- UPnP 1.0 device architecture support. To view the UPnP device architecture specification, visit http://go.microsoft.com/fwlink/?LinkId=60979.

- pnpx-ssdp schema support.

The device must completely implement the UPnP device architecture, including the SSDP discovery response message and the device description file. The device description file must include all elements specified in “SSDP Device Metadata Requirements” in this document. This file includes both UPnP and pnpx-ssdp schema elements.

SSDP devices can be installed only if the following criteria are met:

- The HWID or Compatible ID specified in the device's INF file begins with the prefix UMB\. For a correctly formatted HWID, see “Sample INF File for SSDP Devices” in this document.

- The device is on the same subnet as the computer performing device discovery.

SSDP devices are installed silently (without prompting the user) if a signed driver package is installed on the computer performing device discovery. If no signed driver package is available, the user is prompted to install the device.

#### SSDP Device Metadata Requirements

Device metadata is supplied in an SSDP discovery response message and in a device description file.

Metadata is stored in property keys (PKEYs), and can be queried as described in “Querying Device Metadata” in this document.

The PKEY_PNPX_* property keys are described in “Key Definitions” at MSDN. Other property keys are defined in propkey.h.

Most properties are persisted when the device is installed. Only the pnpx:X_DeviceCategory element (and its corresponding PKEY, PKEY_PNPX_DeviceCategory) are not persisted.

##### SSDP Discovery Response Metadata

The following table shows the metadata elements that must be specified in the SSDP discovery response message. The table also shows the mapping from headers to PKEYs.

| Header | Device PKEY |
| --- | --- |
| USN | PKEY_PNPX_GlobalIdentity |
|  | PKEY_PNPX_ID |
| LOCATION | PKEY_PNPX_XAddrs |
|  | PKEY_Device_LocationInfo |

The USN is a UUID used by PnP to identify the device on the network. The location specifies the URL of the device description file.

Note  The device identifier must be unique. Devices cannot share an identifier, even if the devices use different protocols. If a device implements more than one protocol, the device must have a unique identifier for each protocol.

Note  The total combined length of PKEY_PNPX_GlobalIdentity and PKEY_PNPX_Id cannot exceed 167 characters (not including the NULL terminator).  If this limit is exceeded the installation of the device will fail.

##### Metadata Elements Required by PnP-X

The following table shows the namespace prefix that is used by PnP-X metadata elements in UPnP devices.

| Prefix | Namespace |
| --- | --- |
| pnpx | http://schemas.microsoft.com/windows/pnpx/2005/11 |

The following table shows the metadata elements that must be included in the description file for device installation. The table also shows the mapping from metadata elements to PKEYs.

| Element | Device PKEY |
| --- | --- |
| pnpx:X_DeviceCategory | PKEY_PNPX_DeviceCategory |
| pnpx:X_hardwareID | PKEY_PNPX_HardwareIds |
|  | PKEY_Device_HardwareIds |
|  | PKEY_PNPX_CompatibleTypes |
| pnpx:X_compatibleID | PKEY_PNPX_CompatibleIds |
|  | PKEY_Device_CompatibleIds |
|  | PKEY_PNPX_CompatibleTypes |

Note  The pnpx:X_hardwareID and pnpx:X_compatibleID elements are used to populate the PKEY_PNPX_CompatibleTypes key only if the device is installable, that is, if PKEY_PNPX_Installable is TRUE.

##### Metadata Elements Required by UPnP Architecture

The following table shows the metadata elements that are required by the UPnP architecture specification. Elements are defined in the namespace urn:schemas-upnp-org:device-1-0. The table also shows the mapping from metadata elements to PKEYs.

| Element | Device PKEY |
| --- | --- |
| friendlyName | PKEY_PNPX_FriendlyName |
|  | PKEY_Device_FriendlyName |
| manufacturer | PKEY_PNPX_Manufacturer |
|  | PKEY_Device_Manufacturer |
|  | PKEY_PNPX_CompatibleTypes |
| modelName | PKEY_PNPX_ModelName |
|  | PKEY_Device_Model |
|  | PKEY_PNPX_CompatibleTypes |
| deviceType | PKEY_PNPX_DeviceType |
|  | PKEY_PNPX_CompatibleTypes |

Note  The manufacturer, modelName, and deviceType elements are used to populate the PKEY_PNPX_CompatibleTypes key only if the device is not installable, that is, if PKEY_PNPX_Installable is FALSE.

##### Optional Metadata

The following table shows optional metadata elements that can be included in the metadata message. Elements are defined in the namespace urn:schemas-upnp-org:device-1-0. The table also shows the mapping from metadata elements to PKEYs.

| Element | Device PKEY |
| --- | --- |
| manufacturerURL | PKEY_PNPX_ManufacturerUrl |
|  | PKEY_DriverPackage_VendorWebSite |
| modelNumber | PKEY_PNPX_ModelNumber |
|  | PKEY_PNPX_CompatibleTypes |
| modelDescription | PKEY_Device_DeviceDesc |
| presentationURL | PKEY_PNPX_PresentationUrl |
| serialNumber | PKEY_PNPX_SerialNumber |
| UPC | PKEY_PNPX_Upc |

Note  The modelNumber element is used to populate the PKEY_PNPX_CompatibleTypes key only if the device is not installable, that is, if PKEY_PNPX_Installable is FALSE.

See also in this document:

- Querying Device Metadata

#### Sample SSDP Discovery Response Message

The SSDP discovery response message contains device metadata that is required to identify the device on the network. The unique service name (USN) is used as the device identifier.

The format of the response message is specified by the UPnP device architecture specification. For more information about the specification, see http://go.microsoft.com/fwlink/?LinkId=60979.

The following example shows an SSDP discovery response message.

HTTP/1.1 200 OK

Location: http://192.168.0.1:1234/upnphost/udhisapi.dll?

content=uuid:7364efab-76ed-39bd-bde9-1837415aeb93

Ext:

USN: uuid:7364efab-76ed-39bd-bde9-1837415aeb93:

:urn:schemas-upnp-org:device:Printer:1

Server: Microsoft-Windows-NT/5.1 UPnP/1.0

UPnP-Device-Host/1.0

Cache-Control: max-age=1800

ST: urn:schemas-upnp-org:device:Printer:1

Content-Length:0

#### Sample INF File for SSDP Devices

The following INF file provides driver matching information for the device using the hardware identifier PnPX SampleDevice_HWID.

Note  When adapting this sample for use with your device, you must maintain the UMB\ prefix on the device HWID or compatible ID. If you fail to do so, the device will not be recognized by PnP-X and therefore the device will not be installed.

;/*++

;

;Copyright )) Microsoft Corporation. All rights reserved.

;

;Module Name:

;

;    pnpxsample.inf

;

;Abstract:

;    Sample PnP-X INF file

;

;--*/

[Version]

Signature="$WINDOWS NT$"

Class=system

Class=PNPX

Provider=%MSFT%

ClassGuid={46162fd1-d6ed-4a13-8cae-273b9498c251}

DriverVer=4/8/2005,1.00.0000

[DestinationDirs]

DefaultDestDir = 12

[ClassInstall32]

Addreg=PNPXDevices_ClassReg

[PNPXDevices_ClassReg]

HKR,,,,%ClassName%

HKR,,SilentInstall,,1

HKR,,Icon,,"-52"

[Manufacturer]

%MSFT%=PnpxDevice, ntamd64, ntia64

[PnpxDevice]

; Model Device Description    Install Section       HWIDs, Compatible IDs

; -----------                    -------               --------

%PnpxDevice.DeviceDesc1%=PnpxDevice_Install, UMB\PnPX_SampleDevice_HWID

[PnpxDevice.ntamd64]

%PnpxDevice.DeviceDesc1%=PnpxDevice_Install, UMB\PnPX_SampleDevice_HWID

[PnpxDevice.ia64]

%PnpxDevice.DeviceDesc1%=PnpxDevice_Install, UMB\PnPX_SampleDevice_HWID

[PnpxDevice_Install]

[PnpxDevice_Install.HW]

AddReg=PnpxDevice_Install.HW.AddReg

[PnpxDevice_Install.HW.AddReg]

HKR,,InterfaceGUIDs,0x10000, "{6639fe91-e2e7-4bed-a066-8ddfcf64049d}"

[PnpxDevice_Install.Services]         ; Setup the UMPass service for our device

AddService = UMPass, %SPSVCINST_ASSOCSERVICE%, UMPassService_Install

[UMPassService_Install]

DisplayName    = %umpass.SVCDESC%     ; Friendly Name of the Service

ServiceType    = 1                    ; SERVICE_KERNEL_DRIVER

StartType      = 3                    ; SERVICE_DEMAND_START

ErrorControl   = 1                    ; SERVICE_ERROR_NORMAL

ServiceBinary  = %12%\umpass.sys

LoadOrderGroup = Extended Base

[Strings]      ; Strings that are referenced throughout the INF

MSFT = "Microsoft"

ClassName = "PNP-X Devices"

SPSVCINST_ASSOCSERVICE= 0x00000002

umpass.SVCDESC = "Microsoft UMPass Driver"

PnpxDevice.DeviceDesc1 = "PNP-X - Example Device INF"

#### Sample Device Description File for SSDP Devices

The following XML file specifies metadata for a printer identified by PnPX_SampleDevice_HWID.

<?xml version="1.0" ?>

<root xmlns="urn:schemas-upnp-org:device-1-0">

<specVersion>

<major>major version number</major>

<minor>minor version number</minor>

</specVersion>

<URLBase>device URL</URLBase>

<device>

<pnpx:X_hardwareId xmlns:pnpx=”http://schemas.microsoft.com/windows/pnpx/2005/11”>

PnPX_SampleDevice_HWID

</pnpx:X_hardwareId>

<pnpx:X_compatibleId xmlns:pnpx=”http://schemas.microsoft.com/windows/pnpx/2005/11”>

PnPX_SampleDevice_CPID

</pnpx:X_compatibleId>

<pnpx:X_deviceCategory xmlns:pnpx=”http://schemas.microsoft.com/windows/pnpx/2005/11”>

Printers.Printer

</pnpx:X_deviceCategory >

<deviceType>UPnP device type</deviceType>

<friendlyName>friendly name</friendlyName>

<manufacturer>manufacturer</manufacturer>

<manufacturerURL>manufacturer URL</manufacturerURL>

<modelDescription>model description</modelDescription>

<modelName>model name</modelName>

<modelNumber>model number</modelNumber>

<modelURL>URL to model site</modelURL>

<serialNumber>manufacturer's serial number</serialNumber>

<UDN>device UDN</UDN>

<UPC>universal product code</UPC>

<iconList>

<icon>

<mimetype>image/format</mimetype>

<width>horizontal pixels</width>

<height>vertical pixels</height>

<depth>color depth</depth>

<url>URL to icon</url>

</icon>

<!-- more icons  ->

</iconList>

<serviceList>

<service>

<serviceType />

<serviceId />

<SCPDURL />

<controlURL />

<eventSubURL />

</service>

</serviceList>

<deviceList>

<!-- child device descriptions ->

</deviceList>

<presentationURL>presentation URL</presentationURL>

</device>

</root>

## Querying Device Metadata

Each PnP-X device is associated with a function instance, represented by the IFunctionInstance object. Each function instance has a property store. This property store contains metadata supplied by a PnP-X device.

There are a number of ways to view, set, and query a PnP-X device's metadata. A property store can be opened using the IFunctionInstance::OpenPropertyStore method. The metadata in the property store can be manipulated by using the IPropertyStore interface.

A query can be constructed for devices with specific metadata by specifying a property constraint on a collection query. To do this, call the IFunctionInstanceCollectionQuery::AddPropertyConstraint method on the IFunctionInstanceCollectionQuery object that is returned by the IFunctionDiscovery::CreateInstanceCollectionQuery method.

# PnP-X Reference

This section contains the reference material on using the PnP-X interface. The topics include:

- PnP-X Category Definitions

- pnpx-wsd Schema Reference

## PnP-X Category Definitions

PnP-X categories are the namespaces in which components that provide similar base functionality are grouped.

The PKEY_PNPX_DeviceCategory property key associated with a PnP-X device function instance stores the category value. Like other PKEY values, the category value is specified in the device metadata.

Each category has predefined keywords associated with it. These keywords are used to restrict the list of devices displayed in Network Explorer. When the user types one of the predefined keywords into the Search Category box, Network Explorer displays only the items in the matching category.

Some categories have predefined subcategories. A subcategory can be specified in the device metadata, and its value is stored in the PKEY_PNPX_DeviceCategory property key instead of the parent category value.

Subcategories do not appear in Network Explorer. Instead, a subcategory's parent category value appears. Subcategories can be queried programmatically as described in “Querying Device Metadata” in this document, but they cannot be queried by the end user.

Providers can define and use their own subcategories, but they cannot define their own categories.

The following are the built-in PnP-X categories.

| Constant/value | Description |
| --- | --- |
| PNPX_DEVICECATEGORY_COMPUTER L"Computers" | Computers category. Keywords: Computer, Laptop, Tablet Defined subcategories: Computers.PC, Computers.MediaCenter, Computers.Server |
| PNPX_DEVICECATEGORY_INPUTDEVICE L"Input" | Input Devices category. Keywords: Keyboard, Mouse, Input Defined subcategories: Input.Keyboard, Input.Mouse |
| PNPX_DEVICECATEGORY_PRINTER L"Printers" | Printers category. Keyword: Printer |
| PNPX_DEVICECATEGORY_SCANNER L"Scanners" | Scanners category. Keyword: Scanner |
| PNPX_DEVICECATEGORY_FAX L"FAX" | FAX category. Keyword: FAX |
| PNPX_DEVICECATEGORY_MFP L"MFP" | Multifunction Devices category. Keywords: Multifunction Device, MFP |
| PNPX_DEVICECATEGORY_CAMERA L"Cameras" | Cameras category. Keywords: Camera, Video Camera, Digital Camera Defined subcategories: Cameras.DigitalStillCamera, Cameras.VideoCamera |
| PNPX_DEVICECATEGORY_STORAGE L"Storage" | Storage category. Keywords: Hard Drive, CD, CD-ROM, DVD, DVD-ROM, Flash, SD Defined subcategory: Storage.NAS |
| PNPX_DEVICECATEGORY_NETWORK_INFRASTRUCTURE L"NetworkInfrastructure" | Network Infrastructure category. Keywords: Hub, Switch, Gateway, Router, Access Point, Modem Defined subcategories: NetworkInfrastructure.AccessPoint, NetworkInfrastructure.Router, NetworkInfrastructure.Switch |
| PNPX_DEVICECATEGORY_DISPLAYS L"Displays" | Displays category. Keywords: Monitor, Projector, Television, Digital Picture Frame Defined subcategory: Displays.Monitor |
| PNPX_DEVICECATEGORY_MULTIMEDIA_DEVICE L"MediaDevices" | Media Devices category. Keywords: Music Player, Music Server, MP3 Player, WMA Player, Pika, Media Extender, Stereo, A/V Receiver, Cable, Satellite, DVR, Speakers Defined subcategories: MediaDevices.DVR, MediaDevices.DAR, MediaDevices.MCE, MediaDevices.MusicPlayer |
| PNPX_DEVICECATEGORY_GAMING_DEVICE L"Gaming" | Gaming Devices category. Keywords: Xbox, Controller, Joystick Defined subcategories: Gaming.Xbox, Gaming.Xbox360 |
| PNPX_DEVICECATEGORY_TELEPHONE L"Phones" | Phones category. Keywords: Telephone, Cell Phone, Pager Defined subcategories: Phones.CellPhone, Phones.WindowsMobile |
| PNPX_DEVICECATEGORY_HOME_AUTOMATION_SYSTEM L"HomeAutomation" | Home Automation category. Keywords: Light, Thermostat, Heater, A/C, Air Conditioning Defined subcategories: HomeAutomation.Light, HomeAutomation.Thermostat, HomeAutomation.Heater, HomeAutomation.AC |
| PNPX_DEVICECATEGORY_HOME_SECURITY_SYSTEM L"HomeSecurity" | Home Security category. Keywords: Security, Home Security |
| PNPX_DEVICECATEGORY_OTHER L"Other" | Other Devices category. There are no keywords or subcategories associated with this category. |

Requirements:

- Client: Requires Windows Vista.

- Server: Requires Microsoft Windows Server Code Name "Longhorn".

- Header: Declared in FunctionDiscoveryKeys.h.

See also on the MSDN Web site:

- Function Discovery Categories

## pnpx-ssdp Schema Reference

The pnpx-ssdp schema specifies metadata elements for PnP-X devices that use the SSDP protocol.

### pnpx-ssdp Schema Elements

The pnpx-ssdp schema defines the following elements.

- X_compatibleId (X_PnpXType)

- X_deviceCategory (X_PnpXType)

- X_hardwareId (X_PnpXType)

- X_PnpX

#### X_compatibleId (X_PnpXType) Element

Specifies the compatible identifier(s) of the services provided by a device. Devices may have more than one compatible ID.

<xs:element name="X_compatibleId"

type="string"

/>

The X_compatibleId element is defined by the X_PnpXType complex type.

Remarks

To specify more than one CompatibleID, separate the identifiers with a space character, for example, PnPX_SampleService_HWID_1 PnPX_SampleService_HWID_2 PnPX_SampleService1_HWID_3.

See also in this document:

- For definition context of element in schema, see “X_PnpXType_ComplexType” in this document

- For possible immediate parent element in schema instance, see “X_PnpX Element” in this document

#### X_deviceCategory (X_PnpXType) Element

Specifies the PnP-X category to which the device belongs. More than one category can be specified.

<xs:element name="X_deviceCategory"

type="string"

/>

The X_deviceCategory element is defined by the X_PnpXType complex type.

Remarks

To specify more than one device category, separate the categories with a space. For example, Printers Storage identifies a device with a primary category of Printers and a secondary category of Storage.

Devices can also specify a device subcategory for a more descriptive device category. For example, Phones.WindowsMobile Cameras.DigitalStillCamera MediaDevices.MusicPlayer identifies a device that is a Microsoft Windows Mobile® device with a camera and music player. The primary device category for this device would be Phones.

See also in this document:

- For definition context of element in schema, see “X_PnpXType_ComplexType” in this document

- For possible immediate parent element in schema instance, see “X_PnpX Element” in this document

#### X_hardwareId (X_PnpXType) Element

The X_hardwareId (X_PnpXType) element specifies the hardware identifier(s) of the device. PnP matches this element with the hardware description(s) provided in the [PnpxDevice] section of the device's INF file. Based on this information, the PnP service selects and loads the most appropriate device driver.

<xs:element name="X_hardwareId"

type="string"

/>

The X_hardwareId element is defined by the X_PnpXType complex type.

Remarks

To specify more than one hardware ID, separate the identifiers with a space character, for example, PnPX_SampleService_HWID_1 PnPX_SampleService_HWID_2 PnPX_SampleService1_HWID_3.

See also in this document:

- For definition context of element in schema, see “X_PnpXType_ComplexType” in this document

- For possible immediate parent element in schema instance, see “X_PnpX Element” in this document

#### X_PnpX Element

Specifies SSDP device metadata.

<xs:element name="X_PnpX"

type="X_PnpXType"

/>

### pnpx-ssdp Schema Complex Types

The pnpx-ssdp schema defines the X_PnpXType.

#### X_PnpXType Complex Type

Specifies the required metadata elements for PnP-X devices using the SSDP protocol. This complex type is extensible.

<xs:complexType name="X_PnpXType">

<xs:sequence>

<xs:element name="X_hardwareId"

type="string"

minOccurs="0"

maxOccurs="unbounded"

/>

X_hardwareId

Specifies the hardware identifier(s) of the device.

<xs:element name="X_compatibleId"

type="string"

minOccurs="0"

maxOccurs="unbounded"

/>

X_compatibleId

Specifies the compatible identifier(s) of the services provided by a device.

<xs:element name="X_deviceCategory"

type="string"

minOccurs="0"

maxOccurs="unbounded"

/>

X_deviceCategory

Specifies the PnP-X category to which the device belongs.

<xs:any

processContents="lax"

minOccurs="0"

maxOccurs="unbounded"

namespace="##other"

/>

</xs:sequence>

<xs:anyAttribute

processContents="lax"

namespace="##any"

/>

</xs:complexType>

X_PnpXType

Child elements:

| Element | Type | Description |
| --- | --- | --- |
| X_compatibleId | string | Specifies the compatible identifier(s) of the services provided by a device. |
| X_deviceCategory | string | Specifies the PnP-X category to which the device belongs. |
| X_hardwareId | string | Specifies the hardware identifier(s) of the device. |

## pnpx-wsd Schema Reference

Specifies metadata elements for PnP-X devices that use the WSD protocol.

### pnpx-wsd Schema Elements

The pnpx-wsd schema defines the following elements.

- CompatibleId

- DeviceCategory

- HardwareId

#### CompatibleId Element

Specifies the compatible identifier(s) of the services provided by a device. PnP matches this element with the service description(s) provided in the [PnpxDevice] section of the device's INF file.

<xs:element name="CompatibleId"

type="CompatibleId"

/>

CompatibleId

Remarks

The length of this string must not exceed 196 wide characters (392 bytes). To specify more than one CompatibleID, separate the identifiers with a space character, for example, PnPX_SampleService_HWID_1 PnPX_SampleService_HWID_2 PnPX_SampleService1_HWID_3.

#### DeviceCategory Element

Specifies the PnP-X category to which the device belongs. More than one category can be specified.

<xs:element name="DeviceCategory"

type="DeviceCategoryType"

/>

DeviceCategory

Remarks

To specify more than one device category, separate the categories with a space. For example, Printers Storage identifies a device with a primary category of Printers and a secondary category of Storage.

Devices can also specify a device subcategory for a more descriptive device category. For example, Phones.WindowsMobile Cameras.DigitalStillCamera MediaDevices.MusicPlayer identifies a device that is a Windows Mobile device with a camera and music player. The primary device category for this device would be Phones.

See also in this document:

- PnP-X Category Definitions

#### HardwareId Element

Specifies the hardware identifier(s) of the device. PnP matches this element with the hardware description(s) provided in the [PnpxDevice] section of the device's INF file. Based on this information, the PnP service selects and loads the most appropriate device driver.

<xs:element name="HardwareId"

type="HardwareIdType"

/>

HardwareId

Remarks

The length of this string must not exceed 196 wide characters (392 bytes). To specify more than one HardwareId, separate the identifiers with a space character, for example, PnPX_SampleService_HWID_1 PnPX_SampleService_HWID_2 PnPX_SampleService1_HWID_3.

### pnpx-wsd Schema Simple Types

The pnpx-wsd schema defines the following types.

- CompatibleIdType

- DeviceCategoryType

- HardwareIdType

#### CompatibleIdType Simple Type

Specifies the data type of the CompatibleId element.

<xs:simpleType name="CompatibleIdType">

<xs:list>

<xs:itemType name="string" />

string

</xs:list>

</xs:simpleType>

CompatibleIdType

#### DeviceCategoryType Simple Type

Specifies the data type of the DeviceCategory element.

<xs:simpleType name="DeviceCategoryType">

<xs:list>

<xs:itemType name="string" />

string

</xs:list>

</xs:simpleType>

#### HardwareIdType Simple Type

Specifies the data type of the HardwareId element.

<xs:simpleType name="HardwareIdType">

<xs:list>

<xs:itemType name="string" />

string

</xs:list>

</xs:simpleType>

HardwareIdType

DeviceCategoryType

# References

Contact by E-mail:

rally@microsoft.com

Device Profile for Web Services Specifications

Devices Profile for Web Services
Web Services Dynamic Discovery
http://msdn.microsoft.com/webservices/understanding/specs/default.aspx?pull=/library/en-us/dnglobspec/html/devprof.asp

Function Discovery API Reference on MSDN

http://msdn.microsoft.com/library/default.asp?url=/library/en-us/fundisc/ncd/function_discovery_reference.asp

Portable and Network-Connected Devices – white papers and presentations

http://www.microsoft.com/whdc/device/netAttach/default.mspx

UPnP Device Architecture, Version 1.0

http://www.upnp.org/download/UPnPDA10_20000613.htm

UPnP Forum

http://www.upnp.org

Web Services Specifications on MSDN

http://msdn.microsoft.com/webservices/understanding/specs/default.aspx?pull=/library/en-us/dnglobspec/html/devprof.asp

Windows Rally specifications and white papers

http://www.microsoft.com/rally