A WINDOWS® RALLY™ SPECIFICATION

Windows Connect Now-UFD and Windows Vista

Abstract

This specification provides implementation details for manufacturers who design wireless networking products that work with the Microsoft® Windows® Connect Now-UFD (WCD-UFD) configuration feature of the Microsoft Windows Vista™ operating system. Windows Connect Now is part of the Microsoft Windows Rally™ technologies that enable simple, secure discovery and configuration of network-connected devices.

Version 1.0 – September 15, 2006

LICENSE NOTICE. Access to and viewing and implementation of the technology described in this document is granted under the Microsoft Windows Rally Program License Agreement (“License Agreement”). If you want a license from Microsoft to access, view or implement one or more Licensed Technologies, you must complete the designated information in the License Agreement and return a signed copy to Microsoft. The License Agreement is provided at the end of this document. If the License Agreement is not available with this document, you can download a copy from the Windows Rally Web site at http://www.microsoft.com/rally.

Disclaimer

This is a preliminary document and may be changed substantially prior to final commercial release of the software described herein.

The information contained in this document represents the current view of Microsoft Corporation on the issues discussed as of the date of publication. Because Microsoft must respond to changing market conditions, it should not be interpreted to be a commitment on the part of Microsoft, and Microsoft cannot guarantee the accuracy of any information presented after the date of publication.

This White Paper is for informational purposes only. MICROSOFT MAKES NO WARRANTIES, EXPRESS, IMPLIED OR STATUTORY, AS TO THE INFORMATION IN THIS DOCUMENT.

Complying with all applicable copyright laws is the responsibility of the user. Without limiting the rights under copyright, no part of this document may be reproduced, stored in or introduced into a retrieval system, or transmitted in any form or by any means (electronic, mechanical, photocopying, recording, or otherwise), or for any purpose, without the express written permission of Microsoft Corporation.

Microsoft may have patents, patent applications, trademarks, copyrights, or other intellectual property rights covering subject matter in this document. Except as expressly provided in any written license agreement from Microsoft, the furnishing of this document does not give you any license to these patents, trademarks, copyrights, or other intellectual property.

Unless otherwise noted, the example companies, organizations, products, domain names, e-mail addresses, logos, people, places and events depicted herein are fictitious, and no association with any real company, organization, product, domain name, email address, logo, person, place or event is intended or should be inferred.

© 2006 Microsoft Corporation.  All rights reserved.

Microsoft, Rally, Windows, Windows NT, Windows Server, and Windows Vista are either registered trademarks or trademarks of Microsoft Corporation in the United States and/or other countries.

The names of actual companies and products mentioned herein may be the trademarks of their respective owners.

The current version of this specification is maintained on the Web at: 
http://www.microsoft.com/rally

Revision History

| Date | Revision |
| --- | --- |
| May 2006 | V 1.0 published with specifications for Windows Rally technologies |

Contents

Introduction	4

WCN-UFD File Architecture	5

Autorun.inf	7

Setupsnk.exe	7

Wsetting.wfc	7

Wsetting.wfc  Elements	7

Wsetting.wfc Schema	8

Wsetting.wfc Sample File (WEP Encryption)	11

Wsetting.wfc Sample File (WPA Encryption)	11

Device Configuration File (Xxxxxxxx.wfc)	11

Device Configuration File Elements	11

Device Configuration File Schema	12

Sample Device Configuration File	17

Dual Radio APs	17

Device and Software Requirements	19

Requirements for USB Flash Drives	19

Firmware and Software Requirements	20

UFD Configuration with Earlier Versions of Windows	20

References	21

Introduction

This specification provides implementation details for manufacturers who design wireless networking products that work with the Windows Connect Now (WCN) USB Flash Drive feature of the Microsoft® Windows Vista™ operating system. WCN is part of the Microsoft Windows® Rally™ technologies that enable simple, secure discovery and configuration of network-connected devices.

With the growing popularity of wireless networking, more and more users are creating wireless networks at home to connect PCs and other wireless devices. Creating a secure wireless network and propagating the wireless settings to the clients and to access points (APs) has been difficult even for sophisticated users. WCN-UFD addresses this problem with a simple, secure method for configuring and bootstrapping wireless networking hardware, wireless client PCs, and other wireless devices.

The most visible part of the solution is the Wireless Network Setup Wizard, which was first provided in Microsoft Windows XP Service Pack 2. This wizard uses WCN-UFD to help the user create network settings, store them as XML, and propagate them to wireless devices by using USB flash drives (UFDs).

WCN-UFD improves the consumer experience with Windows and wireless networking by taking advantage of the growing popularity of UFDs and the broad adoption of USB host ports on consumer electronics devices, such as:

| Digital media receiver Digital still cameras Electronic picture frames Network printers Personal computers | Personal digital assistants (PDAs) Photo stations Printer bridges Set-top boxes Wireless access points |
| --- | --- |

References and resources discussed here are listed at the end of this specification.

# WCN-UFD File Architecture

Figure 1 shows the architecture supporting the wizard.

Figure 1. Architecture for Windows Vista Wireless Network Setup Wizard

As shown in Table 1, the wizard generates XML files on the UFD in the “SMRTNTKY” subfolder. The wizard copies an Autorun.inf file and a corresponding script file to configure additional wireless PCs. Each device that is configured generates an XML file to the drive in the \SMRTNTKY\DEVICE\ subfolder that contains configuration information for that device. The filename is autogenerated by each device using the last eight bytes of the MAC address using ASCII-hex—for example, A30f3cd6.wfc. A device configuration file is generated.

Table 1. Files Generated by the Wizard

| File | Description |
| --- | --- |
| Autorun.inf | Configuration file used to tell drive to launch the wizard. Note that Autorun.inf executes only if the media enumerates as a fixed drive or the operating system is Windows XP SP2 or later version. Windows XP SP2 supports Autorun on any media type. If an existing Autorun.inf file is present on the drive, it is renamed Autorun.fcb and renamed back to Autorun.inf when the drive is reinserted at the end of the wizard, if the user chooses to remove the settings from the drive. |
| Setupsnk.exe | Executable that runs when the UFD is installed in client PCs. |
| \SMRTNTKY\Wsetting.wfc | Wireless LAN settings. |
| \SMRTNTKY\DEVICE\A023ef83.wfc | Device Configuration file. Each device that is configured using WCN creates an XML file whose filename is based on the MAC address of the wireless network adapter (8-byte ASCII-hex representation of the MAC address). The XML file contains configuration information about the device. |
| \SMRTNTKY\Wsetting.txt | Text file describing process for configuration wireless settings when the Setupsnk.exe file is run on down-level PCs. |

As shown in Table 2, files and registry settings are created on the PC running the wizard and any subsequent PCs configured using the wizard.

Table 2. Registry Settings Created When the Wizard Runs

| Registry entry | Description |
| --- | --- |
| C:\Documents and Settings\<user> \Application Data\Microsoft\LastFlashConfig.wfc | Local copy of configuration file. If this file is present, then the wizard will allow the user to skip the wireless configuration pages in the wizard and simply create a WCN key with the settings from the last instance the wizard was run. |
| HKLM\SOFTWARE\Microsoft\FlashConfig\authentication | Registry entry to determine the default Network Key type. - A value of “1” = Wireless Protected Access Preshared Static Key (WPA-PSK). - Any other value is Wireless Equivalent Privacy (WEP). |
| Windows XP SP3: HKEY_LOCAL_MACHINE\SOFTWARE\Microsoft\FlashConfig\MultipleProfiles | Registry entry (DWORD) to determine if the wizard creates multiple profiles when the channel2Dot4 and channel5Dot0 elements are in the Wsetting.wfc file. If both elements are in schema, then the value of MultipleProfiles determines how many wireless profiles to create in the preferred list. Each profile uses the original service set identifier (SSID) with a “-n” suffix, where n = 2,3,4,5 …,n. - If n > 9, then n = 9 — meaning that no more than 9 profiles will ever be created. - If key is not present or set to 0 or 1, then 1 profile will be created. - If both elements are not in the schema, this value is ignored. |
| Windows XP SP3: HKEY_LOCAL_MACHINE\SOFTWARE\Microsoft\FlashConfig\ProfileOrder | Registry entry (BOOL) to determine if the order that wireless profiles are created in the preferred list when MultipleProfiles > 1. - If Profile Order is false or not present then the profiles are created in order so that the actual SSID profile (no suffix) is the first profile (highest priority) in the preferred list. - If Profile Order is true then the order is reversed and the profile with the actual original SSID from the Wsetting.wfc file is the last profile in the preferred list. |

## Autorun.inf

The Autorun.inf file is created on the root of the portable media. This configuration file is used to tell the operating system what application to run when the media is inserted. On operating systems earlier than Windows XP SP2, if the media is not DRIVE_FIXED, then Autorun.inf does not work and the user must manually execute the script file.

## Setupsnk.exe

The Setupsnk.exe file is created in the root of the portable media. This application is run by the Autorun.inf file when the portable media is installed on subsequent PCs. If the operating system is Windows XP SP2, then this application creates a wireless profile to match the schema on the portable media. For any other operating system, this application offers to print the wireless settings. If the drive does not enumerate as DRIVE_FIXED, then this script does not run on machines that are not based on Windows XP SP2 or later versions.

## Wsetting.wfc

The Wsetting.wfc file describes the wireless settings of an AP or wireless client device. Various elements in the XML schema are not required for a wireless client but may be used by an AP. The XML schema contains all known configuration options for APs, but all configuration options may not be supported. An example instance of the schema is included to show a typical output from the wizard.

### Wsetting.wfc  Elements

The following elements are included in the wireless configuration file. Some of the items (not in bold type) are included in the schema, but are not supported in the Windows XP SP2 release of the wizard.

Note  The order of the elements must be preserved or Windows invalidates the XML.

configId

A globally unique identifier (GUID) that is generated each time the wizard is executed. For the wizard to validate that the device was successfully configured, the device configuration file configId element must match this GUID.

configAuthorId

The GUID for the author of the instance of the schema.

configAuthor

The name of author of the instance of the schema.

ssid

The wireless beacon.

connectionType {ESS, IBSS}

The ad-hoc or infrastructure mode (default = ESS).

authenticateType

{open, shared, WPA, WPAPSK, WPA-NONE, WPA2, WPA2PSK} 
The authentication type for the wireless network (default = Open).

encryption

{none, WEP, TKIP, AES}
The encryption type for the wireless network.

networkKey

The wireless encryption key. 
Note  If WPA authentication is used, then the default Network Key is a 64-hex character key.

keyProvidedAutomatically

{true, false}

ieee802Dot1xEnabled

{true, false}

The following elements are included in the schema but are not supported in the Windows XP SP2 release of the wizard:

eapMethod

{EAP-TLS, PEAP-EAP-MSCHAPv2, PEAP-EAP-TLS}

An element that is used only for 802.1x authentication.

channel2Dot4

{0,1,2,3,4,5,6,7,8,9,10,11,12,13,14}

An optional element that is used to set the 2.4-GHz radio channel. A value of “0”  indicates “use default”.

channel5Dot0

{0,36, 40, 44, 48, 52, 56, 60, 64, 149, 153, 157, 161}

An optional element that is used to set the 5.0-GHz radio channel. A value of “0” indicates “use default”.

deviceMode

{infrastructure, bridge, repeater, station}

timeToLive

An optional element that can be used to set the length of time that a Network Key is valid.

The value is type integer and represents the number of seconds from the time that the device was configured with the new settings.

configHash

The checksum of the XML file.

keyIndex

{1-4}

### Wsetting.wfc Schema

The following schema is used to generate the Wsetting.wfc. Some elements and values in the schema are not supported by Windows. .

<?xml version="1.0" encoding="utf-8" ?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.microsoft.com/provisioning/WirelessProfile/2004"

xmlns:fc="http://www.microsoft.com/provisioning/WirelessProfile/2004" elementFormDefault="qualified"

version="1">

<xs:element name="wirelessProfile">

<xs:complexType>

<xs:sequence>

<xs:element name="config">

<xs:complexType>

<xs:sequence>

<xs:element name="configId" type="fc:str36" minOccurs="1" maxOccurs="1" />

<xs:element name="configHash" type="fc:str20" minOccurs="0" maxOccurs="1" />

<xs:element name="configAuthorId" type="fc:str36" minOccurs="1" maxOccurs="1" />

<xs:element name="configAuthor" type="fc:strMax128" minOccurs="1" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="ssid" type="fc:strMax32" minOccurs="1" maxOccurs="1" />

<xs:element name="connectionType" minOccurs="1" maxOccurs="1">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="IBSS" />

<xs:enumeration value="ESS" />

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="channel2Dot4" type="xs:positiveInteger" minOccurs="0" maxOccurs="1" />

<xs:element name="channel5Dot0" type="xs:positiveInteger" minOccurs="0" maxOccurs="1" />

<xs:element name="deviceMode" minOccurs="0" maxOccurs="1">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="infrastructure" />

<xs:enumeration value="bridge" />

<xs:enumeration value="repeater" />

<xs:enumeration value="station" />

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="primaryProfile" type="fc:profileInstance" minOccurs="1" maxOccurs="1" />

<xs:element name="optionalProfile" type="fc:profileInstance" minOccurs="0" maxOccurs="32" />

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:complexType name="profileInstance">

<xs:sequence>

<xs:element name="authentication" minOccurs="1" maxOccurs="1">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="open" />

<xs:enumeration value="shared" />

<xs:enumeration value="WPA-NONE" />

<xs:enumeration value="WPA" />

<xs:enumeration value="WPAPSK" />

<xs:enumeration value="WPA2" />

<xs:enumeration value="WPA2PSK" />

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="encryption" minOccurs="1" maxOccurs="1">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="none" />

<xs:enumeration value="WEP" />

<xs:enumeration value="TKIP" />

<xs:enumeration value="AES" />

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="networkKey" type="fc:strMin5Max64" minOccurs="1" maxOccurs="1" />

<xs:element name="keyIndex" type="xs:positiveInteger" minOccurs="0" maxOccurs="1" />

<xs:element name="keyProvidedAutomatically" type="xs:boolean" minOccurs="1" maxOccurs="1" />

<xs:element name="ieee802Dot1xEnabled" type="xs:boolean" minOccurs="1" maxOccurs="1" />

<xs:element name="eapMethod" minOccurs="0" maxOccurs="1">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="EAP-TLS" />

<xs:enumeration value="PEAP-EAP-MSCHAPv2" />

<xs:enumeration value="PEAP-EAP-TLS" />

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="timeToLive" type="xs:positiveInteger" minOccurs="0" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

<!-- Globals simpleTypes-->

<xs:simpleType name="str36">

<xs:restriction base="xs:string">

<xs:length value="36" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="strMax32">

<xs:restriction base="xs:string">

<xs:minLength value="1" />

<xs:maxLength value="32" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="str20">

<xs:restriction base="xs:string">

<xs:length value="20" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="strMin5Max64">

<xs:restriction base="xs:string">

<xs:maxLength value="64" />

<xs:minLength value="5" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="strMax128">

<xs:restriction base="xs:string">

<xs:maxLength value="128" />

<xs:minLength value="1" />

</xs:restriction>

</xs:simpleType>

</xs:schema>

### Wsetting.wfc Sample File (WEP Encryption)

<?xml version="1.0"?>

<wirelessProfile xmlns="http://www.microsoft.com/provisioning/WirelessProfile/2004">

<config>

<configId>DD102F7E-79AD-4272-AE92-A8559B1ACE43</configId>

<configAuthorId>0B4E59B8-8317-46B2-B8C2-C46850162E6C</configAuthorId>

<configAuthor>Microsoft Wireless Network Setup Wizard</configAuthor>

</config>

<ssid xml:space="preserve">FCTest</ssid>

<connectionType>ESS</connectionType>

<primaryProfile>

<authentication>open</authentication>

<encryption>WEP</encryption>

<networkKey xml:space="preserve">8e93898f4bdd42ce974b79b521</networkKey>

<keyProvidedAutomatically>0</keyProvidedAutomatically>

<ieee802Dot1xEnabled>0</ieee802Dot1xEnabled>

</primaryProfile>

</wirelessProfile>

### Wsetting.wfc Sample File (WPA Encryption)

<?xml version="1.0"?>

<wirelessProfile xmlns="http://www.microsoft.com/provisioning/WirelessProfile/2004">

<config>

<configId>000A013E-6740-439C-9935-EE58D85FA38A</configId>

<configAuthorId>0B4E59B8-8317-46B2-B8C2-C46850162E6C</configAuthorId>

<configAuthor>Microsoft Wireless Network Setup Wizard</configAuthor>

</config>

<ssid xml:space="preserve">FCTest324-2</ssid>

<connectionType>ESS</connectionType>

<primaryProfile>

<authentication>WPAPSK</authentication>

<encryption>TKIP</encryption>

<networkKey xml:space="preserve">adfafdgrhknakj fdankjnahiu rhyqa thnblajrtl985762987)</networkKey>

<keyProvidedAutomatically>0</keyProvidedAutomatically>

<ieee802Dot1xEnabled>0</ieee802Dot1xEnabled>

</primaryProfile>

</wirelessProfile>

## Device Configuration File (Xxxxxxxx.wfc)

Each device that is configured by using WCN writes a configuration file back to the UFD. The file name is based on the last 8 nibbles of the MAC address of the wireless network adapter that is being configured, represented in ASCII-hex. The file is copied to the \SMRTNTKY\DEVICE\ folder on the UFD.

If the MAC address of the network adapter is  00-08-0D-1A-DE-67, then the device configuration file name would be  0D1ADE67.WFC.

### Device Configuration File Elements

The following schema is a superset of the elements that the wizard supports. The wizard requires these elements for a valid implementation.

configId

A GUID that must match the value for the configId element in the Wsetting.wfc file that is used to configure the device.

manufacturer

The manufacturer of the device.

modelName

The model name of the device.

serialNumber

The serial number of the device.

firmwareVersion

The version of the firmware of the device.

configError

An indication that the device experienced an error when it was configured.

The value of this element can be 1 to 256 ASCII characters. This text appears on the finish page of the wizard.

deviceType

An element that is not required, but is strongly recommended.

The wizard uses the deviceType element to determine what category of device has been configured.

The remaining elements in the schema are optional and may be useful for diagnostic purposes. Future versions of the wizard may incorporate these elements.

Note  The order of the elements must be preserved or Windows invalidates the XML.

### Device Configuration File Schema

<?xml version="1.0" encoding="utf-8" ?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.microsoft.com/provisioning/DeviceProfile/2004"

xmlns:fc="http://www.microsoft.com/provisioning/DeviceProfile/2004" elementFormDefault="qualified"

version="1">

<xs:element name="device">

<xs:complexType>

<xs:sequence>

<xs:element name="configId"		type="fc:str36" minOccurs="1" maxOccurs="1" />

<xs:element name="configHash"	type="fc:str20" minOccurs="0" maxOccurs="1" />

<xs:element name="configError"	type="fc:strMax256" minOccurs="0" maxOccurs="1" />

<xs:element name="manufacturer" type="fc:strMax256" minOccurs="1" maxOccurs="1" />

<xs:element name="manufacturerURL" type="fc:anyURIMax2084" minOccurs="0" maxOccurs="1" />

<xs:element name="deviceStatus" type="fc:strMax256" minOccurs="0" maxOccurs="1" />

<xs:element name="modelName"	type="fc:strMax256" minOccurs="1" maxOccurs="1" />

<xs:element name="modelURL"		type="fc:anyURIMax2084" minOccurs="0" maxOccurs="1" />

<xs:element name="serialNumber" type="fc:strMax256" minOccurs="1" maxOccurs="1" />

<xs:element name="firmwareVersion" type="fc:strMax256" minOccurs="0" maxOccurs="1" />

<xs:element name="presentationURL" type="fc:anyURIMax2084" minOccurs="0" maxOccurs="1" />

<xs:element name="deviceType" fc:type="deviceTypes" minOccurs="0" maxOccurs="1" />

<!-- Configuration information if the device is a member of a workgroup -->

<xs:element name="workgroup" minOccurs="0" maxOccurs="1">

<xs:complexType>

<xs:sequence>

<xs:element name="deviceName"		type="fc:strName" minOccurs="0" maxOccurs="1" />

<xs:element name="workgroupName"	type="fc:strName" minOccurs="0" maxOccurs="1" />

<xs:element name="deviceDescription" type="fc:strDescription" minOccurs="0" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

</xs:element>

<!-- What network connections does this device support -->

<xs:element name="networkInterface" minOccurs="0" maxOccurs="32">

<xs:complexType>

<xs:sequence>

<xs:element name="IPv4Address" type="fc:dotQuadIPv4Type" minOccurs="0" maxOccurs="1" />

<xs:element name="IPv6Address" type="fc:textIPv6AddressType" minOccurs="0" maxOccurs="32" />

<xs:element name="macAddress" type="fc:macAddress" minOccurs="1" maxOccurs="1" />

<xs:element name="interfaceStatus" type="fc:interfaceStatuses" maxOccurs="1" minOccurs="1" />

<xs:element name="mediaType" type="fc:mediaTypes" minOccurs="1" maxOccurs="1" />

<xs:element name="speed" type="xs:positiveInteger" minOccurs="1" maxOccurs="1" />

<xs:element name="DHCP" type="xs:boolean" minOccurs="1" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

</xs:element>

<!-- General WiFi Configuration information -->

<xs:element name="wifiAuthentication" minOccurs="0" maxOccurs="1">

<xs:complexType>

<xs:sequence>

<xs:element name="open" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="shared" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="WPA-NONE" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="WPA" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="WPAPSK" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="WPA2" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="WPA2PSK" type="xs:boolean" minOccurs="0" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="wifiEncryption" minOccurs="0" maxOccurs="1">

<xs:complexType>

<xs:sequence>

<xs:element name="none" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="WEP" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="TKIP" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="AES" type="xs:boolean" minOccurs="0" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

</xs:element>

<!-- Configuration information if the device is a WiFi client -->

<xs:element name="wifiClient" minOccurs="0" maxOccurs="1">

<xs:complexType>

<xs:sequence>

<xs:element name="regulatoryDomain" type="fc:str3" minOccurs="0" maxOccurs="1" />

<xs:element name="transmitPower" type="fc:intPowerStrength" minOccurs="0" maxOccurs="1" />

<xs:element name="clientNetworkType">

<xs:complexType>

<xs:sequence>

<xs:element name="ieee802Dot11a" type="xs:boolean" />

<xs:element name="ieee802Dot11b" type="xs:boolean" />

<xs:element name="ieee802Dot11g" type="xs:boolean" />

</xs:sequence>

</xs:complexType>

</xs:element>

</xs:sequence>

</xs:complexType>

</xs:element>

<!-- Configuration information of wireless networks visible to device -->

<xs:element name="wifiInfrastructure" minOccurs="0" maxOccurs="100">

<xs:complexType>

<xs:sequence>

<xs:element name="ssid" type="fc:strMax32" minOccurs="1" maxOccurs="1" />

<xs:element name="macAddress" type="fc:macAddress" minOccurs="1" maxOccurs="1" />

<xs:element name="visibleNetworkType" type="fc:visibleNetworkTypes" minOccurs="1" maxOccurs="1" />

<xs:element name="channel" type="xs:positiveInteger" minOccurs="1" maxOccurs="1" />

<xs:element name="signalStrength" type="fc:intPowerStrength" minOccurs="0" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

</xs:element>

<!-- Configuration information if the device is a WiFi access point -->

<xs:element name="wifiAccessPoint" minOccurs="0" maxOccurs="1">

<xs:complexType>

<xs:sequence>

<xs:element name="ieee802Dot11a" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="ieee802Dot11b" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="ieee802Dot11g" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="regulatoryDomain" type="fc:str3" minOccurs="0" maxOccurs="1" />

<xs:element name="macFilter" type="fc:macAddress" minOccurs="0" maxOccurs="252" />

<xs:element name="securityType" minOccurs="0" maxOccurs="1">

<xs:complexType>

<xs:sequence>

<xs:element name="basic" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="WPA" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="WPA2" type="xs:boolean" minOccurs="0" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="beaconInterval" type="fc:intBeaconInterval" minOccurs="0" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

</xs:element>

</xs:sequence>

</xs:complexType>

</xs:element>

<!-- Globals simpleTypes-->

<xs:simpleType name="dotQuadIPv4Type">

<xs:restriction base="xs:string">

<xs:pattern value="([\d]{1,3}.){3}[\d]{1,3}" />

<xs:minLength value="7" />

<xs:maxLength value="15" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="textIPv6AddressType">

<xs:restriction base="xs:string">

<xs:pattern value="(([\dA-Fa-f]{1,4}:){7}[\dA-Fa-f]{1,4})(:([\d]{1,3}.){3}[\d]{1,3})?" />

<xs:minLength value="16" />

<xs:maxLength value="39" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="macAddress">

<xs:restriction base="xs:hexBinary">

<xs:length value="6" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="anyURIMax2084">

<xs:restriction base="xs:string">

<xs:minLength value="1" />

<xs:maxLength value="2084" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="strMax256">

<xs:restriction base="xs:string">

<xs:maxLength value="256" />

<xs:minLength value="1" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="str36">

<xs:restriction base="xs:string">

<xs:length value="36" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="strMax32">

<xs:restriction base="xs:string">

<xs:maxLength value="32" />

<xs:minLength value="1" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="str20">

<xs:restriction base="xs:string">

<xs:length value="20" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="str3">

<xs:restriction base="xs:string">

<xs:length value="3" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="strName">

<xs:restriction base="xs:string">

<xs:minLength value="1" />

<xs:maxLength value="15" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="strDescription">

<xs:restriction base="xs:string">

<xs:minLength value="1" />

<xs:maxLength value="50" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="intBeaconInterval">

<xs:restriction base="xs:positiveInteger">

<xs:maxInclusive value="3600" />

<xs:minInclusive value="1" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="intPowerStrength">

<xs:restriction base="xs:integer">

<xs:minInclusive value="-200" />

<xs:maxInclusive value="-20" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="deviceTypes">

<xs:restriction base="xs:string">

<xs:enumeration value="Computer" />

<xs:enumeration value="Access_Point" />

<xs:enumeration value="Printer" />

<xs:enumeration value="Electronic_Picture_Frame" />

<xs:enumeration value="Digital_Audio_Receiver" />

<xs:enumeration value="Windows_Media_Center_Extender" />

<xs:enumeration value="Personal_Video_Recorder" />

<xs:enumeration value="Printer_Bridge" />

<xs:enumeration value="Projector" />

<xs:enumeration value="Pocket_PC" />

<xs:enumeration value="Other" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="interfaceStatuses">

<xs:restriction base="xs:string">

<xs:enumeration value="active" />

<xs:enumeration value="disabled" />

<xs:enumeration value="noMedia" />

<xs:enumeration value="authenticating" />

<xs:enumeration value="media" />

<xs:enumeration value="noSignal" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="mediaTypes">

<xs:restriction base="xs:string">

<xs:enumeration value="802.3" />

<xs:enumeration value="1394" />

<xs:enumeration value="802.11" />

<xs:enumeration value="usb" />

<xs:enumeration value="bluetooth" />

<xs:enumeration value="other" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="visibleNetworkTypes">

<xs:restriction base="xs:string">

<xs:enumeration value="ieee802Dot11a" />

<xs:enumeration value="ieee802Dot11b" />

<xs:enumeration value="ieee802Dot11g" />

</xs:restriction>

</xs:simpleType>

</xs:schema>

### Sample Device Configuration File

<?xml version="1.0"?>

<device xmlns="http://www.microsoft.com/provisioning/DeviceProfile/2004">

<configId>DD102F7E-79AD-4272-AE92-A8559B1ACE43</configId>

<manufacturer>Microsoft</manufacturer>

<modelName>Windows XP</modelName>

<serialNumber>MyLaptop</serialNumber>

<firmwareVersion>SP2</firmwareVersion>

<deviceType>Computer</deviceType>

</device>

## Dual Radio APs

The WCN wizard in Windows Vista does not support unique configuration of dual radio access points or wireless clients. To work around this limitation, the following logic is proposed:

1.	If a Wsetting.wfc file does not contain both the channel2Dot4 and channel5Dot0 elements, then the AP should configure both radios identically— that is, the same SSID and Network Key).

2.	If a Wsetting.wfc file contains the channel2Dot4 and channel5Dot0 elements, then the AP should configure the 5.0-GHz radio to the SSID value that the file specifies. The 2.4-GHz radio should be configured to the same SSID with a “-2” appended. If the original SSID is greater than 30 characters, then it should be truncated to 30 characters and a “-2” added to the end. All of the remaining configuration elements should remain the same.

The following is an example Wsetting.wfc file that indicates support for unique SSID names for dual radio APs.

<?xml version="1.0"?>

<wirelessProfile xmlns="http://www.microsoft.com/provisioning/WirelessProfile/2004">

<config>

<configId>000A013E-6740-439C-9935-EE58D85FA38A</configId>

<configAuthorId>0B4E59B8-8317-46B2-B8C2-C46850162E6C</configAuthorId>

<configAuthor>Microsoft Wireless Network Setup Wizard</configAuthor>

</config>

<ssid xml:space="preserve">My Wireless Dual Radio</ssid>

<connectionType>ESS</connectionType>

<channel2Dot4>0</channel2Dot4>

<channel5Dot0>0</channel5Dot0>

<primaryProfile>

<authentication>WPAPSK</authentication>

<encryption>TKIP</encryption>

<networkKey xml:space="preserve">adfafdgrhknakj fdankjnahiu rhyqa thnblajrtl985762987)</networkKey>

<keyProvidedAutomatically>0</keyProvidedAutomatically>

<ieee802Dot1xEnabled>0</ieee802Dot1xEnabled>

</primaryProfile>

</wirelessProfile>

In the previous example, the AP configures the 5.0-GHz radio to the following settings:

- SSID: My Wireless Dual Radio

- Network key: adfafdgrhknakj fdankjnahiu rhyqa thnblajrtl985762987)

The 2.4-GHz radio is configured to the following settings:

- SSID: “My Wireless Dual Radio-2”

- Network key: adfafdgrhknakj fdankjnahiu rhyqa thnblajrtl985762987)

The following example shows a Wsetting.wfc file that does not support unique SSID names. Note that the channel2Dot4 and channel5Dot0 elements are not present.

<?xml version="1.0"?>

<wirelessProfile xmlns="http://www.microsoft.com/provisioning/WirelessProfile/2004">

<config>

<configId>000A013E-6740-439C-9935-EE58D85FA38A</configId>

<configAuthorId>0B4E59B8-8317-46B2-B8C2-C46850162E6C</configAuthorId>

<configAuthor>Microsoft Wireless Network Setup Wizard</configAuthor>

</config>

<ssid xml:space="preserve">My Wireless Dual Radio</ssid>

<connectionType>ESS</connectionType>

<primaryProfile>

<authentication>WPAPSK</authentication>

<encryption>TKIP</encryption>

<networkKey xml:space="preserve">adfafdgrhknakj fdankjnahiu rhyqa thnblajrtl985762987)</networkKey>

<keyProvidedAutomatically>0</keyProvidedAutomatically>

<ieee802Dot1xEnabled>0</ieee802Dot1xEnabled>

</primaryProfile>

</wirelessProfile>

In the previous example, the AP configures the 2.4-GHz and the 5.0-GHz radio to the same settings. In the following example, both radios are configured as shown:

- SSID: My Wireless Dual Radio

- Network key: adfafdgrhknakj fdankjnahiu rhyqa thnblajrtl985762987)

If the AP supports dual radios, then it should write two device configuration files (one representing each radio). If the AP has only one MAC address for the two radios, then the second device configuration file name should be based on the MAC address but incremented in value to make it unique. For example:

- If the MAC address of the network adapter is 00-08-0D-1A-DE-67, then the device configuration file name would be 0D1ADE67.WFC.

- The file name for the second radio would be OD1ADE68.WFC.

# Device and Software Requirements

WCN-compliant devices and software must support the features listed in this section.

## Requirements for USB Flash Drives

WCN-compliant devices should work with a variety of USB flash drives to ensure compatibility. There have been many variations of UFDs with a variety of different characteristics—drive type, Master Boot Record, capacity, and so on.  The following shows the recommended configuration.

Recommended Configuration for UFD Device Compatibility with WCN

- USB Interfaced Class 0x8

- USB Mass Storage Subclass 0x6

- USB Interface Protocol 0x50

- USB Mass Storage INQUIRY RMB BIT value of 0x1

- Raw capacity between 200K and 10 GB

- FAT12, FAT16, FAT32

- Master Boot Record

- Maximum current sink of 150 mA

- A single partition

Hardware Requirements for WCN-compliant Devices

WCN devices must support specific hardware requirements to maintain compliance with the WCN licensing agreement and ensure interoperability with WCN implementations in Windows and UFD devices.

- Easily accessible USB host port (USB version 1.1 or later)

- WPA-PSK 64-byte hex Network Keys

- Standard UFDs

- USB Mass Storage Subclass 0x6

- USB Interface Protocol 0x50

- USB Mass Storage INQUIRY RMB bit value of 0x1 or 0x0

- Raw capacity between 200K and 10 GB

- FAT12, FAT16, FAT32 format

- A Master Boot Record present or not present

- Source up to 200 mA on a USB host port

- Multiple partitions. The WCN device should scan each partition of the UFD looking for the WCN configuration files.

Performance Requirements for WCN-compliant Devices

When a WCN device has been successfully configured, it should confirm a successful configuration by flashing the appropriate LEDs three times with a 1-Hz cycle defined as follows:

0.5 sec ON, 0.5 sec OFF, 0.5 sec ON, 0.5 sec OFF, 0.5 sec ON, then OFF

XXXXX_____XXXXX_____XXXXX

When a WCN device is unsuccessful at reading or writing the WCN configuration file, it should respond with a sequence of flashes: two short flashes (0.3-second on duration) and one long flash (1-second on duration) with a 0.3-second delay between all flashes—and then repeat until the SmartKey is removed (or for at least three full cycles). For example:

0.3 sec ON, 0.3 sec OFF, 0.3 sec ON, 0.3 sec OFF, 1 sec ON, 0.3 sec OFF

XXX___XXX___XXXXXXXXXXXX___XXX___XXX___XXXXXXXXXXXX___XXX___XXX___XXXXXXXXXXXX___XXX___XXX___XXXXXXXXXXXX....

## Firmware and Software Requirements

WCN-compliant devices must understand and parse the XML configuration files that are stored on the UFD. Additionally, the device must write an XML configuration file back to the UFD to confirm a successful configuration or provide an explanation for a failure. The Windows Wireless Network Setup Wizard does not confirm any device that writes a configuration file that does not conform to the device configuration schema. If a Wsetting.wfc file does not conform to the schema, then the Wizard ignores the file and does not successfully create a wireless profile.

# UFD Configuration with Earlier Versions of Windows

The “Network Setup” wizard and the “Add a device to the network” wizard use WCN capabilities to configure the network. Configuration using USB flash drives is available  using the Wireless Network Setup Wizard that was first provided in Windows XP SP2. The wizard copies a configuration application to the UFD. This wizard runs automatically if the UFD device enumerates as DRIVE_FIXED or if the machine is running Windows Vista or Windows XP SP2.

The wizard creates a wireless profile on Windows Vistas-based or Windows XP SP2-based machines and provides the user an option to print the settings on down-level machines—that is, machines running Windows 98 through Windows XP SP1. Table 3 describes the behavior on different versions of Windows using UFDs that enumerate as DRIVE_FIXED and DRIVE_REMOVEABLE.

Table 3. UFD Behavior on Earlier Versions of Windows

| Windows version | Drive type | Result |
| --- | --- | --- |
| Windows XP SP2 | DRIVE_FIXED | Autoplay lists Wireless Setup Wizard as top option. |
| Windows XP (before SP2) | DRIVE_FIXED | Autorun runs Setupsnk.exe automatically. |
| Windows 2000 | DRIVE_FIXED | Autorun runs Setupsnk.exe automatically. |
| Windows Me | DRIVE_FIXED | Autorun runs Setupsnk.exe automatically. |
| Windows 98 | DRIVE_FIXED | Autorun runs Setupsnk.exe automatically. |
| Windows XP SP2 | DRIVE_REMOVABLE | Autoplay lists Wireless Setup Wizard as top option. |
| Windows XP (before SP2) | DRIVE_REMOVABLE | Autorun does not run. |
| Windows 2000 | DRIVE_REMOVABLE | Autorun does not run. |
| Windows Me | DRIVE_REMOVABLE | Autorun does not run. |
| Windows 98 | DRIVE_REMOVABLE | Autorun does not run. |

# References

E-mail contact:

Rally @ microsoft.com
For information about plans for extensions to the WCN architecture and proposed schema, please write to the rally e-mail address.

Windows Rally specifications and white papers

http://www.microsoft.com/rally