A WINDOWS® RALLY™ SPECIFICATION

Windows Connect Now-UFD for Windows XP

Abstract

This specification provides implementation details for manufacturers who design wireless networking products that work with the Microsoft® Windows® Connect Now-UFD (WCN-UFD) configuration feature of the Microsoft Windows XP operating system. Windows Connect Now is part of the Microsoft Windows Rally™ technologies that enable simple, secure discovery and configuration of network-connected devices.

Version 1.1 – September 15, 2006

LICENSE NOTICE. Access to and viewing and implementation of the technology described in this document is granted under the Microsoft Windows Rally Program License Agreement (“License Agreement”). If you want a license from Microsoft to access, view or implement one or more Licensed Technologies, you must complete the designated information in the License Agreement and return a signed copy to Microsoft. The License Agreement is provided at the end of this document. If the License Agreement is not available with this document, you can download a copy from the Windows Rally Web site at http://www.microsoft.com/rally.

Disclaimer

This is a preliminary document and may be changed substantially prior to final commercial release of the software described herein.

The information contained in this document represents the current view of Microsoft Corporation on the issues discussed as of the date of publication. Because Microsoft must respond to changing market conditions, it should not be interpreted to be a commitment on the part of Microsoft, and Microsoft cannot guarantee the accuracy of any information presented after the date of publication.

This White Paper is for informational purposes only. MICROSOFT MAKES NO WARRANTIES, EXPRESS, IMPLIED OR STATUTORY, AS TO THE INFORMATION IN THIS DOCUMENT.

Complying with all applicable copyright laws is the responsibility of the user. Without limiting the rights under copyright, no part of this document may be reproduced, stored in or introduced into a retrieval system, or transmitted in any form or by any means (electronic, mechanical, photocopying, recording, or otherwise), or for any purpose, without the express written permission of Microsoft Corporation.

Microsoft may have patents, patent applications, trademarks, copyrights, or other intellectual property rights covering subject matter in this document. Except as expressly provided in any written license agreement from Microsoft, the furnishing of this document does not give you any license to these patents, trademarks, copyrights, or other intellectual property.

Unless otherwise noted, the example companies, organizations, products, domain names, e-mail addresses, logos, people, places and events depicted herein are fictitious, and no association with any real company, organization, product, domain name, email address, logo, person, place or event is intended or should be inferred.

© 2006 Microsoft Corporation.  All rights reserved.

Microsoft, Rally, Windows, Windows NT, Windows Server, and Windows Vista are either registered trademarks or trademarks of Microsoft Corporation in the United States and/or other countries.

The names of actual companies and products mentioned herein may be the trademarks of their respective owners.

The current version of this specification is maintained on the Web at: 
http://www.microsoft.com/rally

Revision History

| Date | Revision |
| --- | --- |
| April 25, 2005 | Previously published as “Windows Connect Now Architecture” |
| May 2006 | Windows Rally technology licensing information provided. |

Contents

Introduction: WCN Flash Configuration	4

WCN Flash Configuration Architecture	5

LastFlashConfig.wfc	7

Write LastFlashConfig.wfc API	7

Read LastFlashConfig.wfc API	8

LastFlashConfig.wfc file notification	9

Create Wireless Profile API	10

Autorun.inf	11

Setupsnk.exe	12

Wsetting.wfc	12

Wsetting.wfc  Elements	12

Wsetting.wfc Schema	13

Wsetting.wfc Sample File (WEP Encryption)	15

Wsetting.wfc Sample File (WPA Encryption)	16

Device Configuration File (Xxxxxxxx.wfc)	16

Device Configuration File Elements	16

Device Configuration File Schema	17

Sample Device Configuration File	22

Dual Radio APs	22

WCN Configuration Support for Devices without USB Host Ports	24

Hardware Requirements	24

Requirements for USB Flash Drives	24

Requirements for WCN Devices	25

Firmware and Software Requirements	25

UFD Configuration with Earlier Versions of Windows	25

References	26

- Introduction: WCN Flash Configuration

This specification provides implementation details for manufacturers who design wireless networking products that work with the Windows Connect Now (WCN) Flash Configuration feature of the Microsoft® Windows® XP operating system. WCN is part of the Windows Rally™ technologies that enable simple, secure discovery and configuration of network-connected devices.

With the growing popularity of wireless networking, more and more users are creating wireless networks at home to connect PCs and other wireless devices. Creating a secure wireless network and propagating the wireless settings to the clients and to access points (APs) has been difficult even for sophisticated users. WCN addresses this problem with a simple, secure method for configuring and bootstrapping wireless networking hardware, wireless client PCs, and other wireless devices.

The most visible part of the solution is the Wireless Network Setup Wizard in Windows XP Service Pack 2. This wizard uses WCN to help the user create network settings, store them as XML, and propagate them to wireless devices by using USB flash drives (UFDs). This document defines the XML schema that is used to transfer settings for wide area network (WANs), local area networks (LANs), and wireless local area networks (WLANs) that the wizard creates. Although the first implementation of this wizard for Windows XP SP2 generates only the wireless schema, future versions will include the LAN and WAN settings.

This architecture defines a method for easily bootstrapping network devices on a home network:

- Portable media is used to transport network settings from a Windows-based PC to networked devices in the home (USB flash drives).

- This architecture is complementary to but not dependent on WS-Security. There is no dependency on UPnP v. 1.0 security or on RADIUS authentication.

WCN Flash Configuration improves the consumer experience with Windows and wireless networking by taking advantage of the growing popularity of UFDs and the broad adoption of USB host ports on consumer electronics devices, such as:

| Digital media receiver Digital still cameras Electronic picture frames Network printers Personal computers | Personal digital assistants (PDAs) Photo stations Printer bridges Set-top boxes Wireless access points |
| --- | --- |

References and resources discussed here are listed at the end of this specification.

# WCN Flash Configuration Architecture

Figure 1 shows the architecture supporting the wizard.

Figure 1. Architecture for Windows XP Wireless Network Setup Wizard

As shown in Table 1, the wizard generates XML files on the UFD in the “SMRTNTKY” subfolder. The wizard copies an Autorun.inf file and a corresponding script file to configure additional wireless PCs. Each device that is configured generates an XML file to the drive in the \SMRTNTKY\DEVICE\ subfolder that contains configuration information for that device. The filename is autogenerated by each device using the last eight bytes of the MAC address using ASCII-hex—for example, A30f3cd6.wfc. A device configuration file is generated.

Table 1. Files Generated by the Wizard

| File | Description |
| --- | --- |
| Autorun.inf | Configuration file used to tell drive to launch the wizard. Note: Autorun.inf will only execute if the media enumerates as a fixed drive or the operating system is Windows XP SP2 or later version. Windows XP SP2 supports Autorun on any media type. If an existing Autorun.inf file is present on the drive, it is renamed Autorun.fcb and renamed back to Autorun.inf when the drive is reinserted at the end of the wizard, if the user chooses to remove the settings from the drive. |
| Setupsnk.exe | Executable that runs when the UFD is installed in client PCs. |
| \SMRTNTKY\Wsetting.wfc | Wireless LAN settings. |
| \SMRTNTKY\DEVICE\A023ef83.wfc | Device Configuration file. Each device that is configured using WCN creates an XML file whose filename is based on the MAC address of the wireless network adapter (8-byte ASCII-hex representation of the MAC address). The XML file contains configuration information about the device. |
| \SMRTNTKY\Wsetting.txt | Text file describing process for configuration wireless settings when the Setupsnk.exe file is run on down-level PCs. |

As shown in Table 2, files and registry settings are created on the PC running the wizard and any subsequent Windows XP SP2-based PCs configured using the wizard.

Table 2. Registry Settings Created When the Wizard Runs

| Registry entry | Description |
| --- | --- |
| C:\Documents and Settings\<user> \Application Data\Microsoft\LastFlashConfig.wfc | Local copy of configuration file. If this file is present, then the wizard will allow the user to skip the wireless configuration pages in the wizard and simply create a WCN key with the settings from the last instance the wizard was run. |
| HKLM\SOFTWARE\Microsoft\FlashConfig\authentication | Registry entry to determine the default network key type. - A value of “1” = Wireless Protected Access Preshared Static Key (WPA-PSK). - Any other value is Wireless Equivalent Privacy (WEP). |
| Windows XP SP3: HKEY_LOCAL_MACHINE\SOFTWARE\Microsoft\FlashConfig\MultipleProfiles | Registry entry (DWORD) to determine if the wizard creates multiple profiles when the channel2Dot4 and channel5Dot0 elements are in the Wsetting.wfc file. If both elements are in schema, then the value of MultipleProfiles determines how many wireless profiles to create in the preferred list. Each profile uses the original service set identifier (SSID) with a “-n” suffix, where n = 2,3,4,5 …,n. - If n > 9, then n = 9 — meaning that no more than 9 profiles will ever be created. - If key is not present or set to 0 or 1, then 1 profile will be created. - If both elements are not in the schema, this value is ignored. |
| Windows XP SP3: HKEY_LOCAL_MACHINE\SOFTWARE\Microsoft\FlashConfig\ProfileOrder | Registry entry (BOOL) to determine if the order that wireless profiles are created in the preferred list when MultipleProfiles > 1. - If Profile Order is false or not present then the profiles are created in order so that the actual SSID profile (no suffix) is the first profile (highest priority) in the preferred list. - If Profile Order is true then the order is reversed and the profile with the actual original SSID from the Wsetting.wfc file is the last profile in the preferred list. |

The following sections describe more details about these files and the related APIs.

## LastFlashConfig.wfc

When the wizard is launched, it searches for LastFlashConfig.wfc on the PC. If this file is present, the user is prompted to either add a new device to the existing network defined in the LastFlashConfig.wfc file or create a new one.

In the previous example, the file LastFlashConfig.wfc was present in the C:\Documents and Settings\<user>\Application Data\Microsoft\ folder. The file is a duplicate of the WSetting.wfc file that was created when the wizard last ran. This file is encrypted using the Windows Cypto APIs (see the following sample code).

The file is stored in a subfolder in the user’s personal file folder.  Only the user who ran the wizard is allowed to propagate the settings. The LastFlashConfig.wfc file is created on any machine that is provisioned by the wizard. Therefore, any machine that has been provisioned can be used to propagate the settings to other PCs or devices.

### Write LastFlashConfig.wfc API

The following sample shows how to write the encrypted and hidden LastFlashConfig.wfc file:

HRESULT EncryptAndWriteData (IXMLDOMNode * pNode)

{

// assumes directory already exists.
    WCHAR szFileName[MAX_PATH*2];

if (FALSE == ::SHGetSpecialFolderPath (NULL, szFileName, CSIDL_APPDATA, FALSE))

return HRESULT_FROM_WIN32 (GetLastError());

HRESULT hr = StringCchCat (szFileName, MAX_PATH*2, L"\\Microsoft\\LastFlashConfig.WFC");

if (FAILED(hr))

return hr;

CComBSTR bstr;

hr = pNode->get_xml (&bstr); // get the XML data, as a string

if (SUCCEEDED(hr)) {

DATA_BLOB blobIn, blobOut = {0};

// put the plaintext in a data_blob

blobIn.cbData = SysStringByteLen (bstr.m_str);

blobIn.pbData = (BYTE *)bstr.m_str;

// encrypt

BOOL b = CryptProtectData (&blobIn,  // plaintext

NULL,     // description

NULL,     // entropy

NULL,     // reserved

NULL,     // prompt struct

CRYPTPROTECT_UI_FORBIDDEN,

&blobOut);

if  (TRUE != b)

hr = HRESULT_FROM_WIN32 (GetLastError());

else {

// write out the cypher text (in bloblOut)

hr = WriteOutYourFile (szFileName,

blobOut.pbData,

blobOut.cbData);

LocalFree (blobOut.pbData);

// hide file

SetFileAttributes (szFileName, FILE_ATTRIBUTE_HIDDEN);

}

}

return hr;

}

### Read LastFlashConfig.wfc API

The following sample shows how to read the LastFlashConfig.wfc file from the local machine.

HRESULT DecryptAndUseLastConfig()

{

// get filename 
    WCHAR szFileName[MAX_PATH*2];
    if (FALSE == ::SHGetSpecialFolderPath (NULL, szFileName, CSIDL_APPDATA, FALSE))

return HRESULT_FROM_WIN32 (GetLastError());

HRESULT hr = StringCchCat (szFileName, MAX_PATH*2, L"\\Microsoft\\LastFlashConfig.WFC");

if (FAILED(hr))

return hr;

// read file into blob
    DATA_BLOB blobIn = {0};

HANDLE hFile = CreateFile (szFileName,          // filename
                               GENERIC_READ,        // open for read/write 
                               0,                   // do not share 
                               NULL,                // default security 
                               OPEN_EXISTING,       // open existing file
                               FILE_ATTRIBUTE_NORMAL,
                               NULL);               // no template 
    if (hFile == INVALID_HANDLE_VALUE)
        hr = HRESULT_FROM_WIN32 (GetLastError());
    else {
        // seek to see how big a buffer to allocate
        blobIn.cbData = SetFilePointer (hFile, 0L, 0L, FILE_END);
        SetFilePointer (hFile, 0L, 0L, FILE_BEGIN);

// allocate space 
        blobIn.pbData = (BYTE *)malloc (blobIn.cbData);
        if (!blobIn.pbData)
            hr = E_OUTOFMEMORY;
        else {
            DWORD dwBytesRead = 0;
            BOOL b = ReadFile (hFile,
                               (LPVOID)blobIn.pbData,
                               blobIn.cbData,
                               &dwBytesRead,
                               NULL);
            if ((FALSE == b) || (dwBytesRead != blobIn.cbData))
                hr = HRESULT_FROM_WIN32 (GetLastError());
            CloseHandle (hFile);
        }
    }

if (SUCCEEDED(hr)) {
        // decrypt
        DATA_BLOB blobOut = {0};
        BOOL b = CryptUnprotectData (&blobIn,  // plaintext
                                     NULL,     // description
                                     NULL,     // entropy
                                     NULL,     // reserved
                                     NULL,     // prompt struct
                                     CRYPTPROTECT_UI_FORBIDDEN,
                                     &blobOut);
        if  (TRUE != b)
            hr = HRESULT_FROM_WIN32 (GetLastError());
        else {
            // use the data here
            UseXmlData (blobOut.cbData/sizeof(OLECHAR), (LPCOLESTR)blobOut.pbData);

LocalFree (blobOut.pbData);
        }
    }
    
    free (blobIn.pbData);

return hr;

}

### LastFlashConfig.wfc file notification

The following sample shows how an application or service can listen for changes to the LastFlashConfig.wfc file. This can be useful as it gives an indication that the user has changed the wireless profile; wireless clients or applications can read the new file and take appropriate actions.

HRESULT WakeUpWhenLastConfigFileChanges()

{

HRESULT hr = S_OK;

// get path
    WCHAR szPathName[MAX_PATH*2];
    ::SHGetSpecialFolderPath (NULL, szPathName, CSIDL_APPDATA, FALSE);
    StringCchCat (szPathName, MAX_PATH*2, L"\\Microsoft");

// get file
    WCHAR szFileName[MAX_PATH*2];
    ::SHGetSpecialFolderPath (NULL, szFileName, CSIDL_APPDATA, FALSE);
    StringCchCat (szFileName, MAX_PATH*2, L"\\Microsoft\\LastFlashConfig.WFC");

// get last write time so I can do a quick check

// to see if my file changed
    WIN32_FILE_ATTRIBUTE_DATA wfad = {0};
    GetFileAttributesEx (szFileName,
                         GetFileExInfoStandard,
                         (LPVOID)&wfad);
    FILETIME ft = wfad.ftLastWriteTime;

// set up notification
    HANDLE hNotify = FindFirstChangeNotification (szPathName,
                                                  FALSE,
                                                  FILE_NOTIFY_CHANGE_LAST_WRITE);
    if (hNotify == INVALID_HANDLE_VALUE) {
        hr = HRESULT_FROM_WIN32 (GetLastError();
    } else {
        do {
            DWORD dwWaitStatus = WaitForSingleObject (hNotify, INFINITE);
            if (dwWaitStatus != WAIT_OBJECT_0) {

// abandoned, timed out or failed

hr = E_FAILED;
                break;

}

// see if my file changed
            GetFileAttributesEx (szFileName,
                                 GetFileExInfoStandard,
                                 (LPVOID)&wfad);
            if ((ft.dwLowDateTime  == wfad.ftLastWriteTime.dwLowDateTime) &&
                (ft.dwHighDateTime == wfad.ftLastWriteTime.dwHighDateTime))
                continue;   // not my file

// update my time for next time
            ft.dwLowDateTime  = wfad.ftLastWriteTime.dwLowDateTime;
            ft.dwHighDateTime = wfad.ftLastWriteTime.dwHighDateTime;

// do something
            WakeUpAndDoSomething();

} while (FindNextChangeNotification (hNotify));

FindCloseChangeNotification (hNotify);
    }

return hr;

}

### Create Wireless Profile API

The following sample shows how to create a wireless profile in the preferred list of WZC using the Wireless Provisioning API. Note that the schema for this API is slightly different from that of the Wsetting.wfc file. If a duplicate SSID already exists then the API will return an error message.

HRESULT CreateASampleProfile (GUID * pGuid)
{   
    CComPtr<IProvisioningProfileWireless> spNetProvProfW = NULL;
    HRESULT hr = CoCreateInstance (__uuidof(NetProvisioning),
                                   NULL,
                                   CLSCTX_ALL,
                                   __uuidof(IProvisioningProfileWireless),
                                   (void**)&spNetProvProfW);
    if (FAILED(hr))
        return hr;

CString szXML;

szXML += L"<?xml version=\"1.0\"?>\n";
    szXML += L"<WirelessProfile xmlns=\"http://www.microsoft.com/provisioning/WirelessProfile\">\n";

// version
    szXML += L"\t<version>";
    szXML += L"1";
    szXML += L"</version>\n";

// ssid
    szXML += L"\t<ssid xml:space=\"preserve\">";
    szXML += Escape (L"my ssid");
    szXML += L"</ssid>\n";

// connectionType
    szXML += L"\t<connectionType>";
    szXML += L"ESS";
    szXML += L"</connectionType>\n";

// authentication
    szXML += L"\t<authentication>";
    szXML += L"Open";
    szXML += L"</authentication>\n";

// encryption
    szXML += L"\t<encryption>";
    szXML += L"WEP";
    szXML += L"</encryption>\n";

// networkKey
    szXML += L"\t<networkKey xml:space=\"preserve\">";
    szXML += Escape (L"12345");
    szXML += L"</networkKey>\n";

// keyProvidedAutomatically
    szXML += L"\t<keyProvidedAutomatically>";
    szXML += L"0";
    szXML += L"</keyProvidedAutomatically>\n";

// ieee802.1xEnabled
    szXML += L"\t<IEEE802.1XEnabled>";
    szXML += L"0";
    szXML += L"</IEEE802.1XEnabled>\n";
        
    szXML += "</WirelessProfile>\n";

ULONG ulStatus = 0;
    hr =  spNetProvProfW->CreateProfile (CComBSTR(szXML), NULL, pGuid, &ulStatus);

switch (ulStatus) {
    case WZC_PROFILE_CONFIG_ERROR_WPA_NOT_SUPPORTED:    // from netprov.idl
        _ASSERT (hr == HRESULT_FROM_WIN32 (ERROR_BAD_CONFIGURATION));
        break;
    default:
        break;
    }
    return hr;

}

## Autorun.inf

The Autorun.inf file is created on the root of the portable media. This configuration file is used to tell Windows what application to execute when the media is inserted. On operating system editions earlier than Windows XP SP2, if the media is not DRIVE_FIXED, then Autorun.inf will not work and the user must manually execute the script file.

## Setupsnk.exe

The Setupsnk.exe file is created in the root of the portable media. This application is executed by the Autorun.inf file when the portable media is installed on subsequent PCs. If the operating system is Windows XP SP2, then this application will create a wireless profile to match the schema on the portable media. For any other operating system, this application will offer to print the wireless settings. If the drive does not enumerate as DRIVE_FIXED, then this script will not execute on systems that are not running Windows XP SP2.

## Wsetting.wfc

The Wsetting.wfc file is used to describe the wireless settings of a access point or wireless client device. Various elements in the XML schema are not required for a wireless client but may be used by an access point. The XML schema contains all known configuration options for access points, but all configuration options may not be supported. An example instance of the schema is included to show a typical output from the wizard.

## Wsetting.wfc  Elements

The elements defined in this section are included in the wireless configuration file. The nonbold items are included in the schema but are not supported in the Windows XP version of the wizard.

Important  The order of the elements must be preserved or Windows will invalidate the XML.

configId

A unique GUID generated each time the wizard is executed. The device configuration file configId element must match this GUID for the wizard to validate the device was successfully configured.

configAuthorId

GUID for the author of the instance of the schema.

configAuthor

Name of author of the instance of the schema.

ssid

The wireless beacon.

connectionType {ESS, IBSS}

Ad hoc or infrastructure mode (default = ESS).

authenticateType

{open, shared, WPA, WPAPSK, WPA-NONE, WPA2, WPA2PSK} 
The authentication type for the wireless network (default = Open).

encryption

{none, WEP, TKIP, AES}
The encryption type for the wireless network.

networkKey

The wireless encryption key. 
Note  If WPA authentication is used then the default network key will be a 64 HEX character key.

keyProvidedAutomatically

{true, false}

ieee802Dot1xEnabled

{true, false}

The following elements are included in the schema but are not supported in the Windows XP SP2 release of the wizard:

eapMethod

{EAP-TLS, PEAP-EAP-MSCHAPv2, PEAP-EAP-TLS}

Used for 802.1x authentication only.

channel2Dot4

{0,1,2,3,4,5,6,7,8,9,10,11,12,13,14}

This optional element is used to set the 2.4gHz radio channel.  A value of “0”  indicates “use default”.

channel5Dot0

{0,36, 40, 44, 48, 52, 56, 60, 64, 149, 153, 157, 161}

This optional element is used to set the 5.0gHz radio channel.  A value of “0” indicates “use default”.

deviceMode

{infrastructure, bridge, repeater, station}

timeToLive

Optional element that can be used to set the length of time a network key is valid.

The value is type integer and represents the number of seconds from when the device is configured with the new settings.

configHash

Checksum of the XML file.

keyIndex

{1-4}

### Wsetting.wfc Schema

The following schema is used to generate the Wsetting.wfc. Some elements and values in the schema are not supported in the Windows XP SP2 release of the wizard.

<?xml version="1.0" encoding="utf-8" ?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.microsoft.com/provisioning/WirelessProfile/2004"

xmlns:fc="http://www.microsoft.com/provisioning/WirelessProfile/2004" elementFormDefault="qualified"

version="1">

<xs:element name="wirelessProfile">

<xs:complexType>

<xs:sequence>

<xs:element name="config">

<xs:complexType>

<xs:sequence>

<xs:element name="configId" type="fc:str36" minOccurs="1" maxOccurs="1" />

<xs:element name="configHash" type="fc:str20" minOccurs="0" maxOccurs="1" />

<xs:element name="configAuthorId" type="fc:str36" minOccurs="1" maxOccurs="1" />

<xs:element name="configAuthor" type="fc:strMax128" minOccurs="1" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="ssid" type="fc:strMax32" minOccurs="1" maxOccurs="1" />

<xs:element name="connectionType" minOccurs="1" maxOccurs="1">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="IBSS" />

<xs:enumeration value="ESS" />

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="channel2Dot4" type="xs:positiveInteger" minOccurs="0" maxOccurs="1" />

<xs:element name="channel5Dot0" type="xs:positiveInteger" minOccurs="0" maxOccurs="1" />

<xs:element name="deviceMode" minOccurs="0" maxOccurs="1">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="infrastructure" />

<xs:enumeration value="bridge" />

<xs:enumeration value="repeater" />

<xs:enumeration value="station" />

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="primaryProfile" type="fc:profileInstance" minOccurs="1" maxOccurs="1" />

<xs:element name="optionalProfile" type="fc:profileInstance" minOccurs="0" maxOccurs="32" />

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:complexType name="profileInstance">

<xs:sequence>

<xs:element name="authentication" minOccurs="1" maxOccurs="1">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="open" />

<xs:enumeration value="shared" />

<xs:enumeration value="WPA-NONE" />

<xs:enumeration value="WPA" />

<xs:enumeration value="WPAPSK" />

<xs:enumeration value="WPA2" />

<xs:enumeration value="WPA2PSK" />

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="encryption" minOccurs="1" maxOccurs="1">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="none" />

<xs:enumeration value="WEP" />

<xs:enumeration value="TKIP" />

<xs:enumeration value="AES" />

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="networkKey" type="fc:strMin5Max64" minOccurs="1" maxOccurs="1" />

<xs:element name="keyIndex" type="xs:positiveInteger" minOccurs="0" maxOccurs="1" />

<xs:element name="keyProvidedAutomatically" type="xs:boolean" minOccurs="1" maxOccurs="1" />

<xs:element name="ieee802Dot1xEnabled" type="xs:boolean" minOccurs="1" maxOccurs="1" />

<xs:element name="eapMethod" minOccurs="0" maxOccurs="1">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="EAP-TLS" />

<xs:enumeration value="PEAP-EAP-MSCHAPv2" />

<xs:enumeration value="PEAP-EAP-TLS" />

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="timeToLive" type="xs:positiveInteger" minOccurs="0" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

<!-- Globals simpleTypes-->

<xs:simpleType name="str36">

<xs:restriction base="xs:string">

<xs:length value="36" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="strMax32">

<xs:restriction base="xs:string">

<xs:minLength value="1" />

<xs:maxLength value="32" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="str20">

<xs:restriction base="xs:string">

<xs:length value="20" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="strMin5Max64">

<xs:restriction base="xs:string">

<xs:maxLength value="64" />

<xs:minLength value="5" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="strMax128">

<xs:restriction base="xs:string">

<xs:maxLength value="128" />

<xs:minLength value="1" />

</xs:restriction>

</xs:simpleType>

</xs:schema>

### Wsetting.wfc Sample File (WEP Encryption)

<?xml version="1.0"?>

<wirelessProfile xmlns="http://www.microsoft.com/provisioning/WirelessProfile/2004">

<config>

<configId>DD102F7E-79AD-4272-AE92-A8559B1ACE43</configId>

<configAuthorId>0B4E59B8-8317-46B2-B8C2-C46850162E6C</configAuthorId>

<configAuthor>Microsoft Wireless Network Setup Wizard</configAuthor>

</config>

<ssid xml:space="preserve">FCTest</ssid>

<connectionType>ESS</connectionType>

<primaryProfile>

<authentication>open</authentication>

<encryption>WEP</encryption>

<networkKey xml:space="preserve">8e93898f4bdd42ce974b79b521</networkKey>

<keyProvidedAutomatically>0</keyProvidedAutomatically>

<ieee802Dot1xEnabled>0</ieee802Dot1xEnabled>

</primaryProfile>

</wirelessProfile>

### Wsetting.wfc Sample File (WPA Encryption)

<?xml version="1.0"?>

<wirelessProfile xmlns="http://www.microsoft.com/provisioning/WirelessProfile/2004">

<config>

<configId>000A013E-6740-439C-9935-EE58D85FA38A</configId>

<configAuthorId>0B4E59B8-8317-46B2-B8C2-C46850162E6C</configAuthorId>

<configAuthor>Microsoft Wireless Network Setup Wizard</configAuthor>

</config>

<ssid xml:space="preserve">FCTest324-2</ssid>

<connectionType>ESS</connectionType>

<primaryProfile>

<authentication>WPAPSK</authentication>

<encryption>TKIP</encryption>

<networkKey xml:space="preserve">adfafdgrhknakj fdankjnahiu rhyqa thnblajrtl985762987)</networkKey>

<keyProvidedAutomatically>0</keyProvidedAutomatically>

<ieee802Dot1xEnabled>0</ieee802Dot1xEnabled>

</primaryProfile>

</wirelessProfile>

## Device Configuration File (Xxxxxxxx.wfc)

Each device that is configured using WCN writes a configuration file back to the UFD. The file name is based on the last 8 nibbles of the MAC address of the wireless network adapter being configured, represented in ASCII-hex.  The file is copied to the \SMRTNTKY\DEVICE\ folder on the UFD.

For example, if the MAC address of the network adapter is 00-08-0D-1A-DE-67, then the device configuration filename would be  0D1ADE67.WFC.

### Device Configuration File Elements

The schema shown below is a superset of the elements supported by the wizard. The following elements are required by the wizard for a valid implementation.

configId

This GUID must match the value for the configId element in the Wsetting.wfc file used to configure the device.

manufacturer

The manufacturer of the device.

modelName

The model name of the device.

serialNumber

The serial number of the device.

firmwareVersion

The version of the firmware of the device.

configError

Included only if the device experienced and error when it was configured.

The value of this element can be 1 – 256 ASCII characters. This text will be displayed on the finish page of the wizard.

deviceType

Not required, but strongly recommended.

The wizard uses the deviceType element to determine what category of device has been configured.

The remaining elements in the schema are optional and can be useful for diagnostic purposes. Future versions of the wizard may incorporate these elements.

Important  The order of the elements must be preserved or Windows will invalidate the XML.

### Device Configuration File Schema

<?xml version="1.0" encoding="utf-8" ?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.microsoft.com/provisioning/DeviceProfile/2004"

xmlns:fc="http://www.microsoft.com/provisioning/DeviceProfile/2004" elementFormDefault="qualified"

version="1">

<xs:element name="device">

<xs:complexType>

<xs:sequence>

<xs:element name="configId"		type="fc:str36" minOccurs="1" maxOccurs="1" />

<xs:element name="configHash"	type="fc:str20" minOccurs="0" maxOccurs="1" />

<xs:element name="configError"	type="fc:strMax256" minOccurs="0" maxOccurs="1" />

<xs:element name="manufacturer" type="fc:strMax256" minOccurs="1" maxOccurs="1" />

<xs:element name="manufacturerURL" type="fc:anyURIMax2084" minOccurs="0" maxOccurs="1" />

<xs:element name="deviceStatus" type="fc:strMax256" minOccurs="0" maxOccurs="1" />

<xs:element name="modelName"	type="fc:strMax256" minOccurs="1" maxOccurs="1" />

<xs:element name="modelURL"		type="fc:anyURIMax2084" minOccurs="0" maxOccurs="1" />

<xs:element name="serialNumber" type="fc:strMax256" minOccurs="1" maxOccurs="1" />

<xs:element name="firmwareVersion" type="fc:strMax256" minOccurs="0" maxOccurs="1" />

<xs:element name="presentationURL" type="fc:anyURIMax2084" minOccurs="0" maxOccurs="1" />

<xs:element name="deviceType" fc:type="deviceTypes" minOccurs="0" maxOccurs="1" />

<!-- Configuration information if the device is a member of a workgroup -->

<xs:element name="workgroup" minOccurs="0" maxOccurs="1">

<xs:complexType>

<xs:sequence>

<xs:element name="deviceName"		type="fc:strName" minOccurs="0" maxOccurs="1" />

<xs:element name="workgroupName"	type="fc:strName" minOccurs="0" maxOccurs="1" />

<xs:element name="deviceDescription" type="fc:strDescription" minOccurs="0" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

</xs:element>

<!-- What network connections does this device support -->

<xs:element name="networkInterface" minOccurs="0" maxOccurs="32">

<xs:complexType>

<xs:sequence>

<xs:element name="IPv4Address" type="fc:dotQuadIPv4Type" minOccurs="0" maxOccurs="1" />

<xs:element name="IPv6Address" type="fc:textIPv6AddressType" minOccurs="0" maxOccurs="32" />

<xs:element name="macAddress" type="fc:macAddress" minOccurs="1" maxOccurs="1" />

<xs:element name="interfaceStatus" type="fc:interfaceStatuses" maxOccurs="1" minOccurs="1" />

<xs:element name="mediaType" type="fc:mediaTypes" minOccurs="1" maxOccurs="1" />

<xs:element name="speed" type="xs:positiveInteger" minOccurs="1" maxOccurs="1" />

<xs:element name="DHCP" type="xs:boolean" minOccurs="1" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

</xs:element>

<!-- General WiFi Configuration information -->

<xs:element name="wifiAuthentication" minOccurs="0" maxOccurs="1">

<xs:complexType>

<xs:sequence>

<xs:element name="open" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="shared" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="WPA-NONE" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="WPA" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="WPAPSK" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="WPA2" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="WPA2PSK" type="xs:boolean" minOccurs="0" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="wifiEncryption" minOccurs="0" maxOccurs="1">

<xs:complexType>

<xs:sequence>

<xs:element name="none" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="WEP" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="TKIP" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="AES" type="xs:boolean" minOccurs="0" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

</xs:element>

<!-- Configuration information if the device is a WiFi client -->

<xs:element name="wifiClient" minOccurs="0" maxOccurs="1">

<xs:complexType>

<xs:sequence>

<xs:element name="regulatoryDomain" type="fc:str3" minOccurs="0" maxOccurs="1" />

<xs:element name="transmitPower" type="fc:intPowerStrength" minOccurs="0" maxOccurs="1" />

<xs:element name="clientNetworkType">

<xs:complexType>

<xs:sequence>

<xs:element name="ieee802Dot11a" type="xs:boolean" />

<xs:element name="ieee802Dot11b" type="xs:boolean" />

<xs:element name="ieee802Dot11g" type="xs:boolean" />

</xs:sequence>

</xs:complexType>

</xs:element>

</xs:sequence>

</xs:complexType>

</xs:element>

<!-- Configuration information of wireless networks visible to device -->

<xs:element name="wifiInfrastructure" minOccurs="0" maxOccurs="100">

<xs:complexType>

<xs:sequence>

<xs:element name="ssid" type="fc:strMax32" minOccurs="1" maxOccurs="1" />

<xs:element name="macAddress" type="fc:macAddress" minOccurs="1" maxOccurs="1" />

<xs:element name="visibleNetworkType" type="fc:visibleNetworkTypes" minOccurs="1" maxOccurs="1" />

<xs:element name="channel" type="xs:positiveInteger" minOccurs="1" maxOccurs="1" />

<xs:element name="signalStrength" type="fc:intPowerStrength" minOccurs="0" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

</xs:element>

<!-- Configuration information if the device is a WiFi access point -->

<xs:element name="wifiAccessPoint" minOccurs="0" maxOccurs="1">

<xs:complexType>

<xs:sequence>

<xs:element name="ieee802Dot11a" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="ieee802Dot11b" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="ieee802Dot11g" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="regulatoryDomain" type="fc:str3" minOccurs="0" maxOccurs="1" />

<xs:element name="macFilter" type="fc:macAddress" minOccurs="0" maxOccurs="252" />

<xs:element name="securityType" minOccurs="0" maxOccurs="1">

<xs:complexType>

<xs:sequence>

<xs:element name="basic" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="WPA" type="xs:boolean" minOccurs="0" maxOccurs="1" />

<xs:element name="WPA2" type="xs:boolean" minOccurs="0" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="beaconInterval" type="fc:intBeaconInterval" minOccurs="0" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

</xs:element>

</xs:sequence>

</xs:complexType>

</xs:element>

<!-- Globals simpleTypes-->

<xs:simpleType name="dotQuadIPv4Type">

<xs:restriction base="xs:string">

<xs:pattern value="([\d]{1,3}.){3}[\d]{1,3}" />

<xs:minLength value="7" />

<xs:maxLength value="15" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="textIPv6AddressType">

<xs:restriction base="xs:string">

<xs:pattern value="(([\dA-Fa-f]{1,4}:){7}[\dA-Fa-f]{1,4})(:([\d]{1,3}.){3}[\d]{1,3})?" />

<xs:minLength value="16" />

<xs:maxLength value="39" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="macAddress">

<xs:restriction base="xs:hexBinary">

<xs:length value="6" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="anyURIMax2084">

<xs:restriction base="xs:string">

<xs:minLength value="1" />

<xs:maxLength value="2084" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="strMax256">

<xs:restriction base="xs:string">

<xs:maxLength value="256" />

<xs:minLength value="1" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="str36">

<xs:restriction base="xs:string">

<xs:length value="36" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="strMax32">

<xs:restriction base="xs:string">

<xs:maxLength value="32" />

<xs:minLength value="1" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="str20">

<xs:restriction base="xs:string">

<xs:length value="20" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="str3">

<xs:restriction base="xs:string">

<xs:length value="3" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="strName">

<xs:restriction base="xs:string">

<xs:minLength value="1" />

<xs:maxLength value="15" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="strDescription">

<xs:restriction base="xs:string">

<xs:minLength value="1" />

<xs:maxLength value="50" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="intBeaconInterval">

<xs:restriction base="xs:positiveInteger">

<xs:maxInclusive value="3600" />

<xs:minInclusive value="1" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="intPowerStrength">

<xs:restriction base="xs:integer">

<xs:minInclusive value="-200" />

<xs:maxInclusive value="-20" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="deviceTypes">

<xs:restriction base="xs:string">

<xs:enumeration value="Computer" />

<xs:enumeration value="Access_Point" />

<xs:enumeration value="Printer" />

<xs:enumeration value="Electronic_Picture_Frame" />

<xs:enumeration value="Digital_Audio_Receiver" />

<xs:enumeration value="Windows_Media_Center_Extender" />

<xs:enumeration value="Personal_Video_Recorder" />

<xs:enumeration value="Printer_Bridge" />

<xs:enumeration value="Projector" />

<xs:enumeration value="Pocket_PC" />

<xs:enumeration value="Other" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="interfaceStatuses">

<xs:restriction base="xs:string">

<xs:enumeration value="active" />

<xs:enumeration value="disabled" />

<xs:enumeration value="noMedia" />

<xs:enumeration value="authenticating" />

<xs:enumeration value="media" />

<xs:enumeration value="noSignal" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="mediaTypes">

<xs:restriction base="xs:string">

<xs:enumeration value="802.3" />

<xs:enumeration value="1394" />

<xs:enumeration value="802.11" />

<xs:enumeration value="usb" />

<xs:enumeration value="bluetooth" />

<xs:enumeration value="other" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="visibleNetworkTypes">

<xs:restriction base="xs:string">

<xs:enumeration value="ieee802Dot11a" />

<xs:enumeration value="ieee802Dot11b" />

<xs:enumeration value="ieee802Dot11g" />

</xs:restriction>

</xs:simpleType>

</xs:schema>

### Sample Device Configuration File

<?xml version="1.0"?>

<device xmlns="http://www.microsoft.com/provisioning/DeviceProfile/2004">

<configId>DD102F7E-79AD-4272-AE92-A8559B1ACE43</configId>

<manufacturer>Microsoft</manufacturer>

<modelName>Windows XP</modelName>

<serialNumber>MyLaptop</serialNumber>

<firmwareVersion>SP2</firmwareVersion>

<deviceType>Computer</deviceType>

</device>

When the wizard is complete, it displays all of the devices that have been configured. The wizard seeks device configuration files in the /SMRTNTNY/DEVICE folder which have a configId GUID value that match the value in the /SMRTNTKY/Wsetting.wfc file. The summary of devices are displayed by the wizard.

The elements from the device file are displayed in the following order when the device has been successfully configured:

"<manufacturer> " " <modelName> " - " <serialNumber> " (" <deviceType> ")"

The elements from the device file are displayed in the following order when the device has not been successfully configured:

"ERROR - " <manufacturer> " - " <modelName> " - " <serialNumber> " (" <deviceTypes> ", " <configError> ")"

## Dual Radio APs

The WCN 1.0 wizard in Windows XP SP2 does not support unique configuration of dual radio access points or wireless clients. To work around this limitation, the following logic is proposed:

1.	If a Wsetting.wfc file does not contain both the channel2Dot4 and channel5Dot0 elements, then the access point should configure both radios identically—that is, same SSID and Network Key).

2.	If a Wsetting.wfc file contains the channel2Dot4 and channel5Dot0 elements then the access point should configure the 5.0-GHz radio to the SSID value specified in the file.

The 2.4-GHz radio should be configured to the same SSID with a “-2” appended.  If the original SSID is greater than 30 characters, then it should be truncated to 30 characters and a “-2” added to the end.  All the other configuration elements should remain the same.

This following example Wsetting.wfc file indicates support for unique SSID names for dual radio APs.

<?xml version="1.0"?>

<wirelessProfile xmlns="http://www.microsoft.com/provisioning/WirelessProfile/2004">

<config>

<configId>000A013E-6740-439C-9935-EE58D85FA38A</configId>

<configAuthorId>0B4E59B8-8317-46B2-B8C2-C46850162E6C</configAuthorId>

<configAuthor>Microsoft Wireless Network Setup Wizard</configAuthor>

</config>

<ssid xml:space="preserve">My Wireless Dual Radio</ssid>

<connectionType>ESS</connectionType>

<channel2Dot4>0</channel2Dot4>

<channel5Dot0>0</channel5Dot0>

<primaryProfile>

<authentication>WPAPSK</authentication>

<encryption>TKIP</encryption>

<networkKey xml:space="preserve">adfafdgrhknakj fdankjnahiu rhyqa thnblajrtl985762987)</networkKey>

<keyProvidedAutomatically>0</keyProvidedAutomatically>

<ieee802Dot1xEnabled>0</ieee802Dot1xEnabled>

</primaryProfile>

</wirelessProfile>

In this example, the AP would configure the 5.0-GHz radio to the following settings:

- SSID: “My Wireless Dual Radio”

- Network Key: “adfafdgrhknakj fdankjnahiu rhyqa thnblajrtl985762987)”

The 2.4-GHz radio in this example would be configured to the following settings:

- SSID: “My Wireless Dual Radio-2”

- Network Key: “adfafdgrhknakj fdankjnahiu rhyqa thnblajrtl985762987)”

The following example Wsetting.wfc file does not support unique SSID names. Note that the channel2Dot4 and channel5Dot0 elements are not present.

<?xml version="1.0"?>

<wirelessProfile xmlns="http://www.microsoft.com/provisioning/WirelessProfile/2004">

<config>

<configId>000A013E-6740-439C-9935-EE58D85FA38A</configId>

<configAuthorId>0B4E59B8-8317-46B2-B8C2-C46850162E6C</configAuthorId>

<configAuthor>Microsoft Wireless Network Setup Wizard</configAuthor>

</config>

<ssid xml:space="preserve">My Wireless Dual Radio</ssid>

<connectionType>ESS</connectionType>

<primaryProfile>

<authentication>WPAPSK</authentication>

<encryption>TKIP</encryption>

<networkKey xml:space="preserve">adfafdgrhknakj fdankjnahiu rhyqa thnblajrtl985762987)</networkKey>

<keyProvidedAutomatically>0</keyProvidedAutomatically>

<ieee802Dot1xEnabled>0</ieee802Dot1xEnabled>

</primaryProfile>

</wirelessProfile>

In this example, the AP would configure the 2.4-GHz and the 5.0-GHz radio to the same settings. In the example, both radios would be configured as follows:

- SSID: “My Wireless Dual Radio”

- Network Key: “adfafdgrhknakj fdankjnahiu rhyqa thnblajrtl985762987)”

If the access point supports dual radios, then it should write two device configuration files, one representing each radio.  If the access point only has one MAC address for the two radios, then the second device configuration filename should be based on the MAC address but incremented in value to make it unique. For example:

- If the MAC address of the network adapter is 00-08-0D-1A-DE-67, then the device configuration filename would be 0D1ADE67.WFC.

- The filename for the second radio would be OD1ADE68.WFC.

## WCN Configuration Support for Devices without USB Host Ports

It is possible to support WCN for provisioning of wireless clients when the wireless networking devices (access points) do not support USB host ports.  Configuration of the PC and access point must be done using a third-party proprietary configuration mechanism that creates the LastFlashConfig.wfc file on the local PC.  When the WCN wizard runs, it seeks this file.  If this file is found, the wizard recommends adding additional devices to the existing wireless network, as opposed to creating a new wireless network.

This specification contains instructions for reading and writing the LastFlashConfig.wfc file. It is recommended that the setup software for access points write this file to allow the user to add additional PCs and devices to the secure wireless network using WCN.

# Hardware Requirements

WCN-compliant devices and software must support the features listed in this section.

## Requirements for USB Flash Drives

WCN-compliant devices should work with a variety of UFDs to ensure compatibility. There have been many variations of UFDs with a variety of different characteristics—drive type, Master Boot Record, capacity, and so on.  The following shows the recommended configuration.

Required Configuration of UFDs for Compatibility with WCN Devices

- USB Interfaced Class 0x8

- USB Mass Storage Subclass 0x6

- USB Interface Protocol 0x50

- USB Mass Storage INQUIRY RMB bit value of 0x1

- Raw capacity between 200K and 10 GB

- FAT 12, FAT 16, FAT 32

- Master Boot Record present

- Maximum current sink 150 mA

- Single partition

## Requirements for WCN Devices

WCN devices must support specific hardware requirements to maintain compliance with the WCN licensing agreement and to ensure interoperability with WCN implementations in Windows and UFD devices.

Requirements for WCN-compliant Devices

- Easily accessible USB host port (USB version 1.1 or later)

- Support WPA-PSK 64-byte hex Network Keys

- Support standard UFDs:

- USB Mass Storage Subclass 0x6

- USB Interface Protocol 0x50

- UFDs with USB Mass Storage INQUIRY RMB bit value of 0x1 or 0x0

- UFDs with raw capacity between 200K and 10 GB

- UFDs with FAT12, FAT16, FAT32 formats

- UFDs with or without Master Boot Record present

- Source up to 200 mA on USB host port

- Support UFDs with multiple partitions.  
The WCN device should scan each partition of the UFD looking for the WCN configuration files.

Performance Requirements for WCN-compliant Devices

When a WCN device has been successfully configured, it should confirm a successful configuration by flashing the appropriate LEDs three times with a 1-Hz cycle as defined here:

0.5 sec ON, 0.5 sec OFF, 0.5 sec ON, 0.5 sec OFF, 0.5 sec ON, then OFF

XXXXX_____XXXXX_____XXXXX

When a WCN device is unsuccessful at reading or writing the WCN configuration file, it should respond with a sequence of flashes: two short flashes (0.3 second ON duration) and one long flash (1 second ON duration) with a 0.3 second delay between all flashes—and then repeat until the SmartKey is removed or for at least three full cycles. For example:

0.3 sec ON, 0.3 sec OFF, 0.3 sec ON, 0.3 sec OFF, 1 sec ON, 0.3 sec OFF

XXX___XXX___XXXXXXXXXXXX___XXX___XXX___XXXXXXXXXXXX___XXX___XXX___XXXXXXXXXXXX___XXX___XXX___XXXXXXXXXXXX....

# Firmware and Software Requirements

WCN-compliant devices must understand and parse the XML configuration files stored on the UFD.  Additionally, the device must write an XML configuration file back to the UFD, confirming a successful configuration or providing an explanation for a failure.  The Windows Wireless Network Setup Wizard does not confirm any device which writes a configuration file that does not conform to the device configuration schema.  If a Wsetting.wfc file does not conform to the schema, then the Wizard ignores the file and does not successfully create a wireless profile.

# UFD Configuration with Earlier Versions of Windows

The “Network Setup” wizard and the “Add a device to the network” wizard use Windows Connect Now to configure the network. Configuration using USB flash drives is available  using the Wireless Network Setup Wizard will only run on in Windows XP SP2 and later. The wizard copies a configuration application to the UFD. This wizard runs automatically if the UFD device enumerates as DRIVE_FIXED or if the machine is running Windows XP SP2.

The wizard creates a wireless profile on Windows XP SP2-based machines and provides the user an option to print the settings on down-level machines—that is, machines running Windows 98 through Windows XP SP1. Table 3 describes the behavior on different versions of Windows using UFDs that enumerate as DRIVE_FIXED and DRIVE_REMOVEABLE.

Table 3. UFD Behavior on Earlier Versions of Windows

| Windows version | Drive type | Result |
| --- | --- | --- |
| Windows XP SP2 | DRIVE_FIXED | Autoplay lists Wireless Setup Wizard as top option. |
| Windows XP (before SP2) | DRIVE_FIXED | Autorun runs Setupsnk.exe automatically. |
| Windows 2000 | DRIVE_FIXED | Autorun runs Setupsnk.exe automatically. |
| Windows Me | DRIVE_FIXED | Autorun runs Setupsnk.exe automatically. |
| Windows 98 | DRIVE_FIXED | Autorun runs Setupsnk.exe automatically. |
| Windows XP SP2 | DRIVE_REMOVABLE | Autoplay lists Wireless Setup Wizard as top option. |
| Windows XP (before SP2) | DRIVE_REMOVABLE | Autorun does not run. |
| Windows 2000 | DRIVE_REMOVABLE | Autorun does not run. |
| Windows Me | DRIVE_REMOVABLE | Autorun does not run. |
| Windows 98 | DRIVE_REMOVABLE | Autorun does not run. |

The wizard offers the user an opportunity to print the network settings for the wireless profile that was created by the wizard. The following information is displayed in Notepad.

Wireless Network Settings

Print this document and store it in a safe place for future reference.  You may need these settings to add additional computers and devices to your network.

Wireless Settings

Network Name (SSID):

Network Key (WEP Key):

Key Provided Automatically (802.1x):

Network Authentication Type:

Data Encryption Type:

Connection Type:

Key Index:

To enable File and Print sharing on this computer, run the Network Setup Wizard

To setup your Internet connection, follow the instructions from your Internet Service Provider (ISP)

# References

E-mail contact:

Rally @ microsoft.com

Windows Rally specifications and white papers

http://www.microsoft.com/rally