Emergency Management Services

Emergency Management Services is a new feature in Microsoft® Windows® Server 2003 that supports remote management and system recovery for servers that are not accessible through a conventional connection. By using Emergency Management Services in conjunction with other software and hardware components, system administrators can remotely perform tasks such as recovering unresponsive servers or powering servers off and on, even when they cannot access the servers through the standard network connection. The alternate connection provided by Emergency Management Services also supports headless servers — those without a locally attached keyboard, video, or mouse. Headless servers can result in significant hardware savings.

#### In This Chapter

Emergency Management Services Overview	2

Understanding Emergency Management Services Features	3

Understanding Out-of-Band Components	9

Configuring Headless Systems	16

Configuring Firmware Settings for Emergency Management Services	16

Enabling Emergency Management Services	22

Using Emergency Management Services	35

Troubleshooting Emergency Management Services	47

Additional Resources	53

#### Related Information in the Resource Kits

- For more information about remote management and remote management tools, see “Remote Administration Overview” and “Remote Management Tools” in the Server Management Guide of the Microsoft® Windows® Server 2003 Resource Kit (or see Remote Administration Overview” and “Remote Management Tools” on the Web at http://www.microsoft.com/reskit).

- For more information about deploying Emergency Management Services, see “Planning for Remote Server Management” in Planning Server Deployments of the Microsoft® Windows® Server 2003 Deployment Kit (or see “Planning for Remote Server Management” on the Web at http://www.microsoft.com/reskit).

# Emergency Management Services Overview

Previously, servers running the Windows operating system could be managed remotely only by using a conventional in-band connection — one that uses the computer’s standard communication channel, typically the network. If a server stopped responding, it had to be managed locally. With Microsoft Windows Server 2003, you can use Emergency Management Services to manage servers remotely even when they are not responding over the network. Emergency Management Services is included with Microsoft® Windows® Server 2003, Standard Edition, Windows® Server 2003, Enterprise Edition, Windows® Server 2003, Datacenter Edition, and Windows® Server 2003, Web Edition.

Emergency Management Services supports out-of-band connections — those using an alternate communication channel, most commonly a serial port. Unlike in-band connections, out-of-band connections do not rely on operating system network drivers to function. Used in conjunction with the appropriate hardware components, Emergency Management Services makes it possible for you to remotely perform almost all management tasks, except for hardware replacement, from when the system starts, through normal operation, and during system stress. This capability is particularly important for servers configured to be headless and for those that have high availability requirements.

When servers can be accessed over the standard network, in-band management is the preferred method. The purpose of Emergency Management Services is to help you return a server to a state where in-band management is possible. Unexpected events, however, can interrupt in-band connectivity. For example, a network adapter problem or system instability can cause a server to become unresponsive to management attempts from client computers that use an in-band connection. Because Emergency Management Services removes dependence on operating system network drivers, client computers can communicate with servers through the out-of-band port in situations such as these, so you can attempt to bring the servers back into service remotely.

Note

It is recommended that before reading this chapter you read “Remote Administration Overview” in the Server Management Guide of the Windows Server 2003 Resource Kit (or see “Remote Administration Overview” on the Web at http://www.microsoft.com/reskit) for background information about in-band and out-of-band management.

With Emergency Management Services, a locally attached keyboard, mouse, and monitor are not required, making it possible to create a headless server configuration. If the system firmware supports it, the keyboard controller and video adapter are also not required. Because these components are optional, headless configurations can present significant savings in space, energy consumption, and hardware costs.

Emergency Management Services is designed to be a last resort remote management option; you should first try in-band tools. For example, do not use Emergency Management Services to monitor your system if other means such as Remote Desktop Connection are still available.

# Understanding Emergency Management Services Features

Emergency Management Services redirects text output to the out-of-band port and provides a command-line environment for managing a server through the out-of-band port. Redirecting text output is known as console redirection. Emergency Management Services provides the following three features:

- Console Redirection

- Special Administration Console (SAC) environment

- !Special Administration Console (!SAC) environment

Emergency Management Services also supports headless operation for computers that have the appropriate out-of-band components.

## Console Redirection

Console redirection means sending and receiving character-mode output or input by using the out-of-band port. For example, a server that supports console redirection is able to receive keyboard input from a managing client computer and send character-mode output back to the client computer by using the out-of-band port. Enabling console redirection does not prevent you from using local input and display devices. Therefore, it is possible for a computer running Windows Server 2003 to process in-band, out-of-band, and local console input and output (I/O) simultaneously.

#### Console Redirection Display Mode

Console redirection uses a character-mode display instead of a graphical user interface (GUI) for the following reasons:

- Hardware compatibility. A character mode display provides compatibility with different types of communication media, from relatively slow serial port connections to higher speed technologies. The serial port is the most commonly used out-of-band device.

- Software compatibility. A character mode display simplifies interoperability issues with different types of terminals and terminal emulation software, including those running on other operating systems such as UNIX.

#### Types of Console Redirection

The Windows Server 2003 operating system, hardware integrated into the motherboard, or add-in devices can provide the following types of console redirection:

- Firmware console redirection. System firmware that supports console redirection provides for remote management before the Windows Server 2003 operating system starts. Types of firmware include BIOS for x86-based computers and extensible firmware interface (EFI) for Itanium-based systems. For more information about EFI, see the Extensible Firmware Interface link on the Web Resources page at http://www.microsoft.com/windows/reskits/webresources.

- Emergency Management Services console redirection. Windows Server 2003 components, such as the Windows loader (Ntldr), support console redirection.

- Service processor console redirection. If your server includes a specialized hardware component called a service processor, the service processor can also provide console redirection. For more information about service processors, see “Service Processors” later in this chapter.

### Firmware Console Redirection

Your computer’s firmware might support console redirection. Typically, firmware for Itanium-based computers supports console redirection; for x86-based computers, contact your computer manufacturer for information about firmware console redirection functionality. If your x86-based computer does not support firmware console redirection, you might be able to obtain a firmware upgrade.

You can use firmware console redirection to remotely manage a server from restart until the initial load of the Windows Server 2003 operating system. Without firmware console redirection, you cannot manage your computer remotely until Windows Server 2003 starts.

With firmware console redirection, you can perform the following remote management tasks before Windows Server 2003 starts.

Note

A service processor can provide console redirection before the Windows Server 2003 operating system starts, even without firmware console redirection. For more information, see “Service” later in this chapter.

##### Remotely view startup progress

You can remotely view computer status before Windows Server 2003 starts, such as Power-on Self-Test (POST) information, disk-related error messages, and other text displayed by the computer’s firmware. A computer that has firmware console redirection typically allows the POST to complete successfully without an attached keyboard, mouse, or display device.

##### Remotely view and edit firmware settings

You can remotely access the configuration program provided by the computer firmware to change hardware settings. For example, you can remotely disable integrated peripherals or change the boot device order. Without firmware console redirection, you must make these types of changes locally.

##### Remotely view and respond to Pre-Boot eXecution Environment prompts

If a server’s firmware supports the Pre-Boot eXecution Environment (PXE) standard, you can use firmware console redirection to remotely view and respond to the F12 network boot prompt. For information about PXE, see “Remote Installation Services” and “Enabling Emergency Management Services with New Installations” later in this chapter.

##### Remotely view and respond to the boot from CD prompt

With firmware console redirection, you can remotely respond to the Press Any Key to Boot from CD prompt when starting your server by using the Windows Server operating system CD.

### Emergency Management Services Console Redirection

If Emergency Management Services is enabled when Windows Server 2003 starts, the operating system assumes the responsibility for console redirection from the firmware. The ability to redirect console I/O to the out-of-band port is built into the Windows Server 2003 components listed in Table 10.1.

Table 10.1   Components That Support Emergency Management Services Console Redirection

| Windows Server 2003 Component or Feature | Description |
| --- | --- |
| Windows loader for x86-based computers (Ntldr) | The Windows Server 2003 operating system loader. When Ntldr is running, you can remotely view and select the Recovery Console or, on x86-based multiple-boot computers, which operating system to start. |
| Windows kernel (Ntoskrnl.exe) | The core (also called the kernel) of the Windows Server 2003 operating system. Code that runs as part of the kernel does so in privileged processor mode and has direct access to system data and hardware. The Windows kernel supports console redirection, which enables you to remotely view system information during normal operation, or remotely view Stop message text when a system problem occurs. |
| Recovery Console | Recovery Console is a command-line environment that allows you to perform advanced troubleshooting and maintenance tasks, such as disabling a driver that you suspect is causing a startup problem. |
| Command Prompt (Cmd.exe) | A character mode command shell user interface that provides an environment for running commands and applications. |
| Text-mode Setup (including the CD-ROM Setup loader) | The initial portion of the Windows Server 2003 installation process when Setup displays character mode prompts and status information text. During text-mode setup, files are copied from the distribution folder to the local hard disk. |
| Startrom.com at 9600 baud for x86-based computers | Starts the x86-based Remote Installation Services (RIS) process. This file is downloaded and run by the RIS client to initiate the operating system installation procedure. Only special versions of Startrom.com that use 9600 baud support Emergency Management Services console redirection. For more information about Startrom.com, see “Remote Installation Services” later in this chapter. |

Emergency Management Services does not provide console redirection for the components listed in Table 10.2.

Table 10.2   Components That Do Not Support Emergency Management Services Console Redirection

| Windows Server 2003 Component | Description |
| --- | --- |
| The Press Any Key to Boot from CD prompt | This prompt appears when you start your system by using the Windows Server 2003 operating system CD. |
| GUI-mode Setup | Emergency Management Services processes only character-mode input and output. Emergency Management Services cannot display GUI-mode graphics, such as windows and dialog boxes, or redirect mouse input. You can, however, perform Special Administration Console (SAC) commands and monitor setup logs during GUI-mode setup. |
| Startrom.com at baud rates greater than 9600 | Startrom.com supports Windows Emergency Management Services console redirection only at 9600 baud. |
| Operating system boot menu for Itanium-based computers | For Itanium computers, the EFI boot manager (IA64ldr), not the Windows loader, displays the list of operating systems installed on the system. To remotely view and select boot manager menu items on Itanium-based systems, you must enable firmware console redirection. |

#### Remote Tasks Supported during Normal Operation

With Emergency Management Services console redirection, you can perform the following remote management tasks when the operating system is functioning normally.

##### Verify that Windows has started

When the Windows loader starts, it displays the following message by using the out-of-band port:

Windows is now starting

The preceding message indicates that the Windows Server 2003 startup process has begun.

When the Windows kernel initializes, it displays the following prompt by using the out-of-band port, indicating that Emergency Management Services is enabled:

SAC>

##### Choose an item on the operating system boot menu (for x86-based computers)

For multiple-boot systems that have Windows Server 2003 installed along with Recovery Console or another Microsoft operating system, Ntldr automatically displays an operating boot menu, both to the local display and to the out-of-band port. By using this operating system boot menu, you can start the Windows Server 2003 operating system or Recovery Console. You also have the option of viewing the Windows Advanced Options Menu by pressing the F8 key. The following is a typical operating system boot menu.

Please select the operating system to start:

Microsoft Windows Server 2003

Recovery Console

Use the up and down arrow keys to move the highlight to your choice.

Press Enter to choose.

Seconds until highlighted choice will be started automatically:  29

For troubleshooting and advanced startup options for Windows, press F8.

##### Monitor the progress of Windows Server 2003 text-mode setup

The text-mode setup phase displays character-based information, which supports Windows Emergency Management Services console redirection. Therefore, you can remotely view and respond to text-mode setup prompts.

##### View and respond to PXE environment prompts

If the server’s firmware supports the PXE standard but does not support firmware console redirection, an alternate version of the Startrom.com file provides Emergency Management Services console redirection. For information about PXE and Startrom.com, see “Remote Installation Services and “Enabling Emergency Management Services with New Installations” later in this chapter.

#### Remote Tasks Supported for Problem Resolution

With Emergency Management Services console redirection, you can perform the following remote management tasks to troubleshoot problems.

##### Use Last Known Good Configuration to troubleshoot problems

Last Known Good Configuration is a troubleshooting option you can use when a driver or application installation problem prevents a computer from starting in normal mode. By using Last Known Good Configuration, you can recover from a problem by reversing the driver and registry changes made since you last started Windows Server 2003. You can invoke Last Known Good Configuration from the Windows Advanced Options Menu. You can display the Windows Advanced Options Menu by pressing F8 during system startup or at the operating system boot menu, if available.

##### Use Recovery Console to troubleshoot problems

Recovery Console is a command-line environment that you can use to remotely perform advanced troubleshooting operations, such as manually replacing system files. You can invoke Recovery Console by installing it on your computer and then selecting it from the operating system boot menu or by starting it from the Windows Server 2003 installation CD.

### Service Processor Console Redirection

If your server is equipped with a service processor, you might have enhanced out-of-band management features and console redirection. Service processor features can vary, so consult your system documentation for more information about management features. For more information about service processors, see “Service Processors” later in this chapter.

## Special Administration Console

Special Administration Console (SAC) is the primary Emergency Management Services command-line environment. SAC is a kernel-mode component that provides out-of-band management functionality when Windows runs in GUI mode.

SAC is available early in the startup process, and you can use it to manage the server during normal system operation, safe mode, and the GUI phase of Windows Server 2003 setup. When Emergency Management Services is enabled, SAC is always active as long as the kernel is running. SAC is a separate environment from the Windows Server 2003 command prompt (Cmd.exe). You can remotely monitor the status of a server or troubleshoot problems by using terminal software to access SAC.

#### Remote Tasks Supported by SAC during Normal Operation

You can use SAC to perform the following remote tasks when the operating system is functioning normally:

- Gather server information, such as computer name and IP address.

- Obtain a list of the services and applications running on the computer.

- Restart or shut down a server as part of planned maintenance.

- Change the system time and date.

#### Remote Tasks Supported by SAC for Problem Resolution

You can use SAC to perform the following troubleshooting tasks:

- Raise or lower the priority of a process, or end a process that is consuming excessive processor time or other system resources. By establishing an out-of-band connection to SAC, you can continue to manage a server even when low system resources have caused it to become unresponsive to in-band management tools.

- Restart or shut down a server that has stopped responding to in-band commands and tools.

- View or change a server’s TCP/IP networking information to resolve issues, such as problems caused by a duplicate IP address.

- Maintain communication with a server during network outages. For example, you might want to verify that a connectivity issue is caused by a problem that affects the primary network rather than by a problem with the server hardware or a driver.

- Generate a list of drivers for diagnostic purposes.

- View setup logs to determine the cause of a problem that prevents Windows Server 2003 installation from completing.

!Special Administration Console

Important

SAC usage is not safeguarded by password and logon requirements. You must secure physical access to computers running Emergency Management Services by placing the appropriate restrictions on all connecting terminals. For more information about securing Emergency Management Services, see “Out-of-Band Security Considerations” later in this chapter.

!Special Administration Console (!SAC) is an abbreviated version of SAC that accepts input and sends output through the out-of-band port. !SAC is a separate entity from SAC and the Windows Server 2003 command prompt. !SAC can become available while a computer is running Windows Server 2003 in normal mode, safe mode, or the GUI-mode phase of Windows Server 2003 setup. Unlike SAC, you cannot invoke !SAC directly. After a specific failure point is reached, Emergency Management Services components make the transition from SAC to !SAC.

!SAC provides a subset of SAC commands and allows you to perform a limited set of tasks, including the following:

- Remotely view Stop message text.

- Restart the computer.

- View an abbreviated log of loaded drivers and some kernel events.

- Obtain computer identification information.

For more information about using !SAC, see “Using !SAC” and “Using Emergency Management Services to Troubleshoot Problems” later in this chapter.

# Understanding Out-of-Band Components

Depending on the out-of-band management tasks you must perform, you might need to use Emergency Management Services in conjunction with out-of-band hardware components. You can manage nearly every operating system state by using only Emergency Management Services and a serial port. However, if you need to manage many servers or need to manage servers in situations when the operating system is not functioning, you need supporting firmware and hardware components. These components can support situations ranging from power up to operating system initialization to unresponsive servers. You might need any combination of the following components:

- Serial or other types of out-of-band ports

- Service processors

- Firmware console redirection

- Terminal concentrators

- Intelligent uninterruptible power supplies or power switches

## Out-of-Band Ports

Serial ports are the most common out-of-band hardware interface, but service processors sometimes provide alternative methods. For example, some service processors provide an RJ-45 Ethernet port for out-of-band connections. In addition, advances in technology are opening up new possibilities for out-of-band ports. For more information about types of out-of-band ports, see “Out-of-Band Port Technology Trends” and “Service Processors” later in this chapter.

To be compatible with Emergency Management Services, out-of-band ports must meet certain requirements.

##### Exclusive access

The Windows Server 2003 operating system must have exclusive access to the out-of-band port. Instead of requesting input and output operations from the Windows I/O Manager, Emergency Management Services writes information directly to Universal Asynchronous/Receive Transmit (UART) hardware.

##### Single out-of-band port

Emergency Management Services supports only one out-of-band management port. Windows Server 2003 does not support a configuration in which one out-of-band port is used to send data and another one is used to receive incoming data.

##### Continuous availability

Emergency Management Services requires that the out-of-band port be available at all times, including when the Windows loader is running, the operating system is fully loaded, and when a Stop error occurs. Furthermore, the device must not be powered off while the system is running.

### Serial Ports

The serial port, which is also referred to as a COM port, is the default out-of-band device for Emergency Management Services. This is because serial ports have several important characteristics.

#### Characteristics of Serial Ports

At least one serial port exists on most computers.

###### Universality

Traditional serial ports use common 16550 or 16450 UART integrated circuits. UART interfaces are standard equipment on most motherboards. Hardware manufacturers can also incorporate inexpensive 16550 or 16450 UART interfaces into add-on adapters that provide out-of-band ports.

###### Simplicity and cost effectiveness

UARTs can be logically located at I/O addresses other than those traditionally reserved for COM1 or COM2 (provided that Emergency Management Services is properly configured to use the corresponding hardware settings). Serial ports enable external connectivity to other serial devices such as terminal concentrators, which act as network interfaces to out-of-band hardware.

###### Hardware flexibility

Several well-defined serial port communication standards exist, enabling interoperability with terminal hardware and software running on operating systems other than Windows, such as UNIX. The computer’s firmware initializes serial ports early in the startup process and serial ports are detected and usable before Windows Server 2003 starts.

###### Interpretability

For more information about Emergency Management Services serial port settings, see “Enabling Emergency Management Services” later in this chapter.

Note

When you use Emergency Management Services with a serial port, use null mode cables that support the Carrier Detect (CD) signal. Cables with the CD pin directly connected to the Request to Send pin also function correctly.

#### Serial Port Compatibility Requirements

In addition to the general requirements described earlier in this section, serial ports must meet a number of other requirements to be compatible with Emergency Management Services.

##### UART device standards

The UART control register must function as a standard 16450 or 16550 UART device. Before using a UART device for Emergency Management Services, Windows Server 2003 tests for this compliance.

##### UART device availability

The UART device must be available before the Windows loader starts. As a rule, out-of-band devices must be configured by the firmware as part of the POST process.

##### UART device I/O address

The UART device I/O address must not change. Emergency Management Services does not support Plug and Play. If you change UART hardware resource settings without making corresponding changes to Emergency Management Services settings, data might be written to an invalid system address.

For more information about out-of-band port requirements, see the Emergency Management Services Design link on the Web Resources page at http://www.microsoft.com/windows/reskits/webresources.

### Out-of-Band Port Technology Trends

A recent industry trend is the movement towards legacy free computers, those that do not use super I/O chips — a single chip that contains IDE controllers, a floppy controller, serial ports, and a parallel port — or serial ports located on the ISA bus. The PC 99 and PC 2001 System Design Guides strongly recommend alternatives to super I/O chips for such legacy-free designs. In these systems, a UART placed on a Peripheral Component Interconnect (PCI) adapter is the preferred mechanism for implementing out-of-band ports. Additionally, hardware manufacturers might develop proprietary out-of-band solutions that combine custom hardware and software solutions. Other possibilities for out-of-band external interfaces include connectivity based on industry standard Universal Serial Bus (USB), IEEE1394 Firewire ports, or other proprietary connections. Contact your hardware vendor for more information about technologies compatible with Emergency Management Services.

For more information about the PC 99 and PC 2001 design standards, see the PC Design Guidelines link on the Web Resources page at http://www.microsoft.com/windows/reskit/webresources.

## Service Processors

Emergency Management Services requires that the Windows loader or kernel be at least partially functioning. If you need to communicate with a server when this is not the case — such as when the computer is completely unresponsive due to a severe error — you might need a remote management channel that is independent of the operating system. Service processors can fill this need.

Service processors are micro controllers that operate independently of the computer processor(s) and operating system and use their own custom firmware. Service processors are typically integrated into the motherboard or a PCI adapter.

You can communicate directly with a service processor by using an out-of-band connection, regardless of the state of the computer. Depending on the manufacturer, a service processor might use any of a variety of out-of-band communication channels. The most common out-of-band channels are serial ports and RJ-45 Ethernet ports. However, if the out-of-band communication channel is a serial port, you can use only one tool — either Emergency Management Services or the service processor — at a time.

#### Serial port connections to service processors

When a service processor uses a serial port for out-of-band connections, it passively monitors the traffic between the UART interface and the serial port. This behavior is sometimes referred to as pass-through mode. When the service processor detects information in the data stream that applies to it (as defined in “Using Terminal Escape Sequences” later in this chapter), it takes control of the out-of-band port. The service processor then presents a remote management user interface. The service processor must release control of the serial port when either of the following occurs:

- The service processor detects control sequences in the serial data stream instructing it to immediately release control of the out-of-band port.

- The firmware POST process completes and the Windows Server 2003 loader starts.

Figure 10.1 illustrates how Emergency Management Services, the UART, a service processor, and a serial port fit together.

Figure 10.1   Serial Port Null Modem and Pass-Through Service Processor Configuration

#### Ethernet port connections to service processors

Some service processors include an integrated RJ-45 Ethernet port that is dedicated for out-of-band use. These network-enabled service processors typically use an Internet Protocol (IP) address different from the host computer and can be connected to a secondary management network for added security and redundancy.

#### Service processor features

Manufacturers vary in the management tools, features, and client interfaces they provide. Management tools and client interfaces range from simple Telnet consoles to complex Web browser environments. Some service processors also include their own power supplies to enhance reliability. Service processor features that you might want to use with Emergency Management Services include the following:

- Remote reset

- Console redirection

- Remote power on

- Remote power off

## Firmware Console Redirection

Firmware console redirection works with Emergency Management Services to provide out-of-band support for any operating state. If you do not have firmware console redirection — or a service processor that provides console redirection— you cannot manage your servers remotely until the Windows Server 2003 operating system begins loading.

For a seamless transition from firmware console redirection to Emergency Management Services console redirection, the firmware or service processor must completely release control of the out-of-band port when the Windows loader starts. For best results, terminal software running on the client computer needs to use serial port and terminal definition settings that are compatible with firmware console redirection and Emergency Management Services console redirection.

For more information about selecting terminal definition settings, see “Selecting Terminal Definition Settings” later in this chapter. For more information about configuring your hardware for Emergency Management Services, see “Configuring Firmware Settings for Emergency Management Services” later in this chapter.

## Terminal Concentrators

A terminal concentrator is a network-capable management device that consolidates serial access to multiple servers. Terminal concentrators have at least one Ethernet port that you can connect to a primary or secondary network and multiple serial ports for connecting to the managed servers. The remote management client connects to the terminal concentrator through the Ethernet port and the managed servers connect to the terminal concentrator with null modem cables.

By using a terminal concentrator, you can establish an in-band connection from a remote management computer, typically by using the Telnet protocol, to monitor many servers simultaneously. By sending specific commands to the terminal concentrator, you can switch from one serial port to another, and therefore from one server to another.

Although the in-band connection from the management client eliminates physical security concerns, it does introduce logical security concerns. Some terminal concentrators provide authentication and encryption security for the in-band connection. For example, some terminal concentrators support Secure Shell (SSH), which is a secure alternative to Telnet. Because security features for terminal concentrators are not standardized, however, you might need to provide your own logical security. For example, you might set up a secondary private management network or use a secure router. It is important to ensure that only authorized users can access a terminal concentrator, because it provides access to the credentials for multiple servers. For more information about terminal concentrators, see “Out-of-Band Security Considerations” later in this chapter.

For more information about how to configure a terminal concentrator for your environment, including security considerations, see “Planning for Remote Server Management” in Planning Server Deployments of the Windows Server 2003 Deployment Kit (or see “Planning for Remote Server Management” on the Web at http://www.microsoft.com/reskit).For more information about SSH, use a Web search engine and search using the keyword “SSH.”

## Intelligent Uninterruptible Power Supplies and Intelligent Power Switches

An Uninterruptible Power Supply (UPS) is an optional component that provides a backup power source to a server in the event of a power failure. Some UPS units, known as intelligent UPSs, allow you to remotely cycle power, providing rudimentary remote management. For Emergency Management Services systems with firmware redirection enabled, an intelligent UPS can enhance remote management capabilities by responding to remote commands sent to it.

When used with Emergency Management Services, an intelligent UPS must passively monitor the serial port traffic and support the key sequences that apply to it (as defined in “Using Terminal Escape Sequences” later in this chapter) and must not affect communication between the managing client and the server running a member of the Windows Server 2003 family operating system. If you also use a service processor, the intelligent UPS must be able to differentiate the key sequences directed to itself, the service processor, and Emergency Management Services. The UPS must also be able to inform the system of a planned power shutdown.

An intelligent power switch is an alternative to an intelligent UPS. An intelligent power switch can provide features similar to an intelligent UPS, except that it does not provide a backup source of power.

Out-of-Band Security Considerations

Note

To take full advantage of intelligent UPS or intelligent power switch capabilities, you need to configure servers to automatically start up when power is applied. Your server’s firmware might provide an option to enable this feature

Out-of-band connections might require both physical and logical security. The level of security that is available for out-of-band connections is highly dependent on the components you use and how you configure them. For example, if you use a modem to configure a remote serial connection, out-of-band security depends on the security built into the modem.

Null modem cables used for direct serial connections do not provide any logical security. Therefore, you need to provide physical security for these types of connections. Keep the servers in locked computer rooms with access restricted by keys, smart cards, or passwords. Use short length null modem cables to prevent anyone from extending the connections outside of the computer room.

Terminal concentrators consolidate access to multiple servers, which means that you can house the servers in centralized, secured rooms to provide physical security for the direct serial connections. The terminal emulation software supported by your terminal concentrator determines the level of built-in security provided. If your terminal concentrator does not provide built-in authentication and encryption, you might need to provide your own logical security for the in-band connection. If your network is visible to the public, you need to protect against unauthorized access to prevent network eavesdroppers from obtaining your server credentials and initiating denial of service attacks. For example, you might use a secure router or a private secondary management network.

If you use a custom out-of-band solution, consult your device documentation for information about security features.

For more information about planning for security of out-of-band connections, see “Planning for Remote Server Management” in Planning Server Deployments of the Windows Server 2003 Deployment Kit (or see “Planning for Remote Server Management” on the Web at http://www.microsoft.com/reskit).

# Configuring Headless Systems

When combined with the appropriate out-of-band management hardware (which includes redirecting firmware that allows the system to pass the POST without a video adapter), Emergency Management Services makes it practical to configure Windows Server 2003 systems for headless operation. Because you can manage and troubleshoot computers running Emergency Management Services by using in-band and out-of-band tools, a locally attached monitor, keyboard, and mouse are optional devices.

Headless systems do not prevent using input devices; depending on your hardware configuration, you might still be able to remove and attach devices, such as USB keyboards and mice, as needed.

Some computer manufacturers might offer pre-configured headless systems without a video adapter, mouse, and keyboard controller. For more information about purchasing a headless system, or configuring your system for headless operation, consult your system manufacturer.

The ability to remotely manage and perform disaster recovery without the need for physical access enables computers running Windows Server 2003 to be placed in secure server rooms rarely entered except for the purposes of maintaining or upgrading hardware.

Configuring Firmware Settings for Emergency Management Services

Note

Applications that use the DirectX API test for and require video display hardware. These applications might not function properly on a headless system.

Before deploying Emergency Management Services for your Windows Server 2003–based systems, you need to view and possibly change firmware settings that affect how Emergency Management Services operates. For most systems, you can use the setup utility built into the firmware. In other cases, you might need to use standalone software or manually manipulate motherboard switches to change hardware settings. For more information about changing hardware resource settings, refer to the documentation provided with your computer or motherboard.

The following settings affect how Emergency Management Services operates:

Caution

Do not modify firmware values unless you have a specific reason to do so. Using incorrect firmware settings can cause system instability and startup failures. If you experience problems after changing firmware settings, restore the settings recommended by your computer or motherboard documentation.

- Serial port settings

- Serial Port Console Redirection table settings

- Firmware console redirection configuration (enabled or disabled)

- Terminal definition settings

## Configuring Serial Port Settings

When preparing a serial port for out-of-band communications, first verify that the serial port is enabled. This is especially important if you have not previously verified that the ports are working properly. For most servers, serial ports are integrated into the system motherboard and can be enabled or disabled by using the setup utility built into the firmware.

To access the x86-based BIOS or Itanium-based EFI setup utility, follow the instructions specified in your computer documentation. Navigate to menu items that relate to device configuration. For example, configuration options similar to the following might be available:

- Chipset features setup

- On-board device setup

Typically, the preceding options contain submenus with additional choices. Look for submenus, similar to those that follow, that describe serial or COM port settings:

- Onboard Serial Port x

- Communications Port x

- COMx Port

In the preceding list, the variable x represents the port number, typically 1 or 2. Serial ports require hardware resources such as I/O addresses and hardware interrupt requests (IRQs). Depending on your system firmware, serial port resources might be user configurable or have Enabled or Disabled as the only available choices. Table 10.3 lists serial port hardware options that might be available on your system.

Table 10.3   Typical Serial Port Resource Settings

| Friendly Name | Typical Hexadecimal I/O Address | IRQ Value |
| --- | --- | --- |
| COM1 | Emergency Management Services supports the industry standard 3F8 (this is typically the firmware default or AUTO value for COM1). Values of 2F8, 3F8, On, and Off might also be available. | 4 (this is typically the default or AUTO value) |
| COM2 | Emergency Management Services supports the industry standard 2F8 (this is typically the firmware default or AUTO value for COM2). Values of 2F8, 3F8, On, and Off might be available. | 3 (this is typically the default or AUTO value) |

The IRQ values listed in Table 10.3 represent default values used by most systems. Assigning specific serial port IRQ values is usually not necessary for Emergency Management Services. Unless you have a specific reason to do so, it is recommended that you do not alter serial port hardware resource values. For general information about I/O addresses and IRQ system resources, see “Device Management” in the Server Management Guide of the Windows Server 2003 Resource Kit (or see “Device Management” on the web at http://www.microsoft.com/reskit).

If you specify COM1 as your out-of-band management port, Emergency Management Services by default uses I/O address 3F8. You can specify a different address if necessary. If another device is using COM1 at that I/O address, you have the option of reassigning the conflicting device to another serial port, or using a different serial port for Emergency Management Services.

For older systems, serial ports might be located on Industry Standard Architecture (ISA) expansion cards. If you find that Windows Server 2003 is not recognizing this type of serial port, you need to manually change resource values or consider upgrading hardware. Always review the manufacturer’s recommendations before changing internal component settings.

## Using the Serial Port Console Redirection Table

The Serial Port Console Redirection (SPCR) table, found on some computers that are equipped with Advanced Configuration and Power Interface (ACPI) compliant firmware, provides information about how the system firmware and the service processor, if available, use the out-of-band management port. Emergency Management Services uses the information in the SPCR table to ensure a smooth transition between the output of the system firmware and the output of Emergency Management Services.

If the out-of-band management port is a serial port, the SPCR table includes the serial port number, the baud rate, terminal type, and other settings. For out-of-band management ports that do not use traditional UARTs, the SPCR table contains information about the logical location and type of UART interface.

The Windows Server 2003 operating system uses information stored in the SPCR table to automatically configure Emergency Management Services hardware resource settings. If an SPCR table is not present, then you must configure Emergency Management Services settings to match firmware out-of-band hardware resource settings. Table 10.4 lists settings that the Windows Server 2003 operating system reads from the SPCR table.

Table 10.4   Out-of-Band Port Settings Used for Emergency Management Services

| Serial Port Firmware Options | Possible Settings | Required or Recommended Setting |
| --- | --- | --- |
| Console Redirection Port | COM1 or COM2 (COM3 or COM4 might also be available) Other non-legacy UARTs | If Emergency Management Services is configured to use a serial port, use the following firmware settings: - COM1 - COM2 if COM1 is not available If Emergency Management Services is configured to use an SPCR table (this includes all Itanium-based systems), use the following settings: - Any COM port. - Any I/O port in memory mapped I/O space. - Use SPCR table information. - Use the serial port EFI console device path (Itanium-based systems only). |
| Throughput | Your firmware or service processor might support settings from 9600 up to 115,200 baud. | For compatibility with other operating systems, Emergency Management Services uses 9600 baud by default. However, using higher baud rates might be possible. Other hardware within your Emergency Management Services infrastructure (such as terminal concentrators) might have specific baud rate constraints. Consult your hardware documentation for compatible throughput settings. |
| Data bits1 | 7 or 8 | 8 |
| Parity1 | On (enabled), None (disabled) | None (disabled) |
| Stop bits1 | 0 or 1 | 1 |

1The term 8N1 is sometimes used to refer to a setting of 8 data bits, no parity, and 1 stop bit.

When attempting to enable or disable SPCR-related settings, consider the following:

- The SPCR table is available only on computers with ACPI-compliant firmware. However, not all ACPI-compliant computers provide an SPCR table.

- You might not be able to locate a firmware option to enable SPCR table functionality. If you are unable to locate a firmware option to enable the SPCR table, try enabling firmware console redirection because the two features might be linked.

The SPCR table is typically enabled if firmware console redirection is enabled or a service processor that supports Emergency Management Services is present. See your system documentation to determine whether your computer supports the SPCR table and whether you need to follow special requirements to enable it.

For more information about SPCR table contents, see “Serial Port Console Redirection Table” at the Emergency Management Services Design link on the Web Resources page at http://www.microsoft.com/windows/reskits/webresources.

## Configuring Firmware Console Redirection

The level of support your system provides for serial port firmware console redirection might fall into one of the following categories:

##### Firmware console redirection is not available

Console redirection is not supported by the firmware.

##### Firmware console redirection is available

Console redirection is supported by the firmware.

##### SPCR table and firmware console redirection are available

An SPCR table is present, enabling Windows Server 2003 to automatically determine and use out-of-band hardware resource settings.

##### SPCR table, firmware console redirection, and a service processor are available

If your computer is equipped with a service processor, you might have additional management options, depending on the tools provided by the service processor manufacturer. Your service processor’s firmware might contain adjustable settings similar to those listed in Table 10.4.

For more information about configuring Windows Emergency Management Services settings, see “Enabling Emergency Management Services” later in this chapter.

## Selecting Terminal Definition Settings

- To successfully establish out-of-band connections, you must use communications software that supports one or more of the following terminal definitions:

- VT-UTF8

- VT-100+

- VT-100

- ANSI

VT-UTF8 is the preferred terminal type for viewing Emergency Management Services, firmware, or service processor output. If VT-UTF8 is not available, then VT100+ is the next best choice, followed by VT100. ANSI should be considered only as a last resort option.

Note

If your computer is equipped with a service processor, you might have additional options, such as a Web browser or custom terminal software provided by the hardware manufacturer.

- Terminal definition options vary, and you need to verify that terminal definition settings used by firmware, Emergency Management Services, and client terminal software are consistent. If you use the same settings for all these components, you can always use the same key sequences to manage a server, regardless of whether the firmware or Emergency Management Services is currently controlling the out-of-band port. If you do not use the same settings, you need to determine what is controlling the port before you can determine which key sequences to use.

##### VT-UTF8 and VT100+ terminal definitions

- The VT-UTF8 and VT100+ terminal definitions are well-defined standards that describe key mappings for all the keys on the standard PC 101-key layout. For example, these include escape sequences that define colors or keys such as the UP ARROW key or F12.

- VT-UTF8 is based on Unicode characters and is the preferred terminal type for Emergency Management Services output. For the English language, VT-UTF8 provides backwards compatibility with the VT100 and VT100+ terminal definitions. Additionally, VT-UTF8 provides support for other Unicode characters including Western Europe and Asian character sets and support for display-enhancing features such as color, block, and line drawing characters.

- VT100+ is a subset of VT-UTF8, but is a superset of VT100. English language Emergency Management Services output is VT100+ compatible. See “Using Terminal Escape Sequences” later in this chapter for Emergency Management Services–related VT100 and VT100+ key sequences.

##### VT100 terminal definition

- The VT100 terminal definition is the typical convention used for configuration and emergency management on UNIX-based servers. However, this terminal definition does not support all the keys on the standard PC 101-key layout, nor does it support color. If you select this terminal definition, you must manually enter escape sequences for certain keys. Use the VT100 terminal definition for a minimum level of compatibility with Emergency Management Services.

- See “Using Terminal Escape Sequences” later in this chapter, for Emergency Management Services related VT100 and VT100+ key sequences.

##### ANSI terminal definition

- The ANSI terminal definition does not support some of the features that are supported by VT100 to enhance terminal displays. Although ANSI is referred to as a terminal standard, an accepted universal specification has never been defined. The lack of a defined standard might result in compatibility problems such as incorrectly displayed characters. Use ANSI only as a last resort.

- Table 10.5 describes the terminal definition settings that might be provided by firmware and client terminal software.

Table 10.5   Terminal Definition Settings Related to Emergency Management Services

| Serial Port Firmware Parameter | Possible Settings | Required or Recommended Setting |
| --- | --- | --- |
| Language and local settings | English, Japanese, French, and others. | The language settings for firmware console redirection and the Windows loader must match whenever possible. If the same language is not available, set the firmware language to English. |
| Output terminal type for firmware console redirection | VT-UTF8, VT100, VT-100+, or ANSI | For systems that use an English language loader (such as English, BIDI, and Hebrew), Asian languages loaders, or for Itanium systems and systems that support the SPCR table, use the following settings in order of preference: - VT-UTF8 - VT100+ - VT100 - ANSI For systems that use localized loaders that support languages such as French, German, Korean, Japanese, Russian, and Spanish, with no support for the SPCR table, use VT-UTF8. If VT-UTF8 is not available, set the language to English and use one of the following terminal types in order of preference: - VT100+ - VT100 - ANSI |

For more information about the VT-UTF8 and VT100+ terminal definitions, see the Emergency Management Services Design link on the Web Resources page at http://www.microsoft.com/windows/reskits/webresources.

# Enabling Emergency Management Services

Emergency Management Services functionality is built into the Windows Server 2003 family operating system: It is not necessary to copy additional files from the Windows Server 2003 installation CD.

You can enable Emergency Management Services at any of the following times:

- During a new Windows Server 2003 installation (including RIS-based and image-based setups)

- During a Windows Server 2003 upgrade (for x86-based systems only)

- After you complete a Windows Server 2003 installation

## Enabling Emergency Management Services with New Installations

You can enable Emergency Management Services functionality as part of an unattended network installation, or if your system supports firmware console redirection, as part of an unattended CD-based or image-based installation. For computers that provide an SPCR table, Setup can automatically detect out-of-band port settings and enable Emergency Management Services.

During the text-mode phase of setup, you can use Emergency Management Services console redirection to remotely monitor progress. However, you cannot use firmware console redirection or Emergency Management Services console redirection to redirect GUI-mode setup graphics such as dialog boxes, windows, and the mouse cursor. Therefore, for setups not started in unattended mode, Setup pauses and requires user input when it reaches the GUI-mode phase. Additionally, network drivers are not yet available when the GUI-mode phase starts, so you cannot establish an in-band connection to the server. You can avoid this problem by configuring unattended setups, which allow Windows Server 2003 installations to proceed from start to finish without user intervention. Network-based unattended setups use the Unattend.txt file. CD-based and image-based unattended setups use the Winnt.sif file.

For headless servers, you can use Express Setup to perform an installation from the Windows Server 2003 operating system CD without using a Winnt.sif file. Express Setup provides default settings for computer configuration so that the GUI-mode phase can complete without interruption. After Setup is complete, you can use SAC to configure settings such as the IP address and computer name, and then you can use an in-band Remote Desktop connection to finish customizing the configuration. For more information about Remote Desktop connections, see “Remote Administration Overview” and “Remote Management Tools” in the Server Management Guide of the Windows Server 2003 Resource Kit (or see “Remote Administration Overview” and “Remote Management Tools” on the Web at http://www.microsoft.com/reskit). Also, see “Configuring Remote Desktop” in the Microsoft® Windows® XP Professional Resource Kit Documentation (or see “Configuring Remote Desktop” on the Web at http://www.microsoft.com/reskit).

To begin a CD-based Windows Server 2003 installation on a headless system, you must enable firmware console redirection so that you can remotely view and respond to the Press any key to boot from CD-ROM prompt that appears when the system starts from the Windows Server 2003 operating system CD.

For more information about available Setup options, navigate to the \Support\Tools directory on the Windows Server 2003 operating system CD and double click Deploy.cab. The Deploy.cab file contains two Help files, Microsoft® Windows® Corporate Deployment Tools User’s Guide (Deploy.chm) and Microsoft® Windows® Preinstallation Reference (Ref.chm), and the Setup Manager (Setupmgr.exe). The Help files provide detailed information about creating unattended setups and the Setup Manager is a tool that you can use to simplify the process of creating custom unattended Setup answer files.

### Unattend.txt for Network-Based Setups

Network administrators typically use Unattend.txt answer files to automate network-based installations. Network locations that contain Windows Server 2003 installation files typically hold at least one custom Unattend.txt file. To enable Windows Emergency Management Services for an unattended network-based setup, use the options listed in Table 10.6. These options are in the [Data] section of the Unattend.txt file.

Table 10.6   Emergency Management Services Installation Options

| Option | Possible Values | Comments |
| --- | --- | --- |
| EMSPort | UseBIOSSettings Comx – where x is the serial port number. | This option enables or disables Emergency Management Services. UseBIOSSettings is the default setting. This setting uses the SPCR table to configure the out-of-band port for x86-based systems. It uses the EFI console device path to configure the out-of-band port for Itanium-based systems. If you use this setting and no SPCR table or appropriate EFI console device path is available, Emergency Management Services is not enabled. Comx is valid only for x86-based systems. This setting is not valid for Itanium-based systems. Use this setting if your system does not have an SPCR table. |
| EMSBaudRate | 9600 19200 57600 115200 | This option is available only for x86-based computers; it is not applicable for Itanium-based computers. The default value is 9600. The other values are possible, depending on the capabilities of the serial port. This option must be used with the EMSPort:COMx option; otherwise, the baud rate is ignored. |

The following section from an Unattend.txt file illustrates how to enable Emergency Management Services console redirection to COM1 at 19,200 baud.

[Data]

EMSPort=COM1

EMSBaudRate=19200

For more information about creating unattended setups, see “Designing Unattended Installations” in Automating and Customizing Installations of the Windows Server 2003 Deployment Resource Kit (or see “Designing Unattended Installations” on the Web at http://www.microsoft.com/reskit) and the Microsoft Windows Corporate Deployment Tools User’s Guide, which is included in the Deploy.cab file on the Windows Server 2003 operating system CD.

### Winnt.sif for CD-Based Setups

Because Windows installation CD media is non-writable, automated CD-based Windows Server 2003 installations must read custom setup information from a file in the floppy disk drive. One of the first tasks that Setup performs is to check the floppy disk drive to determine if a disk with a file named Winnt.sif is present. If Winnt.sif file is not present and the computer is headless, Express Setup runs and applies default configuration settings. If a Winnt.sif file is present, Setup uses the customized information in it to apply configuration settings.

You create the Winnt.sif file by copying an Unattend.txt answer file to a floppy disk and renaming it Winnt.sif. To enable Emergency Management Services during a CD-based setup that uses the Winnt.sif, include the options listed in both Table 10.6 and Table 10.7. These options are in the [Data] section of the Winnt.sif file.

Table 10.7   Additional Emergency Management Services Options for Winnt.sif

| Option | Recommended Values | Comments |
| --- | --- | --- |
| AutoPartition | 1 | This setting installs Windows on the first available partition that has adequate space and does not already contain an installed version of Windows. Either omit the AutoPartition option entirely or set the value to 1. If AutoPartition is set to 1, the /tempdrive option for Winnt32.exe is ignored during setup. If you do not set a value, text-mode Setup installs Windows on the partition where $Win_nt$.~ls is located. |
| MsDosInitiated | 0 | This setting informs the Windows Setup Loader that an unattended setup is running directly from the operating system CD. This value must always be 0. If you run an unattended setup directly from the operating system CD and do not set the value to 0, Setup fails at the start of GUI-mode. |
| UnattendedInstall | Yes | This setting Informs the Windows Setup Loader that an unattended setup is running directly from the operating system CD. The value must be Yes if you boot from the operating system CD to install Windows. If UnattendedInstall is set to Yes, MsDosInitiated must be set to 0. |

The following section from a Winnt.sif file enables Emergency Management Services at the default baud rate and sets console redirection to COM1 for CD-based Windows Server 2003 installations.

[Data]

EMSPort=COM1

AutoPartition=1
MsDosInitiated=0
UnattendedInstall=Yes

Remote Installation Services

Important

To use Winnt.sif files, you must set the CD-ROM as the primary boot device and put a non-bootable floppy disk containing a Winnt.sif file in the floppy disk drive.

You can perform a network-based installation of the Windows Server 2003 operating system on computers that have no operating system by using Remote Installation Services (RIS). For computers equipped with RIS-supported network adapters, RIS eliminates the need to use a Microsoft® MS-DOS® operating system–based network startup disk to access a network distribution folder. Emergency Management Services is implemented for RIS-based installations by using versions of Startrom.com that specifically support Emergency Management Services console redirection. These versions of Startrom.com enable unattended remote setup on a computer whose firmware does not support firmware console redirection.

Without the Emergency Management Services-enabled version of Startrom.com, you cannot remotely view and respond to the Press F12 for network boot prompt on computers whose firmware does not provide console redirection.

#### Pre-Boot eXecution Environment

The Pre-Boot eXecution Environment (PXE) version 2.1 standard defines the way that a network adapter can be used as a bootable device. PXE features are required to initiate RIS-based Windows Server 2003 setups without the operating system CD or additional software. Using PXE requires the following:

- A PXE-compliant network adapter.

- PXE-compliant firmware that provides a boot order option that enables you to set the network interface card as the primary startup device.

Computers equipped with PXE-compliant network interface cards display a Press F12 for network boot prompt at startup. Pressing the F12 key causes the RIS client to download and run a file named Startrom.com, a RIS component that starts network-based installations. Computers that support firmware console redirection are able to redirect PXE prompts.

Important

Check for the latest updates to network adapter PXE firmware. The firmware should be version .99N or later.

Computers that do not support firmware console redirection are not able to redirect the Press F12 for network boot prompt and the corresponding F12 key response unless they use a special Emergency Management Services-enabled version of Startrom.com.

#### Versions of Startrom.com

The Startrom.com file is the first file that a RIS client downloads to begin the boot process. Multiple versions of this file are available to support Emergency Management Services console redirection. Select the version you need, as described in Table 10.8, and rename it to Startrom.com. Make your selection based on the following criteria:

- The serial port that you plan to use

- Whether your server supports firmware console redirection

Table 10.8   Startrom.com Versions

| Startrom.com Filename | Serial Port | Supports firmware console redirection? |
| --- | --- | --- |
| Hdlscom1.com | COM1 | No |
| Hdlscom2.com | COM2 | No |
| Hdlscom1.n12 | COM1 | Yes |
| Hdlscom2.n12 | COM2 | Yes |

All these versions of the Startrom.com file redirect at 9600 baud. If you want to monitor Setup from a remote management client, you must also have the client set to 9600 baud to be able to read the redirected output.

Hdlscom1.com and Hdlscom2.com

Note

These versions support Emergency Management Services console redirection. However, if you have firmware console redirection, it is preferable to rely on it instead and use the standard Startrom.com file, because the standard version is more flexible in terms of baud rate.

Use Hdlscom1.com and Hdlscom2.com for computers that do not support firmware console redirection. These files redirect the Press F12 for network boot prompt to serial port 1 or 2. Hdlscom1.com and Hdlscom2.com start a network-based setup process when you press F12 on the remote computer, or exit the network-based startup process if you do not press F12.

##### Hdlscom1.n12 and Hdlscom2.n12

Use Hdlscom1.n12 and Hdlscom2.n12 for computers that support firmware console redirection. These files do not redirect the Press F12 for network boot prompt displayed by the firmware or network adapter. Hdlscom1.n12 and Hdlscom2.n12 start a network-based setup process when you press F12 on the remote computer, or exit the network-based startup process if you do not press F12.

By default, these files are located in the \\servername\Reminst\Oschooser\i386 folder, where servername is the name of the RIS server running the Boot Information Negotiation Layer (BINL) service. To use one of these files, you must rename it to Startrom.com.

RIS Startup Scenarios

Important

The default device boot order for many systems is typically set to start the computer by using the floppy disk or CD-ROM (if bootable media is present), followed by the hard drive, and the network interface card last. You might need to change the default boot order to use PXE features.

Depending on the capabilities of your computer’s firmware, one of the following scenarios occurs when you set a PXE network interface card as the primary boot device in a RIS environment and restart your computer:

##### Scenario 1

The computer’s firmware or network adapter displays the Press F12 for network boot prompt and uses the network adapter as the startup device regardless of the boot order settings. Remotely pressing the F12 key causes the RIS client to download and run Startrom.com. If you do not press F12, the network boot process ends, and the remote system attempts to use the next device in the boot order sequence. This is the default PXE implementation for most systems.

##### Scenario 2

The system uses the network adapter as the startup device according to the device boot order sequence stored in firmware. For example, if the boot order is set to A, Network, C, the computer starts from the network adapter only if a bootable floppy is not present. If a bootable floppy is not present, the computer’s firmware displays the Press F12 for network boot prompt. If you do not press F12, the network boot process ends and the system moves to the next device in the boot order sequence.

For the two preceding scenarios, if your computer supports firmware console redirection, the Press F12 for network boot prompt is redirected and you should use Hdlscom1.n12 or Hdlscom2.n12. Otherwise, use Hdlscom1.com or Hdlscom2.com.

Emergency Management Services can determine the appropriate out-of-band port to use by reading the SPCR table, if present, or by using the same out-of-band port used by Startrom.com. If your computer does not provide an SPCR table, you can configure Emergency Management Services settings in the Ristndrd.sif file, which is the RIS version of the Unattend.txt answer file. For RIS-based setups, you can associate custom answer files to specific setup images by using the RIS Client Installation Wizard. For more information about creating RIS-based unattended setups, see “Designing RIS Installations” in Automating and Customizing Installations of the Microsoft Windows Server Deployment Resource Kit (or see “Designing RIS Installations” on the Web at http://www.microsoft.com/reskit). For details about Remote Installation Services and PXE, see the Microsoft Technet link on the Web Resources page at http://www.microsoft.com/windows/reskits/webresources.

## Enabling Emergency Management Services During Windows Upgrade

You can enable Emergency Management Services as part of an upgrade from a previous version of Windows on x86-based systems. Upgrade options are not available on Itanium-based systems.

To start an operating system upgrade, use Winnt32.exe, which is located on the Windows Server 2003 operating system CD. When you start Winnt32.exe, you can use the Emergency Management Services-related options listed in Table 10.6 earlier in this chapter.

To enable Emergency Management Services during an upgrade

- Insert the Windows Server 2003 operating system CD into your computer, connect to a network share, or map a network drive connection to the path containing the installation files.

- Start the command prompt from the Start menu by clicking Run and typing cmd in the Open box.

- Type drive\i386\winnt32.exe or \\server\share\i386\winnt32.exe followed by one or any combination of the options listed in Table 10.6. The variable drive represents the drive letter assigned to the CD-ROM drive or network share.

The following example illustrates how to upgrade using installation files located on drive E, and specify an out-of-band port on serial port 1 at a baud rate of 115,200:

e:\i386\winnt32.exe /EMSPort:COM1 /EMSBaudRate:115200 /unattend

Alternatively, you can upgrade and keep all the existing configuration settings, as illustrated by this example:

e:\i386\winnt32.exe /unattend

In this case, after Setup is complete you can use a Remote Desktop connection to modify any configurations. When you use this method, you can monitor the text mode of Setup from a remote client computer.

Winnt32.exe is a GUI-based tool, so you cannot monitor Winnt32.exe status by using Windows Emergency Management Services console redirection. To runWinnt32.exe remotely, use an in-band tool such as Remote Desktop or an equivalent application.

Winnt32.exe copies setup files to your system and then prompts you to confirm a system restart. After restarting, text-mode setup begins, followed by the GUI mode phase. In text-mode phase, Emergency Management Services console redirection allows you to monitor Setup progress by viewing Setup messages. During GUI-mode phase, the SAC prompt appears and you cannot use Emergency Management Services to view or respond to Setup prompts.

Enabling Emergency Management Services After Setup

Note

When you specify the /unattend option, Winnt32.exe reads information from the registry and uses the information to automate the upgrade process.

If you choose not to enable Emergency Management Services when you install the Windows Server 2003 operating system, you can enable it at a later time. At any time after Setup has been run, you can enable Emergency Management Services for use when the Windows loader and operating system run or for use with the Recovery Console.

### Enabling Emergency Management Services for Windows

If you choose to enable Emergency Management Services on systems that are already running Windows Server 2003, you need to use the Bootcfg.exe tool to add Emergency Management Services settings to the Boot.ini file. These settings enable console redirection when the Windows loader and operating system run.

For x86-based systems, you can use the Bootcfg.exe tool from the Windows command line or the bootcfg command from Recovery Console. For Itanium-based systems, you can use the Bootcfg.exe tool from the Windows command line.

You can also automate Emergency Management Services changes in the Boot.ini file by using custom scripts. For more information about scripting, see the Windows Script Technologies link on the Web Resources page at http://www.microsoft.com/windows/reskits/webresources.

Using the Bootcfg.exe tool for x86-based systems

Note

The Recovery Console bootcfg command is not the same as the Windows Bootcfg.exe command-line tool. Bootcfg.exe is located in the systemroot\System32 folder and is a standalone command-line tool that you cannot use in Recovery Console.

To enable Emergency Management Services by using the Bootcfg.exe tool from the Windows command line, use the /ems option, as follows:

bootcfg /EMS RedirectionValue [/S system [/U [domain\]user [/P password]]] [/PORT port] [/BAUD baudrate] [/ID bootID]

The /ems option allows you to enable or disable Emergency Management Services, or to modify current settings.

Table 10.9 lists Bootcfg.exe options and parameters that you can use to enable or configure Emergency Management Services.

Table 10.9   Bootcfg.exe Emergency Management Services Parameters

| Option / Parameter | Description |
| --- | --- |
| RedirectionValue | - Use ON to enable Emergency Management Services. - Use OFF to disable Emergency Management Services. - Use EDIT to change current settings. |
| /S system | Specifies the name of the remote system to connect to. The default is the local computer. |
| /U [domain\]user | Specifies the user context under which the command should execute. The default is the credentials for the user who is issuing the command. |
| /P password | Specifies the password for the given user context. |
| /PORT port | Specifies the COM port to be used for redirection. Valid ports are COM1, COM2, COM3, COM4, or BIOSSET (specifies that Emergency Management Services uses SPCR table settings). |
| /BAUD baudrate | Specifies the baud rate to be used for redirection. Valid baud rates are 9600, 19200, 57600, 115200. |
| /ID bootID | Specifies the boot entry number for which you are enabling or disabling Emergency Management Services. Typically, the boot entry number is the boot menu sequence number for multiple-boot systems, or 1 for single-boot systems. To verify the boot entry number, type bootcfg /query. You must enter this parameter if RedirectionValue is set to ON or OFF. |

For more information about verifying the boot entry number, see “Verifying Emergency Management Services Installation” later in this chapter.

The following example of a Boot.ini file illustrates the options that enable Emergency Management Services. These Emergency Management Services options were added to the Boot.ini file by using the following Bootcfg.exe command:

bootcfg /ems on /port com1 /baud 9600 /id 1

[boot loader]

timeout=30

default=multi(0)disk(0)rdisk(0)partition(1)\WINDOWS

redirect=com1

redirectbaudrate=9600

[operating systems]

multi(0)disk(0)rdisk(0)partition(1)\WINDOWS="Microsoft Windows Server" /redirect

For more information about Bootcfg.exe commands, in Help and Support Center for Windows Server 2003 click Tools, and then click Command-line reference A-Z.

#### Using the bootcfg command from Recovery Console

To enable Emergency Management Services by using the Recovery Console bootcfg command, use the /redirect option, as follows:,

bootcfg /redirect [[COMPort] [baudrate]] | [usebiossettings]

The /redirect option specifies console redirection for the Windows loader.

Table 10.10 describes the parameters for the /redirect option.

Table 10.10   Recovery Console Bootcfg Parameters for Emergency Management Services

| Parameter | Possible Values | Comments |
| --- | --- | --- |
| COMport | COM1 COM2 COM3 COM4 | Specifies the serial port to use. |
| baudrate | 9600 19200 57600 115200 | Specifies the baud rate to use for the Emergency Management Services COM port chosen. |
|  | usebiossettings | Specifies that the SPCR table be used to determine the serial port and baud rate. If an SPCR table is not present, Emergency Management Services is not enabled. |

For more information about the bootcfg command, type bootcfg /? in Recovery Console.

#### Using the Bootcfg.exe tool for Itanium-based systems

To enable Emergency Management Services on Itanium-based systems, you need to use the Bootcfg.exe tool from the Windows command line to edit EFI boot manager settings. Use the /ems option, as described for x86-based systems earlier in this section, with the following options.

bootcfg /EMS RedirectionValue [/ID bootID]

For more information about the meaning of each parameter, see Table 10.9 earlier in this section.

### Enabling Emergency Management Services Console Redirection in Recovery Console

Recovery Console is a command-line environment that you can start from the Windows Server 2003 operating system CD. For x86-based systems, you can also install Recovery Console as a startup option. By using Recovery Console you can perform advanced operations such as stopping services and replacing corrupted files. You have the choice of using either firmware console redirection or Emergency Management Services console redirection for Recovery Console I/O.

Enabling Emergency Management Services console redirection for Windows does not automatically enable console redirection for Recovery Console. To enable Emergency Management Services console redirection for Recovery Console on x86-based systems, you have the following options:

- If you have not yet installed Recovery Console, you can enable Emergency Management Services as part of Recovery Console setup.

- If you have installed Recovery Console, you can manually edit the Winnt.sif file in the Cmdcons folder on the system partition root.

Enabling Emergency Management Services During Recovery Console Installation

Note

The Winnt.sif file for Recovery Console is not the same as the floppy disk Winnt.sif file that is used to automate CD-based Windows Server 2003 installations.

For x86-based systems, you can enable console redirection as part of Recovery Console installation by using the following syntax:

drive:\i386\winnt32.exe /cmdcons /emsport:{com1|com2\usebiossettings\off} /emsbaudrate:baudrate

-or-

\\server\share\i386\winnt32.exe /cmdcons /emsport:{com1|com2\usebiossettings\off} /emsbaudrate:baudrate

Refer to Table 10.6 earlier in this chapter for a list of available parameters.

#### Editing the Winnt.sif File

You can also enable console redirection for Recovery Console by editing the systemdrive\Cmdcons\Winnt.sif file. Add the parameters listed in Table 10.6 to the [Data] section, and restart the computer. The following example illustrates the changes required to enable Emergency Management Services console redirection to COM1 at 9600 baud in Recovery Console.

[data]

EMSPort=COM1

EMSBaudRate=9600

## Verifying Emergency Management Services Installation

To verify that Emergency Management Services is active without actually connecting devices to the out-of-band port, you can use Bootcfg.exe or examine the registry.

### Using Bootcfg.exe to Verify Installation

You can use Bootcfg.exe to verify that Emergency Management Services options are present in the Boot.ini file on x86-based computers and in Nonvolatile Random Access Memory (NVRAM) on Itanium-based computers.

To use Bootcfg.exe to view boot entries

- In the Run dialog box, type bootcfg /query.

The following output illustrates the results of the query:

Boot Loader Settings

--------------------

redirect:         com1

redirectbaudrate: 115200

timeout:          30

default:          multi(0)disk(0)rdisk(0)partition(1)\WINDOWS

redirect:         com1

redirectbaudrate: 115200

Boot Entries

------------

Boot entry ID:    1

OS Friendly Name: "Microsoft Windows Server 2003, Enterprise Edition"

Path:             multi(0)disk(0)rdisk(0)partition(1)\WINDOWS

OS Load Options:  /fastdetect /redirect

Boot entry ID:    2

OS Friendly Name: "Microsoft Windows Recovery Console"

Path:             C:\CMDCONS\BOOTSECT.DAT

OS Load Options:  /cmdcons

### Using the Registry Editor to Verify Installation

If enabled, Emergency Management Services stores configuration information in the registry subkey Headless. You can use the registry editor Regedit.exe to check for the presence of the subkey HKEY_LOCAL_MACHINE\SYSTEM\CurrentControlSet\Services\Headless. The contents of this subkey should never be modified. In addition, adding this subkey to the registry does not enable Emergency Management Services functionality.

Using Emergency Management Services

Caution

Do not edit the registry unless you have no alternative. The registry editor bypasses standard safeguards, allowing settings that can damage your system, or even require you to reinstall Windows. If you must edit the registry, back it up first and see the Registry Reference on the Microsoft Windows Server 2003 Resource Kit companion CD or at http://www.microsoft.com/reskit.

- You can use SAC or !SAC to perform out-of-band management tasks during distress situations by using terminal emulation software or the Telnet protocol. Because SAC and !SAC do not rely on in-band connections, you can manage a server when normal network connectivity is lost.

## Using SAC

SAC is an operating system component that runs in kernel mode and displays the following prompt when you establish an out-of-band connection to a computer running Emergency Management Services:

SAC>

In addition to providing a variety of commands that support remote management, SAC provides access to the Windows command prompt and to Setup logs. When you use SAC, you can create multiple user sessions or channels and switch among them so that you can use SAC commands while concurrently running command-line commands or viewing Setup logs.

#### SAC Commands

Table 10.11 lists the commands available in the SAC environment. The commands are not case-sensitive. Some commands require you to specify a process identifier or PID number; you can obtain PID numbers by using the t command, which causes SAC to list active processes and the PIDs assigned to them.

Table 10.11   SAC Commands

| SAC Command | Description |
| --- | --- |
| ? or help | Lists the available SAC commands. |
| ch | Lists all available channels. See Table 10.12 for a complete list of channel management commands. |
| cmd | Creates a Windows command prompt channel once logon credentials are validated. |
| Crashdump | Manually generates a Stop message and forces a memory dump file to be created. |
| D | Dumps the current kernel log |
| f | Toggles the information output of the tlist command between showing processes only or processes and threads |
| I | Configures IP parameters by providing network number, IP address, and subnet information using the following format. <network#> <iPaddress> <subnet> If no parameters are passed, this command lists IP information. |
| id | Displays computer identification information. |
| k <PID> | Ends the process specified by its process ID (PID). |
| l < PID> | Lowers the priority of a process (and any associated child processes) to the lowest possible level. |
| lock | Locks access to command prompt channels. |
| m < PID> <MB-allow> | Limits the memory usage of a process (and any child processes) to the specified number of megabytes. |
| p | Causes tlist command output to pause after displaying one screen of information. |
| r < PID> | Raises the priority of a process and any associated child processes by one level. |
| restart | Restarts the computer immediately. |
| s | Displays the current date (using 24-hour clock format) if you provide no parameters. Sets the system time if you provide date and, optionally, time in the following format: mm/dd/yyyy hh:mm |
| shutdown | Shuts down the computer |
| t | Lists the processes and threads currently running; the output is referred to as a task list or tlist. |

SAC Channels

Note

The SAC prompt (SAC>) might not immediately appear after a connection to a remote server running Emergency Management Services is established. If the SAC prompt does not appear, Press ENTER several times.

SAC supports three types of channels: SAC, Windows command prompt, and Setup logs.

##### SAC Channel

The SAC channel (which is always active) is assigned a channel name of SAC and a channel number of 0 (zero).

##### Windows Command Prompt Channels

The Special Administration Console Helper service (Sacsvr) is a user mode service that enables you to create command prompt channels when you type the cmd command from the SAC prompt. To use a Windows command prompt channel, you must provide valid logon credentials that have administrative rights on the server. You must log on separately to each command prompt session. SAC assigns a name of “Cmd000x” to all command prompt channels, where x represents the next available channel number.

When using a Windows command prompt channel, you can run text-based tools as though you were logged on locally. These tools include command shell commands, such as dir, and text-based console programs, such as the boot configuration tool (Bootcfg.exe).

As with other services, the Special Administration Console Helper service requires initialization time when Windows Server 2003 starts. Therefore, command prompt channels might not be immediately available, and you might have to wait until the operating system startup process is nearly complete.

Setup Log Channels

Note

The Windows command prompt might not be available or might become unavailable if system resources become low. In these situations, you can typically continue to manage the server by using SAC.

You can access the Setup log channels only during the GUI-mode phase of Windows Server 2003 setup. Viewing the following Setup logs allows you to monitor the installation process or diagnose problems that might cause Setup to stop or pause.

Provides information about files copied to your system by Windows Setup.

###### Setuplog.txt

Provides information about actions performed by Windows Setup.

###### Setupact.log

Provides Setup-related warning and error information.

###### Setuperr.log

The following illustrates output generated by the SAC ch command during the GUI-mode phase of Windows Server 2003 setup.

SAC>ch

Channel List

(Use "ch -?" for information on using channels)

# Status  Channel Name

0 (AV)    SAC

1 (AR)    setuplog.txt

2 (AR)    setupact.log

3 (AR)    setuperr.log

#### SAC Channel Commands

You can create and switch among up to ten concurrent channels. Table 10.12 provides a list of channel commands.

Table 10.12   SAC Channel Commands

| Channel Command | Description |
| --- | --- |
| ch | Lists all channels. |
| ch -si <#> | Switches to a channel by number. Press ENTER to confirm your channel selection. |
| ch -sn <name> | Switches to a channel by name. Press ENTER to confirm your channel selection. |
| ch -ci <#> | Closes a channel by number. Press ENTER to confirm your channel selection. |
| ch -cn <name> | Closes a channel by name. Press ENTER to confirm your channel selection. |
| <Esc> followed by <Tab> | Cycles through the available channels. Press ENTER to confirm your channel selection. |
| <Esc> then <Tab> followed by 0 | Returns to channel 0 (the SAC channel). Press ENTER to confirm your channel selection. |

SAC returns channel status information in the following format:

<Channel #> <Two-character status> <Channel name>

The following illustrates output generated by the SAC ch command:

SAC>ch

Channel List

(Use "ch -?" for information on using channels)

# Status  Channel Name

0 (AV)    SAC

1 (AV)    Cmd0001

2 (AV)    Cmd0002

To switch from SAC (Channel 0) to the next available channel (Channel 1), press ESC followed by TAB , and then press ENTER. To return to SAC, press ESC followed by TAB , type 0, and then press ENTER.

Table 10.13 describes the information provided by the two-character channel status code.

Table 10.13   SAC Channel Status Information

| Status Code (AB) | Description |
| --- | --- |
| A (First Character) | Indicates channel status. A = Active Channel, I = Inactive Channel |
| B (Second Character) | Channel terminal emulation type. V = VT-UTF8, R = Raw (no emulation) |

Using !SAC

Note

Although you can create and use multiple channels to manage a computer running Windows Server 2003, you can only access a single channel at a time. Therefore, multi-user access to SAC is not possible.

!SAC provides a last-resort environment if a computer running Windows Server 2003 stops responding. When !SAC becomes available, the following prompt appears:

!SAC>

The !SAC prompt might be preceded by a Stop message that contains information about the problem that caused the error condition to occur.

#### !SAC Commands

Table 10.14 lists available !SAC commands.

Table 10.14   !SAC Commands

| !SAC Command | Description |
| --- | --- |
| ? or help | Lists available commands. |
| D | Displays all log entries (screen pauses at each page of information). |
| Id | Displays computer identification information. |
| Restart | Restarts the computer. |

No other channels are available when !SAC becomes active. Therefore, it is not possible for multiple users to access !SAC simultaneously.

Preventing Automatic Restarts After a Stop Message

Note

The !SAC prompt might not immediately appear after you establish a connection to a remote server running Emergency Management Services. If the !SAC> prompt does not appear, press ENTER several times.

An Automatically reboot setting in Control Panel controls whether your system restarts after a Stop message occurs. This option is enabled by default. It causes your computer to restart after a Stop message instead of enabling !SAC. To use !SAC after a Stop message occurs, you must disable the Automatically reboot setting. This setting is important for troubleshooting because it enables you to remotely view and record Stop message text for later reference while searching through technical resources, such as Microsoft Knowledge Base articles.

To disable an automatic restart after a Stop message

- From the Start menu, click Settings, and then click Control Panel.

- Double-click the System icon.

- Select the Advanced tab. In Startup and Recovery, click Settings.

- Clear the Automatically reboot checkbox.

## Using Terminal Escape Sequences

If your terminal software does not support VT100+ or VT-UTF8, you cannot directly use all the keys on the standard PC keyboard. However, you can set your terminal software to use VT100 and enter the VT100+ or VT-UTF8 keystrokes manually. For example, to send an F1 or F2 keystroke to a remote computer from a terminal using the VT-UTF8 or VT100+ terminal definition, press F1 or F2. However, from a VT100 terminal, you must press and release <ESC> and type 1 to send an F1 keystroke, or press and release <ESC> and type 2 to send an F2 keystroke (as listed in Table 10.15).

Table 10.15   VT100+ and VT-UTF8 Keyboard to Escape Sequence Mappings

| VT100+ or VT-UTF8 Key | Equivalent Keystrokes When Using VT100 |
| --- | --- |
| Home | <ESC>h |
| End | <ESC>k |
| Insert | <ESC>+ |
| Delete | <ESC>- |
| Page Up | <ESC>? |
| Page Down | <ESC>/ |
| Alt | <ESC>^A |
| Ctrl | <ESC>^C |
| F1 | <ESC>1 |
| F2 | <ESC>2 |
| F3 | <ESC>3 |
| F4 | <ESC>4 |
| F5 | <ESC>5 |
| F6 | <ESC>6 |
| F7 | <ESC>7 |
| F8 | <ESC>8 |
| F9 | <ESC>9 |
| F10 | <ESC>0 |
| F11 | <ESC>! |
| F12 | <ESC>@ |

Table 10.16 lists special control and escape sequences reserved for special management commands to the computer firmware, intelligent UPS, intelligent power switch, or service processor. Consult your computer or power infrastructure documentation to determine the specific commands supported.

Table 10.16   Special Control and Escape Sequences

| Required Escape Sequence | Action |
| --- | --- |
| ^F | Invoke the built-in firmware setup utility |
| <ESC>R<ESC>r<ESC>R | Reset the computer without displaying a user interface The computer will reset, without shutting down Windows. This command requires a service processor, intelligent power switch, or intelligent UPS support. |
| <ESC>( | Invoke the service processor The service processor monitors the out-of-band port data stream for this escape sequence. If detected, the service processor assumes control of the serial port and immediately sends an acknowledgement. Further interaction with Windows Emergency Management Services is not possible until you exit the service processor interface. This command requires service processor support. If the service processor was previously invoked, it must send the acknowledge sequence defined in this table within 1 second, before sending any other communication. |
| <ESC>) | Invoke the uninterruptible power supply (UPS) or intelligent power switch The intelligent UPS or intelligent power switch monitors the out-of-band port data stream for this escape sequence. If detected, the intelligent UPS assumes control of the serial port and immediately sends an acknowledgement. Further interaction with Emergency Management Services is not possible until you exit the intelligent UPS interface. This command requires an intelligent UPS or an intelligent power switch device. If the UPS was previously invoked, it sends the acknowledge sequence defined in this table within 1 second, before sending any other communication. |
| <ESC>^ | Wake-up or power-on If the service processor supports sending a power signal to the computer, it does so. If the system is already fully powered on, the service processor must not affect the operation of the system in response to this command. |
| <ESC>* | Intelligent UPS, intelligent power switch, or service processor acknowledgement After taking control of the out-of-band port, the UPS or Service Processor sends this escape sequence. This indicates that the intelligent UPS, intelligent power switch, or service processor has assumed control of the out-of-band port and is ready to receive commands. This command requires service processor, intelligent power switch, or intelligent UPS support. |
| Q | Exit intelligent UPS, intelligent power switch, or service processor (might display a user interface). The intelligent UPS, intelligent power switch, or service processor releases control of the out-of-band port. Before it does so, it might request confirmation in the form a message such as “Do you want to save settings and exit? [y/n]”. This command requires service processor, intelligent power switch, or intelligent UPS support. |
| <ESC>Q | Exit UPS or service processor (does not display a user interface) The intelligent UPS, intelligent power switch, or service processor releases control of the out-of-band port control without displaying a user interface. This command requires service processor, intelligent power switch, or intelligent UPS support. |

For complete information about the VT100 terminal definition, see the VT100 link on the Web Resources page at http://www.microsoft.com/windows/reskits/webresources.

## Using Emergency Management Services to Troubleshoot Problems

Because SAC is always available as long as the Windows Server 2003 kernel is running, you can use it during many system states to troubleshoot problems by executing SAC commands, as listed in Table 10.11 earlier in this chapter. You might also be able to use control or escape sequences, as listed in Table 10.16 earlier in this chapter, to send management commands to other out-of-band components. If a system failure occurs, you might be able to use !SAC commands.

You can use Emergency Management Services to troubleshoot problems during any of the following system states:

##### Powering on or off

The computer is powering up or is in the process of shutting down.

##### Firmware initializing the system

The computer is running the POST, performing rudimentary hardware checks, and determining the devices available.

##### Windows loading

This is the startup phase in which the Windows loader and kernel load and initialize system components such as drivers and services. For multiple-boot systems, the Windows loader might display a boot menu listing the operating systems installed. Based on your selection, the appropriate system files are loaded and initialized.

##### Windows running

Windows Server 2003 starts without problems and transitions into one of the following states:

- Text-mode setup phase.

- GUI-mode setup phase.

- Normal operation.

##### Windows in distress

Windows Server 2003 is experiencing software or hardware problems that might adversely affect system responsiveness or interrupt network connectivity.

##### Windows unresponsive

Windows Server 2003 is unresponsive to normal in-band connection methods. You can use Emergency Management Services to attempt recovery. However, if software or hardware problems are sufficiently severe, Emergency Management Services might become unavailable. In these situations, the ability to remotely recover from problems depends on the hardware features built into the system firmware, motherboard, service processor, and UPS.

### Options for Troubleshooting

This section describes Emergency Management Services options for troubleshooting problems based on the system states previously described. Some options, such as a service processor, might not be available for your server. Also, not all UPSs support out-of-band management features. Consult your system and power supply documentation to determine the specific features supported.

If a problem prevents Windows Server 2003 from starting properly, you might need to use recovery features that the operating system provides, such as Last Known Good Configuration, Safe Mode with networking, or Recovery Console. The following are typical problems and troubleshooting options.

##### Power is off

- Send the appropriate escape sequence (listed in Table 10.15) to the service processor to power-on the computer.

- Send the appropriate escape sequence (listed in Table 10.15) to the intelligent UPS or intelligent power switch to supply power to the computer.

- Use Wake-on-LAN functionality by sending a network request that causes the computer to awaken from a power management sleep state. This feature must be supported by both your system motherboard and network adapter firmware. Follow the instructions provided with your computer and network adapter to enable this functionality.

##### POST or pre-startup fails

Windows Server 2003 cannot start if your computer stops responding before completing the POST.

- Connect to the service processor and restart the computer, or perform diagnostic procedures by using out-of-band management tools provided by the service processor.

- Use the intelligent UPS or intelligent power switch to restart the computer.

- If your computer supports firmware redirection, verify that the firmware settings are correctly set according to the system documentation. Adjust the values if necessary. If the problem persists, an on-site investigation and possibly repair to computer components might be required.

##### Loader or kernel initialization fails

- Use SAC or !SAC to restart the computer, or view log information by using the d command. See Table 10.11 and Table 10.14 for a list of SAC and !SAC commands.

- If the server stops responding and SAC and !SAC become unavailable, see the options under “System Stops Responding” later in this section.

##### Text-mode Setup fails

- If a Stop message occurs, use !SAC to restart the system and restart Setup.

- If the problem persists, verify that your Unattend.txt or Winnt.sif unattended setup files are correctly configured. See “Enabling Emergency Management Services with New Installations” earlier in this chapter.

- If the server stops responding and SAC and !SAC become unavailable, see the options under “System Stops Responding” later in this section.

##### GUI-mode Setup fails

- Monitor the progress of GUI-mode Setup or check the Windows Setup logs by viewing the Setup log channels. For more information about Setup log channels, see “Using SAC” earlier in this chapter.

- If a problem occurs during GUI-mode Setup, use SAC to restart the system and restart Setup.

- If the problem persists, verify that your Unattend.txt or Winnt.sif unattended setup files are correctly configured. See “Enabling Emergency Management Services with New Installations” earlier in this chapter.

- If a Stop message occurs, use !SAC to restart the system and restart Setup.

- If the server stops responding and SAC and !SAC become unavailable, see the options under “System Stops Responding” later in this section.

##### Performance severely degrades

- If certain processes deplete system resources until you cannot access the computer with in-band tools or cannot log on, use SAC to terminate the problematic process or lower its priority level.

- Use the d command to view SAC or !SAC log information.

- If the condition does not improve or you cannot determine which process is causing the problem, restart the system by using SAC.

- If a Stop message occurs due to low system resources, use !SAC to restart the system.

- If the server stops responding and SAC and !SAC become unavailable, see the options under “System Stops Responding” later in this section.

##### Stop error occurs

- If a Stop error occurs and the automatic restart option is disabled, use !SAC to view Stop message information and restart the system. For more information about disabling automatic restarts when a Stop message occurs, see “Using !SAC” earlier in this chapter.

- If the server stops responding and !SAC becomes unavailable, see the options under “System Stops Responding” later in this section.

##### System stops responding

- If SAC and !SAC become unavailable, connect to the service processor and restart the computer, or perform diagnostic procedures by using out-of-band management tools provided by the service processor.

- Restart the computer by using the intelligent UPS, intelligent power switch, or by manually cycling power.

### Troubleshooting Examples

The following examples illustrate out-of-band management using SAC and !SAC.

#### Example 1

A domain controller stops responding to user logon requests. You use the ping command to obtain the server’s IP address and computer name. Although the server successfully responds to the ping request, you are unable to establish a network connection to the server, preventing you from using Windows Server 2003 in-band management tools. As part of your troubleshooting efforts, you previously verified that the network infrastructure is otherwise functioning properly. Because Emergency Management Services requires only modest system resources, you can use Emergency Management Services to diagnose this situation and change the priority of a process that is consuming excessive resources.

To troubleshoot example 1 by using Emergency Management Services

- Start your terminal software or Telnet client to access the domain controller running Emergency Management Services. The steps required to do this depend on the software and out-of-band solution used by your organization.

- From the SAC> prompt, type id to verify that you are connected to the correct server.

- Type t to obtain a list of processes, as well as the PID and processor time used by each. Wait at least five seconds and type t again. Compare the two lists and note the PID of the processes consuming the greatest amount of processor and kernel resources.

- Type l <PID> to lower the priority of the process consuming the most resources.

- If you are certain that a process is causing the problem, you can choose to end the errant process by typing kill <PID>.

- Wait five seconds and type t again to verify that you have successfully ended or lowered the priority of the process.

- Attempt to log on to the network.

- After logging on, you can use in-band management tools, such as Remote Desktop, to further troubleshoot the problem.

#### Example 2

You install a new network driver, which causes the server to immediately stop responding. You can use Emergency Management Services to check if a Stop error occurred, and use Emergency Management Services console redirection to troubleshoot the problem.

To troubleshoot example 2 by using Emergency Management Services

- Start your terminal software or Telnet client to access the computer running Emergency Management Services. The steps required to do this varies depending on the software and out-of-band solution used by your organization.

- If a Stop error occurred, !SAC redirects Stop message text to the out-of-band port. In this case, you verify that a Stop error has occurred. Record the Stop message information displayed. Type the command restart to restart the computer.

- Console redirection enables you to remotely access the Windows Advanced Options Menu. Select Last Known Good Configuration to reverse the network driver update.

- Use the Stop message information to search for more information about the problem on the Microsoft Knowledge Base. For more information, see the Microsoft Knowledge Base link on the Web Resources page at http://windows.microsoft.com/windows/reskit/webresources.

### Task List Details

You can use the SAC t command as described in troubleshooting example 1 to provide details about the status of your computer. This command displays information that represents a snapshot of the server’s system state and can be used to detect and resolve issues related to system resource consumption. The following is an example task list created by using the t command.

User Time   Kernel Time    Ws   Faults  Commit Pri Hnd Thd  Pid Name

21360   330048                          File Cache

0:00:00.000  21:07:49.897    20        1       0  0    0   1    0 Idle Process

0:00:00.000   0:03:17.774    44    22176      32  8  683  61    4 System

0:00:00.010   0:00:00.180   200      687     180 11   22   3  336 smss.exe

0:00:11.176   0:00:38.585  1612    16024    1680 13  544  12  392 csrss.exe

0:00:09.153   0:00:14.711  4684    41567   15692 13  632  21  416 winlogon.exe

0:00:06.779   0:00:28.591  2364    14667    3744  9  344  16  460 services.exe

0:00:35.881   0:00:23.233  4608    48543    4396  9  474  32  472 lsass.exe

0:00:00.711   0:00:00.580  1792     2788    1100  8  228   9  640 svchost.exe

0:00:00.961   0:00:00.460  1188     8121    3876  8  182  17  692 svchost.exe

0:05:22.874   0:03:15.781 12388  3999657   20008  8 1342  50  820 svchost.exe

0:00:00.520   0:00:00.610  1104     4952     612  8   71   4  848 svchost.exe

Table 10.17 explains the meaning of each column in the task list display.

Table 10.17   SAC Task List Parameters

| Column | Description |
| --- | --- |
| User Time | - The amount of user-mode time consumed by a running process. A process can be an executable program, such as Microsoft® Word, or an operating system component, such as the print spooler service. User time information is displayed in H:MM:SS format. |
| Kernel Time | - The amount of kernel-mode time consumed by a running process. Information is displayed in H:MM:SS format. |
| Ws | - Working set memory for a process. The amount of virtual memory resident in physical memory. |
| Faults | - The number of times a process requested memory that was not resident in memory, requiring the information to be read from the disk virtual memory paging file. |
| Commit | - The total amount of virtual memory allocated to a process. |
| Pri | - The priority of a process. |
| Hnd | - The number of handles assigned to a process. Handles are used to uniquely identify objects, such as a file or registry key. |
| Thd | - The number of threads running within a process. A thread allows concurrent operations within a process and enables different portions of a program to run simultaneously on different processors. |
| Pid | - A numerical program identifier assigned to a process. |
| Name | - Friendly name of a process. |

# Troubleshooting Emergency Management Services

Under some circumstances, Emergency Management Services or firmware console redirection might not function as expected. This section describes problems that you might observe and discusses ways to resolve them.

## Black-Out and Gray-Out Periods

Windows Server 2003 components that provide Emergency Management Services capabilities load and unload as part of the startup process. During these transition periods, components might be briefly disabled or provide limited functionality, and the following conditions might occur:

- The out-of-band port output might briefly be unavailable, with no displayed status information. This period is known as a black-out period.

- Status information is displayed, but Emergency Management Services does not respond to terminal input. This is known as a gray-out period. These periods might become interactive if the component provides !SAC functionality.

Emergency Management Services is designed to minimize black-out and gray-out periods. However, if you experience problems with SAC or !SAC not responding, wait to allow additional time for Windows Server 2003 components to initialize before attempting to restart or power down the system by using the service processor, intelligent power switch, or intelligent UPS. Also, consult your system documentation to determine what other diagnostic tools your service processor provides.

## Kernel Debuggers and Emergency Management Services

Emergency Management Services and a kernel debugger cannot simultaneously use the same serial port. Using the same serial port for Emergency Management Services and a kernel debugger interferes with proper operation of both and the information displayed becomes unreadable. Therefore, you must devote a different serial port to each one. Consequently, you must be careful to specify the correct COM port parameters in the Boot.ini file for Emergency Management Services and the kernel debugger.

Additionally, when a kernel debugger is running, !SAC becomes unavailable because the debugger takes control of the system when a Stop error occurs. To avoid this problem, disable kernel debugging on servers that have Emergency Management Services enabled.

To disable kernel debugging

- Make a backup of the existing Boot.ini file.

- From the command-prompt, type:

bootcfg /debug off /id x

The value x specifies the operating system boot entry ID in the [operating system] section of the Boot.ini file. For more information about using Bootcfg.exe to enable or disable debugging, in Help and Support Center for Windows Server 2003 click Tools, and then click Command-line reference A-Z.

## Guidelines for Troubleshooting Emergency Management Services

Emergency Management Services is designed to be reliable and straightforward to use. However, you might encounter problems if system settings are not configured properly. The tables in this section list and describe such problems and the methods to resolve them.

The problems described in Table 10.18 occur when POST console redirection is in effect.

Note

The following guidelines assume that you are using the correct cables and that they are connected properly.

Table 10.18   Problems that Occur Before Windows Starts

| Problem | Course of Action |
| --- | --- |
| Your server’s firmware supports serial port console redirection, but no text output appears during the POST. | - Verify that firmware console redirection is enabled. - Verify that terminal settings for your communication software match those used for the out-of-band serial port. Otherwise, information received might be distorted or not appear on the remote display. - Test the physical COM port assignments by attaching the null modem cable to another serial port. The COM1 and COM2 ports might not be clearly labeled or their assignments might be reversed. - Check that the serial ports are enabled and that their assigned motherboard or serial adapter hardware resources do not conflict with other devices. - Verify that your null modem cable supports the carrier detect (CD) signal. - Check for and install new firmware versions. |
| You computer supports console redirection via the service processor, but no text output appears during the POST. | - Consult your computer documentation for information about configuring the service processor. |
| You are unable to view text when using configuration utilities built into adapter ROM (SCSI cards, for example). | - Consult your computer documentation for information about updated adapter firmware or ROM that supports console redirection and headless systems. - Some adapters send status information to the screen and do not function properly if a video adapter is not present. Contact the manufacturer for more information about upgrading or purchasing new hardware that supports headless configurations. |

The problems described in Table 10.19 occur when Emergency Management Services console redirection is in effect.

Table 10.19   Problems that Occur When Windows Is Loading

| Problem | Course of Action |
| --- | --- |
| Your server’s firmware supports console redirection and has an SPCR table. You configure Emergency Management Services to use SPCR table settings, but no text output appears when the loader starts. | - Verify the items listed in Table 10.18 earlier in this section. - Use Bootcfg.exe to verify that Emergency Management Services is enabled. For more information, see “Enabling Emergency Management Services After Setup” earlier in this chapter. - If you are using the UseBIOSSettings parameter, your computer must be ACPI-compliant and must support the SPCR table. Setup does not enable Emergency Management Services when you use the UseBIOSSettings parameter on a system without an SPCR table. - For x86-based systems, use Bootcfg.exe with the /EMS parameter to specify a serial port. Try using different console redirection settings available for the firmware until text output appears. For more information, see “Enabling Emergency Management Services After Setup” earlier in this chapter. |
| Your server’s firmware either does not support console redirection or does not support the SPCR table, and you used Bootcfg.exe to specify a COM port in the Boot.ini file. Text output does not appear when the loader starts. | - Verify the items listed in Table 10.18 earlier in this section - Follow the instructions provided in “Enabling Emergency Management Services” earlier in this chapter. Verify that the Redirect= parameter specified in the Boot.ini [boot loader] section match firmware serial port resources. - For x86-based systems, use Bootcfg.exe with the /EMS parameter to specify a serial port. Try using different console redirection settings available for the firmware until text output appears. For more information, see “Enabling Emergency Management Services After Setup” earlier in this chapter. |
| This is an Itanium system and the loader does not redirect text output | - For Itanium-based systems, the loader does not provide console redirection and the firmware must provide console redirection to allow you to remotely select boot manager options. Consult your system documentation for information on how to configure a serial port or the service processor as a console device. |

The problems that occur in Table 10.20 occur when SAC is typically available.

Table 10.20   Problems that Occur When Windows Is Running

| Problem | Course of Action |
| --- | --- |
| Windows Server 2003 provides console redirection for the loader, but SAC is unavailable. | - Verify the items listed in Table 10.18 earlier in this section - Follow the instructions provided in “Enabling Emergency Management Services After Setup,” earlier in this chapter. To enable SAC and !SAC, you must use Bootcfg.exe with the /EMS parameter . - Verify that other software is not using the same resources or serial port. Programs such as a kernel debugger can interfere with Emergency Management Services operation. For more information see “Kernel Debuggers and Emergency Management Services” earlier in this chapter. - Verify that you are running Windows Server 2003. Windows XP Professional does not support Emergency Management Services. - Verify that you allowed enough time for the startup process. Windows Server 2003 kernel components might be in the process of initializing, and this might be a black-out or gray-out period. |
| This is an Itanium system and the SAC console never becomes available. | - Verify the items listed in Table 10.18 earlier in this section - Verify that the EFI firmware supports the SPCR table or has a serial port specified on the console output device path. - Follow the instructions provided in “Enabling Emergency Management Services After Setup,” earlier in this chapter. - Verify that you are running Windows Server 2003. Windows XP Professional does not support Emergency Management Services. |

The problems that occur in Table 10.21 occur when !SAC is typically available.

Table 10.21   Problems that Occur After a Stop Message

| Problem | Course of Action |
| --- | --- |
| SAC and !SAC are unavailable when a Stop message occurs. | - If the system is set to restart automatically when a Stop message occurs but !SAC does not become available, follow the procedure in “Using !SAC” earlier in this chapter. - Disable kernel debugging by using Bootcfg.exe. For more information, see “Kernel Debuggers and Emergency Management Services” earlier in this chapter. - Restart the computer by using the service processor, intelligent power switch, or intelligent UPS. |

If your out-of-band port implements custom hardware or software solutions, you might have additional troubleshooting options. For information about these options, consult your out-of-band hardware documentation.

# Additional Resources

These resources contain additional information related to this chapter.

##### Related Information in the Resource Kits

- “Remote Administration Overview” in this book.

- “Remote Management Tools” in this book.

- “Device Management” in this book for more information about hardware resource settings.

- “Planning for Remote Server Management” in Planning Server Deployments of the Windows® Server 2003 Deployment Kit (or see “Planning for Remote Server Management” on the Web at http://www.microsoft.com/reskit).

- “Configuring Remote Desktop” in the Windows® XP Professional Resource Kit Documentation (or see “Configuring Remote Desktop” on the Web at http://www.microsoft.com/reskit).

- Automating and Customizing Installations of the Windows Server 2003 Deployment Resource Kit (or see Automating and Customizing Installations on the Web at http://www.microsoft.com/reskit).

##### Related Information Outside the Resource Kits

- The Extensible Firmware Interface link on the Web Resources page at http://www.microsoft.com/windows/reskits/webresources.

- The Emergency Management Services Design link on the Web Resources page at http://www.microsoft.com/windows/reskits/webresources.

- The PC Design Guidelines link on the Web Resources page at http://www.microsoft.com/windows/reskit/webresources for more information about the PC 99 and PC 2001 design standards.

- The Microsoft Technet link on the Web Resources page at http://www.microsoft.com/windows/reskits/webresources for more information about Remote Installation Services and PXE.

- The Windows Script Technologies link on the Web Resources page at http://www.microsoft.com/windows/reskits/webresources.

- The VT100 link on the Web Resources page at http://www.microsoft.com/windows/reskits/webresources for more information about the VT100 terminal definition.

##### Related Help Topics

- Microsoft Windows Corporate Deployment Tools User’s Guide from Deploy.chm in the Deploy.cab file in the \Support\Tools directory on the Windows Server 2003 operating system CD for more information about creating files for unattended installations.

- “Bootcfg” in Help and Support Center for Windows Server 2003.