Universal Audio Architecture Hardware Design Guidelines

June 12, 2006

Abstract

The Microsoft Universal Audio Architecture (UAA) initiative, in combination with PC audio device technologies, is key to delivering a simple yet compelling user experience with current and future versions of the Microsoft® Windows® operating systems. Devices that meet UAA hardware requirements can take advantage of Microsoft-provided class drivers that provide complete driver support for audio devices for Windows.

This paper provides design guidelines for meeting UAA hardware requirements for USB audio, IEEE 1394 AV/C audio and Intel HD Audio. This information is intended for PC system manufacturers, BIOS developers, and manufacturers of PC audio devices.

This information applies for the following operating systems:
	Microsoft Windows Vista™
	Microsoft Windows Server™ 2003
	Microsoft Windows XP
	Microsoft Windows 2000

The current version of this paper is maintained on the Web at:
http://go.microsoft.com/fwlink/?LinkId=50734.

Contents

Introduction	3

Features of UAA	4

Scenarios Supported by UAA Class Drivers	7

Benefits of UAA for Hardware Manufacturers	8

UAA Driver Models	9

UAA Driver Model for USB Audio Devices	9

UAA Driver Model for IEEE 1394 AV/C Audio Devices	10

UAA Driver Model for HD Audio Devices	12

UAA Design Guidelines for USB Audio Devices	14

Streaming Data	14

Isochronous Endpoint Types	15

Topology	15

UAA Design Guidelines for IEEE 1394 AV/C Audio Devices	16

AV/C Device Capabilities	16

AV/C Streaming Formats	18

Audio Subunit Controls	19

Control Notifications	19

AV/C Audio Driver Defaults	19

UAA Design Guidelines for HD Audio Devices	20

Design Guidelines for HD Audio Devices	20

Design Guidelines for HD Audio Bus Controllers	24

Design Guidelines for HD Audio Codecs	25

Design Guidelines for PC Systems with HD Audio Solutions	26

UAA and the Windows Logo Program	27

References	27

Disclaimer

This is a preliminary document and may be changed substantially prior to final commercial release of the software described herein.

The information contained in this document represents the current view of Microsoft Corporation on the issues discussed as of the date of publication. Because Microsoft must respond to changing market conditions, it should not be interpreted to be a commitment on the part of Microsoft, and Microsoft cannot guarantee the accuracy of any information presented after the date of publication.

This White Paper is for informational purposes only. MICROSOFT MAKES NO WARRANTIES, EXPRESS, IMPLIED OR STATUTORY, AS TO THE INFORMATION IN THIS DOCUMENT.

Complying with all applicable copyright laws is the responsibility of the user. Without limiting the rights under copyright, no part of this document may be reproduced, stored in or introduced into a retrieval system, or transmitted in any form or by any means (electronic, mechanical, photocopying, recording, or otherwise), or for any purpose, without the express written permission of Microsoft Corporation.

Microsoft may have patents, patent applications, trademarks, copyrights, or other intellectual property rights covering subject matter in this document. Except as expressly provided in any written license agreement from Microsoft, the furnishing of this document does not give you any license to these patents, trademarks, copyrights, or other intellectual property.

Unless otherwise noted, the example companies, organizations, products, domain names, e-mail addresses, logos, people, places and events depicted herein are fictitious, and no association with any real company, organization, product, domain name, email address, logo, person, place or event is intended or should be inferred.

© 2003-2005 Microsoft Corporation. All rights reserved.

Microsoft, Windows, Windows NT, Windows Server, and Windows Vista are either registered trademarks or trademarks of Microsoft Corporation in the United States and/or other countries.

The names of actual companies and products mentioned herein may be the trademarks of their respective owners.

Intel High Definition Audio Disclaimer

The information in this document is furnished for informational use only, is subject to change without notice, and should not be construed as a commitment by Intel Corporation. Intel assumes no responsibility or liability for any errors or inaccuracies that may appear in this document or any software that may be provided in association with this document.

CERTAIN INFORMATION IN THIS DOCUMENT IS PROVIDED IN CONNECTION WITH INTEL PRODUCTS. NO LICENSE, EXPRESS OR IMPLIED, BY ESTOPPEL OR OTHERWISE, TO ANY INTELLECTUAL PROPERTY RIGHTS IS GRANTED BY THIS DOCUMENT. EXCEPT AS PROVIDED IN INTEL'S TERMS AND CONDITIONS OF SALE FOR SUCH PRODUCTS, INTEL ASSUMES NO LIABILITY WHATSOEVER, AND INTEL DISCLAIMS ANY EXPRESS OR IMPLIED WARRANTY, RELATING TO SALE AND/OR USE OF INTEL PRODUCTS INCLUDING LIABILITY OR WARRANTIES RELATING TO FITNESS FOR A PARTICULAR PURPOSE, MERCHANTABILITY, OR INFRINGEMENT OF ANY PATENT, COPYRIGHT OR OTHER INTELLECTUAL PROPERTY RIGHT. Intel products are not intended for use in medical, life saving, life sustaining applications. Intel may make changes to specifications and product descriptions at any time, without notice.

Designers must not rely on the absence or characteristics of any features or instructions marked "reserved" or "undefined." Intel reserves these for future definition and shall have no responsibility whatsoever for conflicts or incompatibilities arising from future changes to them.

This document contains information on products in the design phase of development. Do not finalize a design with this information.

Introduction

The Microsoft Universal Audio Architecture (UAA) initiative, in combination with PC audio device technologies, is key to delivering a simple yet compelling user experience with current and future versions of the Microsoft® Windows® operating systems.

This paper provides design guidelines and information about UAA compatibility for PC system manufacturers, BIOS developers, and manufacturers of PC audio devices.

Goals of the Microsoft UAA Initiative. The Microsoft UAA initiative has the following goals:

- Provide users with a class driver architecture that provides basic audio functionality in the operating system.

- Offer an alternative to third-party drivers for users who experience compatibility problems with audio on their systems or who might not need third-party, value-added audio features.

Several classes of audio devices that Windows directly supports can take advantage of UAA support in the operating system if the devices are UAA compliant. To be UAA compliant, an audio device must implement all relevant industry standards and follow the additional UAA design guidelines presented in this paper. An audio device that is UAA compliant can rely entirely on the class driver provided by the operating system for driver support, making it unnecessary for the hardware vendor to supply a custom driver for the device. The UAA design guidelines apply to all audio devices in Windows PCs.

The Windows Vista™ operating system provides class drivers for the following classes of audio devices:

- USB audio

- IEEE 1394 AV/C (to be released some time after the first release of Windows Vista)

- Intel High Definition Audio

In addition, Microsoft is providing UAA driver support for Windows Server™ 2003, Windows XP, and Windows 2000.

Note

In this paper, "Windows Vista and earlier versions of Windows" includes Windows Vista, Windows Server™ 2003, Windows XP, and Windows 2000. It does not include Windows Me or Windows 98 except as noted.

USB audio and IEEE 1394 AV/C. For external bus audio devices, existing USB audio and IEEE 1394 AV/C standards make it possible for Microsoft to provide class drivers to support audio hardware that is compatible with these standards.

- USB audio devices that are compatible with the USB audio class system driver, usbaudio.sys, in Windows Vista and earlier versions of Windows are automatically UAA compliant, without additional work on the part of the manufacturer. Usbaudio.sys in Windows 98 and Windows Me is not a UAA class driver and the UAA initiative does not cover those operating systems.

- Similarly, IEEE 1394 AV/C audio devices that are compatible with the IEEE 1394 AV/C audio class system driver, avcaudio.sys, in Windows Vista, are automatically UAA compliant.

This paper provides guidelines for UAA compatibility for USB audio and IEEE 1394 AV/C audio and High Definition Audio devices with Windows Vista and earlier versions of Windows. For information about the class drivers usbaudio.sys and avcaudio.sys, see the Windows Driver Kit (WDK) documentation.

High Definition Audio. An HD Audio device can either reside on an internal audio adapter or be integrated into a chip set on a motherboard. An HD Audio device that complies with the Intel High Definition Audio specification and follows the design guidelines provided in this paper and other papers published by Microsoft is automatically UAA compliant, without additional work on the part of the manufacturer. For information, see the HD Audio resources listed at the end of this paper.

The UAA class driver hdaudio.sys provides basic audio functions for HD Audio devices in Windows Vista. Microsoft is planning to provide a version of this driver that can be installed for Windows Server 2003, Windows XP, and Windows 2000.

Advantages of the UAA class driver architecture. In 2004, the introduction of the Intel High Definition Audio specification brought a new level of standardization to audio devices connecting to an internal bus such as the PCI or PCI Express bus. Before that time, differences among audio chip sets and add-in adapter designs prevented Microsoft from supplying a generic audio driver with the operating system for PCI audio devices. The lack of an industry standard compelled manufacturers to provide their own custom audio drivers.

Because of the diversity of mutually incompatible PCI audio devices, Microsoft was unable to guarantee support for all possible audio hardware solutions. If a user had problems with a third-party audio driver for a PCI audio device, no generic driver was available to control the basic audio functions of the device. Instead, the user experienced poor-quality audio, no audio, or system instability caused by an incompatible audio driver.

Hardware vendors whose audio devices require custom drivers eventually face the following problem: they must either continue to maintain drivers for audio devices that they no longer manufacture, or deal with unhappy customers. In contrast, a UAA-compliant audio device can use the UAA class driver, which Microsoft plans to maintain over successive operating system upgrades.

Hardware vendors whose USB audio, IEEE 1394 AV/C, or HD Audio devices are UAA compliant can rely on the UAA class drivers to support their devices. To take advantage of UAA support in Windows, audio devices must implement all relevant industry standards and follow the additional UAA design guidelines presented in this paper. For more information, see the UAA design guidelines for USB audio, IEEE 1394 AV/C, and HD Audio devices later in this paper.

## Features of UAA

The purpose of UAA is to improve the stability and reliability of the PC audio experience. Another goal of UAA is to raise the baseline quality and functionality of PC audio to a level that is comparable to that of consumer electronics devices.

The UAA class drivers provide the following benefits for UAA-compliant USB audio, IEEE 1394 AV/C, and HD Audio devices:

- Simpler installation of audio peripherals. The operating system can detect and configure a UAA-compliant audio device when it is connected to the system, without requiring the user to find and load a driver.

- Performance advantages. UAA class drivers are designed to consume a minimum amount of CPU resources during streaming, and to take advantage of increased bandwidth in hardware that can support data rates that are comparable to those of high-end consumer electronics.

- Glitch-free audio. In Windows Vista, the UAA class driver for HD Audio is designed to use the operating system's low-latency scheduling capabilities to achieve nearly glitch-free audio. For more information, see A Wave Port Driver for Real-Time Audio Streaming listed in the references at the end of this paper.

- Security for protected content. UAA class drivers support the content protection technologies in released and future versions of Windows.

- Evolving driver support. Through successive Windows releases, existing audio hardware devices automatically benefit from improvements made to UAA class drivers to use new operating system capabilities.

The following sections describe the features that the UAA class drivers support specifically for USB audio, IEEE 1394 AV/C, and HD Audio devices that are UAA compliant.

### USB Audio Devices

The UAA class driver for USB audio devices, usbaudio.sys, supports the following USB audio data types:

- In Windows Vista and earlier versions of Windows, the usbaudio.sys driver fully supports all Type I formats with the exception of signed 8-bit pulse-code modulation (PCM), for which there is no corresponding Windows format. Type I consists of uncompressed PCM-based audio formats.

- In Windows Vista, the usbaudio.sys driver fully implements Type III formats but recognizes only a subset of the data-type codes that are defined in the Universal Serial Bus Device Class Definition for Audio Data Formats, Release 1.0. This subset consists of the following formats:

AC-3 (Dolby Digital)
MP3
Windows Media Audio (WMA)
WMA Pro

Type III formats are based on the IEC 60958 and IEC 61937 (S/PDIF) formats for packaging compressed data into what is effectively a PCM-like stream.

In Windows Server 2003 and earlier versions of Windows, usbaudio.sys does not support any Type III formats.

- In Windows Vista, the usbaudio.sys driver provides full MIDI support, as defined in the Universal Serial Bus Device Class Definition for MIDI Devices, Release 1.0, including support for MIDI elements.

In Windows Server 2003 and earlier versions of Windows, usbaudio.sys provides full MIDI support, with the exception of MIDI elements.

For more information about the usbaudio.sys driver's support for USB audio data types, see "Streaming Data" later in this paper.

The usbaudio.sys driver supports the following additional features:

- Full translation of USB audio units to Windows Driver Model (WDM) kernel-streaming topologies.

- Full support for all units defined in the Universal Serial Bus Device Class Definition for Audio Devices, Release 1.0. (The version of usbaudio.sys in Windows Vista supports all units defined in release 2.0 of this specification, provided that this version of the specification is published before the release of Windows Vista.)

- Support for vendor-defined extension units for vendor-specific functions.

- Plug and Play support for hot-plugging and hot-unplugging of devices.

- Multiple simultaneous streams based on the capabilities determined from the device description.

- In Windows Vista, usbaudio.sys supports synchronous, asynchronous, and adaptive endpoints.

In Windows Server 2003 and earlier versions of Windows, usbaudio.sys supports adaptive and synchronous endpoints, but not asynchronous endpoints.

### IEEE 1394 AV/C Audio Devices

In Windows XP and later versions of Windows, the UAA class driver for IEEE 1394 AV/C audio devices, avcaudio.sys, supports the following data types:

- AM824 data formats, as defined by the IEC 61883 specification. More specifically, avcaudio.sys supports the IEC 60958 form of stereo (two-channel) pulse-coded modulation (PCM). The AM824 data can span a range of audio formats, including 16-bit, 20-bit, and 24-bit PCM audio data at sample rates of 32 kHz, 44.1 kHz, 48 kHz, and 96 kHz.

- S/PDIF digital pass-through formats, as defined by the set of IEC 61937 format specifications.

- Formats for transmitting multichannel PCM audio data to DVD-Audio devices. For more information, see Guideline of Transmission and Control for DVD-Video/Audio through IEEE1394 Bus, Version 1.0, listed in the references at the end of this paper.

In Windows Server 2003 and Windows XP, avcaudio.sys is not a true class driver and it supports only a subset of the data formats in the preceding list. For more information, see "AV/C Streaming Formats" later in this paper.

All versions of the avcaudio.sys driver support the following additional functions:

- Feature, mixer, and process function blocks

- Plug and Play support for hot-plugging and hot-unplugging of devices

The IEEE 1394 specifications incompletely define AV/C audio devices, so the avcaudio.sys driver requires some information outside of the specifications to ensure proper interoperability. For more information, see "AV/C Audio Driver Defaults" later in this paper.

### HD Audio Devices

In Windows Vista and earlier versions of Windows, the UAA class driver for HD Audio devices, hdaudio.sys, provides the following basic audio functions:

- Recording of multiple independent streams at the highest quality that the device can provide.

- Playback of multiple independent streams at the highest quality that the device can provide.

- Power management of the device according to industry-wide operating system standards:

- Support for D0 and D3 power states.

- Wake-from-S3 support.

- Power optimizations to allow subdevices to reach the D3 power state.

- Support for Secure Audio Path (Windows Media digital rights management).

- INF-based, programmable default audio settings for volume levels, microphone gain, and other audio parameters.

- Microphone boost control integrated into the gain control circuitry.

- Dynamic detection of insertion of peripherals into front-panel audio jacks.

- Configuration of rear-panel audio jacks for multichannel streaming or for default behavior.

- Parsing of device topology based on programmed default configurations.

- Fully localized driver and property page.

- Support for digital inputs and outputs:

- Support for 44.1-kHz, 48-kHz, 96-kHz, and 192-kHz PCM data through digital inputs and outputs.

- Support for S/PDIF pass-through transmission of non-PCM formats such as Dolby Digital (AC-3) and Windows Media Audio Professional (WMA Pro) to external devices (digital output only).

- A user setting to control whether plugging in headphones automatically disconnects the speakers.

- Support for the HD Audio control-reporting mechanism to notify the operating system of changes in hardware controls.

- Windows Management Instrumentation (WMI).

## Scenarios Supported by UAA Class Drivers

The UAA class drivers are designed to support the following scenarios::

- Operating system installation or upgrade. For Windows Vista and later versions of Windows, UAA provides guaranteed audio device coverage for operating system installation and upgrade.

- Stable, secure audio for server applications. UAA provides stable, secure audio for scenarios such as server applications in which stability and security are more important than advanced audio features.

- Reduced need for vendor-supplied drivers. UAA-compliant devices can rely on UAA class drivers in the operating system for basic audio support, which reduces the amount of driver code that must be developed, tested, and supported. However, vendors have the option of supplying custom drivers to support the advanced features of their devices.

The following sections describe these three scenarios.

### Operating System Installation or Upgrade Scenario

In this scenario, a user installs an audio device on a PC running Windows Vista. However, the vendor's driver for the device is unavailable for one of the following reasons:

- The third-party driver is incompatible with the new operating system.

- The third-party driver is not on the operating system CD, and the user does not have Internet access to look for the driver on Windows Update.

- The third-party driver is not on the operating system CD and the customer has Internet access, but the driver is not available on Windows Update.

- The third-party driver is not available because the vendor no longer supports the audio device or is no longer in business.

After the customer installs the audio device, the operating system loads the correct UAA class driver for the device. This scenario also encompasses system integrators who depend on operating-system-supplied drivers to build and sell PC systems, and users who upgrade their PCs from Windows Vista to a future version of the operating system.

### Stable, Secure Audio Scenario

In this scenario, a customer purchases or upgrades a UAA-compliant PC or server. The customer wants audio support but values stability and security over advanced audio features. The customer installs one of the Windows UAA drivers on the computer to guarantee stability and security without sacrificing basic audio support.

### Reduced Need for Vendor-Supplied Drivers Scenario

In this scenario, a vendor implements a UAA audio design with features supported by the UAA class driver. The vendor does not need to supply a custom driver to make the device work.

The same vendor develops a UAA-compliant audio device that provides some advanced audio features that are not supported by the UAA class driver. The vendor writes a custom driver that enables the advanced features of the device. The user can install the vendor's custom driver to enable all of the capabilities of the hardware, or the user can rely on the UAA class driver for basic audio features.

For both devices, the vendor can take advantage of the new audio user interface in Windows Vista to add branding and options that are specific to the device without also having to provide a custom application.

## Benefits of UAA for Hardware Manufacturers

UAA class drivers offer significant benefits to system manufacturers and audio device vendors:

- Less driver code to develop, test, and support. UAA class drivers provide basic audio support for UAA-compliant devices, which reduces the amount of driver code that must be developed, tested, and supported by the vendor. Vendors can expose additional audio hardware features through simpler driver components that interoperate with the UAA driver framework, which leaves vendors free to innovate beyond the capabilities of the UAA class driver.

- Increased stability for server SKUs. Manufacturers of servers and other systems for which uptime is critical can rely on UAA class drivers for stable, secure audio.

- Reduced exposure to product support calls and returns. Increased system stability and guaranteed device coverage provided by UAA class drivers should help to reduce product-support calls and product returns for UAA-compliant devices.

- Reduced need to upgrade drivers. UAA class drivers ensure that UAA-compliant devices are compatible with future versions of the operating system (following Windows Vista), which should reduce the need for manufacturers to upgrade drivers and provide sustained engineering on existing drivers.

- Easier phase-out for end-of-life products. Guaranteed basic audio support for UAA-compliant devices reduces the need for manufacturers to support older product lines past profitability.

# UAA Driver Models

UAA encompasses driver models for the following audio device types:

- USB audio

- IEEE 1394 AV/C

- HD Audio

The following sections describe the driver model for each of these device types.

## UAA Driver Model for USB Audio Devices

The UAA class driver for USB audio devices, usbaudio.sys, services USB audio devices that are UAA compliant. Microsoft recommends that hardware vendors use the UAA class driver for their USB audio devices instead of writing custom adapter drivers.

The usbaudio.sys driver is included in Windows Vista and earlier versions of Windows, including Windows Me and Windows 98. However, usbaudio.sys is a true class driver only in Windows Vista, Windows Server 2003, Windows XP, and Windows 2000. To work with this class driver, a USB audio device must be UAA compliant, which means that it must implement all relevant industry standards and follow the additional UAA design guidelines presented in this paper.

The usbaudio.sys driver is implemented as an AVStream minidriver that provides driver support for USB audio devices that comply with the Universal Serial Bus Device Class Definition for Audio Devices, Release 1.0. This specification is available at the USB Implementers Forum Web site listed in the references at the end of this paper. The usbaudio.sys driver supports a subset of the features that are described in the USB audio specification. For a list of UAA requirements for USB audio devices, see "UAA Design Guidelines for USB audio Devices" later in this paper.

The following figure shows the driver hierarchy for a USB audio device. All of the driver components shown in the figure are provided by Microsoft as part of the operating system.

Figure 1. Windows Driver Hierarchy for USB Audio Devices

During Plug and Play device enumeration, if an audio device identifies itself as USB audio-compliant, the system automatically loads the usbaudio.sys driver to drive the device. The usbaudio.sys driver manages the device directly, without the aid of a vendor-supplied, custom adapter driver. This means that a device that complies with the USB audio specifications requires no custom adapter driver.

If a USB audio device implements advanced features that are not supported by the corresponding UAA class driver, the hardware vendor can supply a function driver to enable those features. The custom hardware features that the vendor-supplied driver controls must be implemented in a way that that does not prevent the device from operating as a fully UAA-compatible device under control of the UAA class driver.

For more information, see "UAA Design Guidelines for USB Audio Devices" later in this paper.

## UAA Driver Model for IEEE 1394 AV/C Audio Devices

In Windows Vista, the UAA class driver for IEEE 1394 AV/C audio devices, avcaudio.sys, services IEEE 1394 devices that are UAA compliant. Microsoft recommends that hardware vendors use the UAA class driver for their IEEE 1394 audio devices instead of writing custom adapter drivers.

The avcaudio.sys driver in Windows Vista is a true class driver. To work with this class driver, an IEEE 1394 audio device must be UAA compliant, which means that it must implement all relevant industry standards and follow the UAA design guidelines in this paper.

An earlier version of the avcaudio.sys driver ships in the Windows Server 2003 and Windows XP releases. Although this version of the driver represents part of the UAA initiative, it is not a true class driver. It was developed to work specifically with the small number of audio devices that were available for the IEEE 1394 bus at the release of Windows XP. Thus, UAA compliance alone might not be sufficient to guarantee that an IEEE 1394 AV/C device works with this version of avcaudio.sys. For more information, contact Microsoft at the e-mail address listed at the end of this paper.

The avcaudio.sys driver is implemented as an AVStream minidriver that provides driver support for AV/C audio devices that comply with the relevant IEEE 1394 specifications listed in the references at the end of this paper. The avcaudio.sys driver supports a subset of the features that are described in the IEEE 1394 AV/C specifications. For more information, see "UAA Design Guidelines for IEEE 1394 AV/C Audio Devices" later in this paper.

The following figure shows the driver hierarchy for an IEEE 1394 audio device in Windows XP. In Windows XP and later versions of Windows, all of the driver components shown in this figure are provided by Microsoft with the operating system.

Figure 2. Windows XP Driver Hierarchy for IEEE 1394 AV/C Audio Devices

When an audio device identifies itself as an IEEE 1394 audio device during Plug and Play device enumeration, the system automatically loads the avcaudio.sys driver to drive the device. The avcaudio.sys driver manages the device directly, without the aid of a vendor-supplied, custom adapter driver. This means that a device that complies with the appropriate IEEE 1394 specifications requires no custom adapter driver.

If an IEEE 1394 AV/C audio device implements advanced features that are not supported by the corresponding UAA class driver, the hardware vendor can supply a function driver to enable those features. The custom hardware features that the vendor-supplied driver controls must be implemented in a way that does not prevent the device from operating as a fully UAA-compatible device under control of the UAA class driver.

For related design guidelines, see "UAA Design Guidelines for IEEE 1394 AV/C Audio Devices" later in this paper.

## UAA Driver Model for HD Audio Devices

Microsoft is developing new drivers for audio devices that implement the Intel High Definition Audio specification:

- The UAA class driver for HD Audio devices, hdaudio.sys, is a PortCls-based adapter driver that provides driver support for HD Audio codec devices.

- Microsoft is also providing an HD Audio bus driver, hdaudbus.sys, to support HD Audio bus controller devices.

The HD Audio specification describes a new audio architecture that is being developed as the successor to the Intel AC'97 specification. Audio devices that connect to the PCI or PCI Express bus and expose the HD Audio register set can be serviced by hdaudio.sys without requiring a custom driver from the hardware vendor.

Versions of hdaudio.sys and hdaudbus.sys are included with Windows Vista. In addition, Microsoft is planning to provide versions of these drivers that can be installed for Windows Server 2003, Windows XP, and Windows 2000. Microsoft recommends that hardware vendors use the UAA class driver for their HD Audio devices instead of writing custom function drivers.

The hdaudio.sys class driver provides driver support for UAA-compliant audio devices that implement the HD Audio register set and connect to an internal bus. No hardware-specific driver is needed for a UAA-compliant device. To be UAA compliant, HD Audio codec and controller devices must implement a device as specified in the Intel High Definition Audio specification and follow the additional UAA design guidelines presented in this paper and the other UAA-related design papers listed at the end of this paper. Certain features of the HD Audio specification are not required for UAA compliance. For more information, see "UAA Design Guidelines for HD Audio Devices" later in this paper.

The HD Audio architecture provides a uniform programming interface for digital audio controllers and codecs. Today's audio codecs usually conform to the AC'97 industry standard; digital audio controllers usually connect to one or more AC'97 codecs through another industry standard called AC-Link. These standards help ensure that codecs and links are implemented consistently, but they do not define the interface to the digital audio controller. Although vendor solutions might be very similar for different system-integrated AC'97 digital audio controllers, each AC'97 solution requires a separate driver. The HD Audio architecture is intended to eliminate the need for solution-specific drivers.

Figure 3 shows a diagram of the UAA driver architecture in Windows Vista for HD Audio devices. Microsoft provides the UAA class driver and HD Audio bus driver that appear in the figure. The modem driver is provided by the hardware vendor for the modem codec.

Figure 3. Windows Vista UAA Driver Architecture for HD Audio Devices

The UAA class driver provides the streaming interface to the operating system audio stack above the driver (not shown in Figure 3).

At device initialization, the operating system assigns an audio function group in a codec to the UAA class driver. The class driver sends codec commands through the bus driver to query the function group for audio jacks (or fixed, internal connections) that are connected to peripherals such as speakers and microphones. Each individual audio device in the function group owns one or more jacks. For devices with more than one jack, the class driver combines the subdevices that service the jacks to form aggregate devices to expose to audio applications. An example of an aggregate device is a group of three stereo jacks that play 5.1-channel audio through a set of six speakers. For each device that it identifies, the class driver registers the corresponding device interface with the operating system.

The HD Audio bus driver directly accesses the HD Audio bus controller device. The bus driver provides an interface through which the UAA class driver or modem driver reserves and programs the direct memory access (DMA) engines, and sends commands to the codecs. The HD Audio bus driver handles all interrupts, Plug and Play notifications, and power-management events on behalf of the audio and modem codecs on the HD Audio Link.

The HD Audio bus controller contains the DMA engines and command buffers that function drivers use to transfer commands and data to codecs on the HD Audio Link. The codecs can be either audio or modem codecs, and they can be connected to jacks or to internal devices such as mobile PC speakers.

In addition to the version of the HD Audio bus driver that runs in Windows Vista, Microsoft has developed a version of the HD Audio bus driver for systems running Windows Server 2003, Windows XP with SP1 and later, and Windows 2000 Professional with SP4.

If an HD Audio device implements advanced features that are not supported by the corresponding UAA class driver, the hardware vendor can supply a function driver to enable those features. The custom hardware features that the vendor-supplied driver controls must be implemented in a way that does not prevent the device from operating as a fully UAA-compatible device under control of the UAA class driver.

For related design guidelines, see "UAA Design Guidelines for HD Audio Devices" later in this paper.

# UAA Design Guidelines for USB Audio Devices

The UAA class driver for USB audio devices, usbaudio.sys, ships with Windows Vista and earlier versions of Windows. This driver supports a subset of the hardware features that are described in the USB audio specifications.

To ensure compatibility with usbaudio.sys, hardware vendors must design their USB audio devices as described in this paper and must comply with the following USB audio specifications:

- Universal Serial Bus Device Class Definition for Audio Devices, Release 1.0

- Universal Serial Bus Device Class Definition for Audio Data Formats, Release 1.0

- Universal Serial Bus Device Class Definition for Terminal Types, Release 1.0

- Universal Serial Bus Device Class Definition for MIDI Devices, Release 1.0

For availability of all public USB specifications, see the references at the end of this paper.

Hardware vendors must also follow the design guidelines in this paper for streaming data, isochronous endpoint types, and topology. Hardware vendors whose USB audio devices follow these design guidelines can use the Microsoft usbaudio.sys driver to control their devices.

## Streaming Data

The Universal Serial Bus Device Class Definition for Audio Data Formats defines four categories of data types for USB audio devices: Type I, Type II, Type III, and MIDI.

- Type I consists of uncompressed pulse-code-modulation (PCM)-based formats. These formats are all fully supported by all versions of the usbaudio.sys driver, with the exception of signed 8-bit PCM, for which there is no corresponding Windows format.

- Type II consists of compressed formats. In Windows Server 2003 and earlier versions of Windows, including Windows Me and Windows 98, the usbaudio.sys driver supports the AC-3 Type II format.

In Windows Vista, usbaudio.sys does not support any Type II formats. No USB audio hardware for the PC uses this format. For this reason, no later versions of Windows support it.

- Type III formats are based on the IEC 60958 and IEC 61937 formats for packaging data into what is effectively a PCM-like stream. In Windows Server 2003 and earlier versions of Windows, including Windows Me and Windows 98, the usbaudio.sys driver provides no support for Type III formats.

In Windows Vista, usbaudio.sys fully implements Type III streaming but supports only the AC-3 and MP3 data-format specifiers, as defined in the Universal Serial Bus Device Class Definition for Audio Data Formats, Release 1.0. In addition, the driver might support WMA and WMA Pro formats if the necessary definitions are available in time for the release of Windows Vista.

- MIDI format communication is performed through a bulk pipe, in contrast with other formats that take advantage of the isochronous capabilities of the USB bus. In Windows 2000 and Windows 98, the usbaudio.sys driver provides no support for MIDI formats.

In Windows XP and Windows Me, usbaudio.sys partially implements the MIDI specification, but lacks support for MIDI elements, which often leads to broken topologies and sometimes causes system crashes.

In Windows Vista, full MIDI support, as defined in the Universal Serial Bus Device Class Definition for MIDI Devices, Release 1.0, is planned for usbaudio.sys.

## Isochronous Endpoint Types

The USB specification defines three types of isochronous endpoints: adaptive, synchronous, and asynchronous.

In Windows Server 2003 and earlier versions of Windows, including Windows Me and Windows 98, usbaudio.sys supports the adaptive and synchronous endpoints, but does not implement the asynchronous endpoint correctly. Full support for asynchronous endpoints in usbaudio.sys is planned for Windows Vista.

For device compatibility with earlier versions of Windows, vendors can choose to continue to use adaptive endpoints. Keep in mind that the use of a lock delay for adaptive endpoints adds latency to the start of a stream.

## Topology

A USB audio device describes its capabilities to the system through a series of device descriptors. These descriptors are defined in the USB audio specifications. Device descriptors describe the internal topology, control capabilities, and data formats for the device.

### Interface Descriptors

USB devices are described by a series of USB-defined interfaces. The USB bus driver, usbd.sys, groups together the associated interfaces for each audio function and creates a single physical device object (PDO) for each group. Of these interfaces, the streaming interfaces and their alternate interfaces define the AVStream pins for the driver. Each streaming interface from the device results in a single pin. Each alternate interface for a streaming interface results in a separate data range for that pin. For more information, search the WDK documentation for "audio data ranges."

### Zero-Bandwidth Interface

At least one of the alternate interfaces for each interface must be a zero-bandwidth interface. The USB bus driver uses the zero-bandwidth interface to free bus bandwidth when a pin is not in use. (In kernel-streaming terminology, a pin represents a data path through which a stream enters or leaves a device. For more information, see the WDK documentation.) The USB bus driver fails enumeration for any device that does not implement a zero-bandwidth alternate setting for each interface.

# UAA Design Guidelines for IEEE 1394 AV/C Audio Devices

The UAA class driver for IEEE 1394 AV/C audio devices, avcaudio.sys, ships with the Windows Vista, Windows Server 2003, and Windows XP. This driver supports a subset of the hardware features described in the IEEE 1394 AV/C specifications. To ensure compatibility with avcaudio.sys, IEEE 1394 AV/C audio devices must implement many functions that are designated as optional in the IEEE 1394 and AV/C specifications, and they must interpret the IEEE 1394 AV/C audio specifications in the way that this paper describes.

For compatibility with UAA, hardware vendors must design their IEEE 1394 AV/C audio devices to comply with the appropriate sections of the specifications that are listed in Table 1.

Table 1. Relevant IEEE 1394 specifications

| Specification | Purpose |
| --- | --- |
| AV/C Audio Subunit Specification, Revision 1.0 | Sets the framework for the majority of the control and status parameters necessary to control an audio subunit. |
| IEC 61883-1 and IEC 61883-6 (IEC 60958) | Describes the data transfer layer of the device. |
| AV/C Digital Interface Command Set General Specification, Revision 3.0 | Specifies basic AV/C commands and status requests. Some commands that are optional in the specification are required for Windows compatibility, as described in this paper. |
| AV/C Stream Format Information Specification, Revision 1.0 | Supports the discovery of audio stream formats. |
| AV/C Connection and Compatibility Management Specification, Revision 1.0 | Establishes connections to the PC and any other device (optional). |
| 1394 Trade Association Document 2001024: Audio and Music Data Transmission Protocol, Revision 2.1 | Extends the IEC 61883-6 specification to allow for multichannel formats, extended lists of sample rates, and additional data formats to support the capabilities of newer devices. |

For availability of IEEE 1394 audio specifications, see the references at the end of this paper.

Hardware vendors must also follow the design guidelines in this paper for AV/C device capabilities, AV/C streaming formats, audio subunit controls, control notifications, and AV/C audio driver defaults. Hardware vendors whose IEEE 1394 audio devices follow these design guidelines can use the Microsoft avcaudio.sys driver to control their devices.

## AV/C Device Capabilities

This section describes the information that the avcaudio.sys driver must receive from an AV/C audio device during initialization of the driver.

### General AV/C Command Set Requests

After the IEC 61883 capabilities of the device have been discovered, the avcaudio.sys driver starts initialization. During initialization, the driver sends the following basic AV/C requests, as defined in the AV/C Digital Interface Command Set General Specification, to determine the capabilities of the device as a whole:

SUBUNIT INFO

PLUG INFO

CONNECTIONS

CONNECT

POWER

INPUT PLUG SIGNAL FORMAT

OUTPUT PLUG SIGNAL FORMAT

RESERVE

Unless the device implements connection management through the AV/C Connection and Compatibility Management Specification (CCM), it should support at least one CONNECTIONS or CONNECT request to allow the driver to determine which internal connections are possible. If the device employs CCM, the CONNECT and CONNECTIONS commands might not apply. For more information, see "Connection Management Determination" later in this paper.

The responses or lack of response to these requests help the avcaudio.sys driver to determine the following:

- The connections to the audio subunit.

- Whether other subunits share Serial Bus plugs and external plugs.

- Minimal information about which data formats are accepted.

If the device advertises any analog-to-digital (A/D) or digital-to-analog (D/A) conversion (for a device such as a microphone or a speaker), the driver expects at least one Serial Bus plug and one external plug, each flowing in the appropriate direction.

Note: The AV/C specification requires devices to support GENERAL INQUIRY responses for all AV/C functions, even for functions that the device does not support. The AV/C specification also requires devices to support SPECIFIC INQUIRY request responses for all supported functions for which this response is appropriate.

### Connection Management Determination

The avcaudio.sys driver is capable of two types of connection management:

- Through the basic AV/C commands that were described earlier in this paper.

- Through the techniques that are described in the AV/C Connection and Compatibility Management Specification.

During initialization, the driver sends several status requests to determine the device's ability to perform connection-and-compatibility management (CCM) requests. The driver uses this information to find the Serial Bus plugs that the subunit can connect to and to determine the formats that the plugs support. The driver also uses this information during setup for audio recording or playback to connect the device to the computer's Serial Bus plugs.

If CCM is not available on the device, the driver uses a combination of general AV/C requests and connection-management protocol (CMP) register manipulation for connection management.

### Serial Plug Data Format Capabilities

During initialization, the avcaudio.sys driver determines whether the device responds to requests as defined in the AV/C Stream Format Information Specification. If so, the driver determines the data format capabilities of the device and advertises those format capabilities to the system through pin data range structures that are associated with the subunit plugs. For more information, search the WDK documentation for "audio data ranges."

The pins and associated formats that the driver advertises to the system are only the representation of the subunit plugs, so the avcaudio.sys driver must be able to determine the unit plugs that each of the subunit plugs is allowed to connect to. If the driver cannot make definitive associations between unit and subunit plugs based on the information from the device, the driver makes associations based on defaults. For more information, see "AV/C Audio Driver Defaults" later in this paper.

### The AV/C Audio Subunit

The AV/C audio subunit provides crucial information about the inner topology of the device. The AV/C Audio Subunit Specification (and the avcaudio.sys driver) requires the device to provide an audio device descriptor that provides the topology information.

At a minimum, the device's topology descriptor should complete the topology by telling the driver that the subunit destination plugs connect directly to the subunit source plugs. To expand this topology, the different types of function blocks defined by the specification can provide clients with access control parameters for attributes such as volume-level controls, mute controls, mixers, and selectors. If the topology incorporates function blocks, it is absolutely necessary for the topology to begin and end at a subunit plug, in order to fully define the path through the device. During device enumeration, the avcaudio.sys driver rejects any devices that are found to have incomplete topologies.

When creating an audio subunit descriptor, remember that:

- Descriptors, like the data transmitted over the IEEE 1394 interface, are in big-endian format. In bit maps for items such as controls, the first bit is the most significant bit (MSB).

- The avcaudio.sys driver currently supports all feature function block controls except panning. The device should employ separate volume-level controls instead of a panning control.

- The avcaudio.sys driver separates feature function block capabilities into sequential topology nodes.

- Selector function blocks are parsed into multiplexer (MUX) topology nodes. Ensure that the channel configurations and channel counts of the input sources to the selector are identical to those of the output, because the Selector function block cannot convert the inputs internally to its output configuration.

## AV/C Streaming Formats

The AV/C Stream Format Information Specification defines how the driver can discover the data formats that the device supports. The driver uses this information to construct the pin data range structures in the device capabilities that it reports to the system.

In Windows Server 2003 and Windows XP, the AM824 data format, as defined by the IEC 61883 specification, is the only format that the avcaudio.sys driver supports. More specifically, the IEC 60958 form of stereo pulse-coded modulation (PCM) is the only format that Windows XP supports because devices that supported any other format were unavailable at the release of Windows XP.

In Windows Vista, the avcaudio.sys driver supports all of the streaming formats that are supported in Windows XP. In addition, planned support in avcaudio.sys for Windows Vista includes the set of IEC 61937 formats, plus raw PCM and DVD-A-formatted multichannel audio formats. To enable the driver to transmit or receive any of these additional formats, the device must respond to the AV/C Stream Format Information Specification requests. The device must allow the driver to specify "don't care" fields at every level of the request hierarchy so that the driver can determine formats systematically.

## Audio Subunit Controls

The avcaudio.sys driver is based on the Windows AVStream driver architecture. In accordance with this architecture, avcaudio.sys provides a set of property handlers through which clients can manipulate hardware controls. These handlers are driver routines that handle I/O-control (IOCTL) requests for manipulating audio controls such as volume level, mute, tone, and graphic equalization. The parameters in these requests are specified in a bus-independent manner that is uniform across all audio devices. For more information about AVStream, see "AVStream Overview" in the WDK documentation.

The device reports the parameter ranges for the controls. Clients should send parameter values in these ranges. For any controls that restrict parameter values to ranges, the controls should, at minimum, implement the MINIMUM, MAXIMUM, RESOLUTION, and CURRENT attributes to inform the system of the parameter ranges. Function blocks that are supported by the avcaudio.sys driver include all feature function block capabilities, with the exception of panning, Selector function blocks, and all processing function blocks except the generic processing function block.

## Control Notifications

Currently, no IEEE 1394 specification exists for human interface devices (HIDs). To date, the specification that comes closest is the AV/C Panel Subunit Specification, which Windows does not support. Thus, the avcaudio.sys driver must resort to periodically polling volatile controls to detect changes in their settings.

For the benefit of applications that might potentially damage equipment, Microsoft recommends that an IEEE 1394 AV/C audio device implement control notifications to prevent audio parameters such as volume levels from reaching unexpected levels.

## AV/C Audio Driver Defaults

For IEEE 1394 AV/C audio devices that do not fully describe themselves to the avcaudio.sys driver, the driver applies the following defaults:

- If the device does not implement the AV/C Stream Format Information Specification, the driver sends an INPUT/OUTPUT PLUG SIGNAL FORMAT request to get as much information as possible about data rates. The driver assumes that the device supports only IEC 60958 stereo PCM data.

- If the device does not support either the AV/C Stream Format Information Specification or the PLUG SIGNAL FORMAT requests, the driver assumes that the device supports only 48-kHz IEC 60958 PCM.

- If the device supports only a single (audio) subunit and the subunit plugs are not permanently connected (as determined through AV/C CONNECTIONS, CONNECT, and possibly CCM), and if there is only a single Serial Bus plug for the related data-flow direction, the driver assumes that the connection is permanent and uses the data formats of the associated unit plug.

- To a pin that represents a Serial Bus plug or external plug, the driver assigns a globally unique identifier (GUID) that identifies the plug as a "Microphone" or "Speaker" because there is currently no more precise way to label a Serial Bus plug or external plug. A client can supply a more precise label if it knows what kind of audio function the plug is associated with.

# UAA Design Guidelines for HD Audio Devices

Windows Vista provides a UAA function class driver, hdaudio.sys, for HD Audio codec devices. Windows Vista also provides an HD Audio bus driver, hdaudbus.sys, which controls an HD Audio bus controller device and works in conjunction with hdaudio.sys. Microsoft plans to make versions of these drivers available for use with Windows Server 2003 and earlier versions of Windows.

Hardware vendors can use the UAA function class driver for HD Audio and the HD Audio bus driver to control codec and bus controller devices that conform to the Intel High Definition Audio specification and that follow additional UAA design guidelines from Microsoft.

To ensure compatibility with the UAA function class driver for HD Audio and the HD Audio bus driver, hardware vendors must design their HD Audio devices according to the design guidelines in this paper and in the following documents from Microsoft:

- Plug and Play Guidelines for High Definition Audio Devices

- Pin Configuration Guidelines for High Definition Audio Devices

For availability of these documents, see the references at the end of this paper.

If the HD Audio device implements advanced features that are not supported by the corresponding UAA class driver, the hardware vendor can supply a function driver to enable those features. The vendor-supplied driver must be compatible with the HD Audio driver model.

The remainder of this section presents design guidelines for UAA-compliant HD Audio devices, HD Audio bus controllers, HD Audio codecs, and PC systems with HD audio solutions. In the following section, there are numerous recommendations, and for the purpose of this document and these HW design guidelines they are worded as guidelines. Some of these recommendations are actually Windows Logo Program requirements for Windows Vista, so it is important to review the Windows Vista logo requirements for hardware to ensure your implementation will be compliant with those requirements.

## Design Guidelines for HD Audio Devices

The following design guidelines apply to HD Audio devices (bus controllers and codecs) that are controlled by the HD Audio-specific UAA class driver.

### General Recommendations for HD Audio Devices

A UAA-compliant audio device should have enough hardware resources—DMA channels, analog-to-digital and digital-to-analog converters, and physical connectors—to support two simultaneous uses: real-time communication (RTC) plus one additional instance of media consumption. For example, the user should be able to simultaneously make an RTC call and play back audio from another media source through separate audio jacks on the same PC.

Microsoft recommends the following:

- The UAA audio device should support two independent input streams to guarantee that sufficient capture resources are available for RTC, even if the system is doing other audio recording at the same time.

- The UAA audio device should support two independent stereo or multichannel output streams to guarantee that resources are available for RTC, even if the system is playing other audio content at the same time.

- The UAA audio device should not place arbitrary limits on the number of channels per stream apart from the practical limit of bus bandwidth capacity. For example, for HD Audio devices, the UAA function class driver for HD Audio and the HD Audio bus driver allocate link bandwidth in a way that allows any single stream to acquire as much of the bandwidth on a serial data in (SDI) line or set of serial data out (SDO) lines as is available (that is, currently unallocated). These drivers do not hold any unallocated bandwidth in reserve in anticipation of the potential needs of other streams.

- A desktop system that contains a UAA-compliant audio device might provide front-panel audio jacks for easy access. If so, these jacks should have their own analog-to-digital and digital-to-analog conversion (ADC and DAC) resources and be able to operate independently of the rear-panel audio jacks. The front- and back-panel jacks do not each require a separate HD Audio codec (although that is preferred for fidelity reasons). However, the ADC and DAC resources that the audio codec dedicates to the front panel should not be used for any other purpose at any time. Keeping these resources available at all times ensures immediate access to always-on, real-time communication, regardless of other media consumption that might be in progress on the PC.

- If a UAA-compliant laptop system contains integrated speakers and provides audio jacks, these jacks should have their own analog-to-digital and digital-to-analog conversion resources and be able to operate independently of the integrated speakers and the microphone. Keeping these resources available at all times ensures immediate access to always-on, real-time communication, regardless of other media consumption that might be in progress on the PC.

- The UAA audio device or the system should assign default behaviors to all audio jacks, and the system's default behavior for each jack should be indicated by color coding. The device designer has the option of allowing a user to override the default behavior of one or more jacks. For example, if the default behavior of a set of jacks is to support 5.1-channel surround speakers and stereo headphones, the driver might allow the user to configure the default headphones jack to drive an additional pair of speakers for 7.1-channel audio. In Windows Vista, the user configures the jacks through a user-interface (UI) program. For Windows Server 2003 and earlier versions of Windows, hardware vendors must write their own UI programs for this purpose.

- The UAA audio device and the system should support jack-presence detection on all system analog output 1/8” jacks and optionally on RCA jacks, as described in the Intel High Definition Audio specification. The device and the system should be able to detect the presence of a plug for an audio peripheral in a jack and inform the operating system when the peripheral is plugged into or removed from the jack. This should be done in a fashion compatible with the HD Audio specification. Using solutions acceptable for AC-97 implementations is not allowed.

Microsoft also recommends that UAA-compliant audio devices follow these fidelity design guidelines:

- The UAA-compliant HD Audio codec should support a sample rate of 44.1 kHz and 48 kHz or higher and audio sample precision of at least 16 bits. Providing a higher sample rate or greater sample precision is optional.

- The UAA-compliant HD Audio bus controller should satisfy all the support requirements for sample rates, sample bit depths, and number of channels per stream that are set forth in the Intel High Definition Audio specification.

- For an audio output, the minimum dynamic range and total harmonic distortion plus noise of a PC system supporting a UAA-compliant audio device should meet or exceed the audio fidelity requirements for the Windows Logo Program.

- For an audio input, the minimum dynamic range and total harmonic distortion plus noise of a PC system supporting a UAA-compliant audio device should meet or exceed the audio fidelity requirements for the Windows Logo Program.

### Recommendations for HD Audio Devices to Support Digital Media Experiences

To support digital media experiences enabled by Windows Media Center Edition and premium versions of Windows Vista, a UAA-compliant audio device should have enough hardware resources—DMA channels, ADCs, DACs, and physical connectors—to support three simultaneous uses: RTC plus two other types of media consumption by independent users. For example, two users in different rooms should be able to simultaneously use the device as follows:

- Each user plays back audio from a media source. Each user uses a different media source and plays the audio through a separate set of audio jacks on the same PC.

- One user consumes a TV or other broadcast signal that is unrelated to the other media source.

- Either user makes an RTC call.

The detailed design guidelines for supporting the digital media experiences are as follows:

- The UAA audio device should support two independent input streams so that RTC and audio recording can occur separately and at the same time.

- The UAA audio device should support three independent stereo or multichannel output streams so that the following can occur simultaneously: a phone conversation by user A, media consumption by user B (or A), and different media consumption by user C (in another location within the house).

- The UAA audio device should not place arbitrary limits on the number of channels per stream apart from the practical limit of bus bandwidth capacity. For example, for HD Audio devices, the UAA function class driver for HD Audio and the HD Audio bus driver allocate link bandwidth in a way that allows any single stream to acquire as much of the bandwidth on a serial data in (SDI) line or set of serial data out (SDO) lines as is available (that is, currently unallocated). These drivers do not hold any unallocated bandwidth in reserve in anticipation of the potential needs of other streams.

- The UAA audio device and the system should have a digital output jack (for example, an S/PDIF port) through which any one of the active output streams can be transmitted in digital pass-through mode. The digital output should support all the following audio encoding formats: AC-3, WMA Pro, Digital Theatre Systems (DTS), and stereo PCM. For non-PCM streams, the audio driver can rely on the audio application to add padding to the compressed synchronization frames so that the digital output jack can transmit them in pass-through mode as a series of fixed-size blocks.

- The UAA-compliant HD Audio device should have sufficient hardware resources to play a WMA Pro-encoded 5.1-channel audio stream that the operating system has decoded into a 6-channel PCM stream and sent to the HD Audio device to be rendered. The format for a PCM stream is specified by a WAVEFORMATEXTENSIBLE structure. The device and the system should provide three stereo analog jacks through which it outputs the signals needed to play the stream through five surround speakers and a subwoofer.

- A desktop system that contains a UAA-compliant audio device might provide front-panel audio jacks for easy access. If so, these jacks should have their own analog-to-digital and digital-to-analog conversion (ADC and DAC) resources and be able to operate independently of the rear-panel audio jacks. The front- and back-panel jacks do not each require a separate HD Audio codec (although that is preferred for fidelity reasons). However, the ADC and DAC resources that the audio codec dedicates to the front panel should not be used for any other purpose at any time. Keeping these resources available at all times ensures immediate access to always-on real-time communication (RTC) regardless of other media consumption that might be in progress on the PC.

- The UAA audio device or the system should assign default behaviors to all audio jacks, and the system's default behavior for each jack should be indicated by color coding. The device designer has the option of allowing a user to override the default behavior of one or more jacks. For example, if the default behavior of a set of jacks is to support 5.1-channel surround speakers and stereo headphones, the driver might allow the user to configure the default headphones jack to drive an additional pair of speakers for 7.1-channel audio. In Windows Vista, the user configures the jacks through a user-interface (UI) program. For Windows Server 2003 and earlier versions of Windows, hardware vendors must write their own UI programs for this purpose.

- The UAA audio device and the system should support jack-presence detection on all system analog output 1/8” jacks and optionally on RCA jacks. The audio device and the system should be able to detect when an audio peripheral is inserted into or removed from a jack and alert the operating system to the change in the device's status. This should be done in a fashion compatible with the HD Audio spec. Using solutions acceptable for AC-97 implementations is not allowed.

Under these recommended design guidelines, a UAA-compliant audio devices should meet the following fidelity design guidelines:

- The UAA audio device should support 24-bit or better audio sample precision, and the native sample rate of the device should be 96 kHz or higher.

- For an audio output, the minimum dynamic range and total harmonic distortion plus noise of a PC system supporting a UAA-compliant audio device should exceed the audio fidelity requirements for the Windows Logo Program.

- For an audio input, the minimum dynamic range and total harmonic distortion plus noise of a PC system supporting a UAA-compliant audio device should exceed the audio fidelity requirements for the Windows Logo Program.

- The device should be able to play multichannel audio streams with at least six channels. This capability is needed to support 5.1 channel playback.

## Design Guidelines for HD Audio Bus Controllers

To be compatible with the Microsoft bus driver for HD Audio, an HD Audio bus controller device should comply with the Intel High Definition Audio specification. In addition, the device should follow the UAA design guidelines in this paper.

To be UAA compliant, a hardware controller should implement the following changes to the Intel High Definition Audio specification:

- A UAA device should support 256 entries each for the command output ring buffer (CORB) and the response input ring buffer (RIRB).

- The controller should have at least three separate DMA engines for input streams and at least four separate DMA engines for output streams. These DMA engines are not required to serve any particular roles.

The following examples show how these DMA engines might be used:

- Input

DMA1: RTC (headset)

DMA2: Audio recording (separate from RTC)

DMA3: Modem

- Output

DMA1: RTC (headset)

DMA2: Multichannel audio playback. The HD Audio codec listens to this data stream for both digital and analog output at the same time.

DMA3: Modem

DMA4: Reserved for future use. For example, this DMA engine might be used for High-Definition Multimedia Interface (HDMI) when it is available.

For implementation details, see the Intel High Definition Audio Specification listed at the end of this paper.

- To keep stream latency small, a DMA engine in an HD Audio bus controller implementation should avoid adding a significant hardware-buffering delay to an audio data stream. Setting an upper limit on the buffering delay requires restricting the DMA engine's internal first-in, first-out (FIFO) buffer to a maximum size in bytes that varies according to the stream format. For each format, the hardware designer should limit the FIFO to a size that keeps the latency small while providing glitch-free audio.

In addition, it is not necessary for a UAA-compliant HD Audio bus controller to implement several features of the Intel High Definition Audio specification:

- The bus driver does not use the DMA Position Lower Base Address (DPLBASE) and DMA Position Upper Base Address (DPUBASE) registers (at offsets 0x70 and 0x74).

- The bus driver does not use the Immediate Command Output, Immediate Response Input, and Immediate Command Status registers (at offsets 0x60, 0x64, and 0x68).

- The bus driver does not use the flush control bit in the Global Control register (at offset 0x08).

An HD Audio bus controller design can omit these features and still be fully compatible with the HD Audio bus driver. However, a hardware vendor should consider whether these features might be necessary for compatibility with other device-specific software. For example, a BIOS routine might make use of the immediate command, response, and status registers.

For UAA version 1.0, the HD Audio hardware version must be 1.0. (The VMAJ and VMIN registers must specify a major version number of 0x01 and a minor version number of 0x00.)

If an audio adapter card contains both an HD Audio bus controller device and a digital signal processor (DSP), the DSP should be exposed as an independent PCI function that is separate from the HD Audio bus controller.

## Design Guidelines for HD Audio Codecs

To be compatible with the Microsoft UAA class driver for HD Audio devices, an HD Audio codec device should comply with the Intel High Definition Audio specification. In addition, the device should follow the UAA design guidelines in this paper for HD Audio codecs and codec topologies and the other papers listed in the references at the end of this paper.

### General Guidelines for HD Audio Codecs

To be UAA compliant, an HD Audio codec should implement the following features, which are not necessarily required by the Intel High Definition Audio specification:

- A UAA-compliant codec should derive the clock that it uses to sample its analog audio inputs from the clock signal that is broadcast on the HD Audio Link. This guideline does not apply to digital inputs. In the case of digital input, the clock is set by the user when running the Microsoft class driver for HD Audio.

- Speaker compensation is the only valid scenario for processing of an audio stream by a codec, and then it is valid only if the speakers are hardwired to the pin complex that contains the processing node (for example, integrated laptop speakers).

- When all of an HD Audio codec's widgets are configured in the benign processing state, the codec performs no nonlinear or time-variant processing on the audio streams that pass through it. For instance, speaker compensation and equalization (EQ) processing are compatible with the benign state, but 3-D spatialization processing is not.

- To ensure predictable operation, software should be able to force the transition of all processing nodes to the benign processing state from whatever state the device might be in. In the benign state, the codec should function according to UAA standard design guidelines.

- An HD Audio codec should be accessible only through the HD Audio bus controller. The codec should not make any of its registers or other hardware mechanisms accessible through either memory or I/O address space.

- Modem and audio functionality should not be combined. Although the modem and audio functions can reside on the same piece of silicon, they should operate as separate and independent devices and should not share any software or hardware resources (for example, ADCs or DACs).

- When the HD Audio Link is in the running state (that is, the HD Audio bus controller is in the full-power state, D0), HD Audio-compliant UAA codecs should respond to commands even when they are in a powered-down device power-management state. In effect, the digital section of the codec should remain powered up even when the rest of the codec is in a power-saving state.

- Codecs should respond to a verb even if it is addressed to a non-existent widget within the codec or if the verb itself is invalid.

- Function group nodes should have node IDs in the range 0 to 127. This restriction does not apply to node IDs for widget nodes.

- In a system with one or more HD Audio codecs, either the HD Audio codec hardware or system BIOS should initialize the Configuration Default register for each codec pin widget in accordance with Microsoft guidelines so that the topology parser in the UAA function class driver for HD Audio can create a functional device topology for the codec. It is preferable to do this in the system BIOS because the system implementer is more likely to know which audio inputs and outputs should be exposed. The topology should not misrepresent the hardware capabilities, and the Configuration Default registers should not be null (all zeros). For more information, see Pin Configuration Guidelines for High Definition Audio Devices listed in the references at the end of this paper.

- A function group in an HD Audio codec should expose a nonzero subsystem ID. The BIOS is responsible for overwriting the subsystem ID if necessary. If the BIOS is unable to program the subsystem ID or if it does so incorrectly, the hardware should supply a default, vendor-specific subsystem ID. For more information, see Plug and Play Guidelines for High Definition Audio Devices listed at the end of this paper.

### Guidelines for HD Audio Codec Topologies

In addition to compliance with the Intel High Definition Audio specification, HD Audio codecs should follow these design guidelines for compatibility with the topology parsing algorithm in the Microsoft function driver for HD Audio:

- Each pin widget's Configuration Default register should contain valid information.

- On a recording path, an ADC widget should be the last widget on the audio data path before the connection to the HD Audio Link.

- On a playback path, a DAC widget should be the first widget on the audio data path following the connection to the HD Audio Link.

- The HD Audio codec should implement HD Audio-compliant jack-presence detection to support the audio set-up wizard in Windows Vista.

- All audio-related ADCs in a codec should have the same bit depth, number of channels, and sample-format capabilities.

- All audio-related DACs in a codec should have the same bit depth, number of channels, and sample-format capabilities.

- The Microsoft parsing algorithm does not support vendor-defined widgets. If the algorithm encounters vendor-defined widgets, it safely ignores them.

- The Microsoft parsing algorithm ignores any widget whose Configuration Default register is null (all zeros).

For more information, see Pin Configuration Guidelines for High Definition Audio Devices listed in the references at the end of this paper.

## Design Guidelines for PC Systems with HD Audio Solutions

To be compatible with the HD Audio class driver framework in Windows, a PC system should not circumvent proper operation of an HD Audio-based audio solution, and it should adhere to all system implementation guidelines as described in the Intel High Definition Audio specification and papers listed in "HD Audio Resources" at the end of this paper.

In addition, the system should follow these UAA design guidelines:

- In the case where the default settings in the BIOS and codec hardware are not the same, the BIOS should restore the hardware registers containing the subsystem ID and pin-widget configurations to their default settings when resuming operation after hibernation. Although the function driver might subsequently overwrite the pin-widget configuration registers with the settings that it saved before the device entered hibernation, the BIOS should load the default configuration settings as a backup measure to avoid any possibility of the device resuming operation in an undefined state.

- If waking up is possible and enabled on an HD Audio codec, the system should ensure that the codec receives power while the system is in a power-saving state so that the codec can wake up if a status change event occurs. For example, while the system is in a low-power state, a modem codec should have enough power to respond when the phone rings to indicate an incoming call.

- The HD Audio codec hardware or system BIOS should initialize the Configuration Default register for each codec pin widget so that the UAA class driver topology parser can create a functional device topology for the codec. The topology should not misrepresent the hardware capabilities, and the Configuration Default registers should not be null (all zeros). For more information about how system manufacturers should program these default values using the system BIOS, see Pin Configuration Guidelines for High Definition Audio Devices listed at the end of this paper.

- Power-management technologies such as CPU throttling should not cause hardware underruns during power-state transitions.

# UAA and the Windows Logo Program

To be UAA compliant, an audio device must conform to the relevant industry specifications supported by the UAA initiative, meet all applicable requirements for the Windows Logo Program, and follow the UAA design guidelines in this paper and the other relevant Microsoft design guidelines.

For Windows Vista and later versions of Windows, the Windows Vista logo program requires every PC system to ship with a UAA-compliant hardware solution, which might consist of a UAA-compliant USB audio device, a UAA-compliant IEEE 1394 AV/C audio device, or an HD Audio device.

Microsoft recommends UAA compliance for USB audio, IEEE 1394 AV/C audio, and Intel HD Audio devices in systems running Windows Server 2003, Windows XP, and Windows 2000.

# References

Call to Action

- For system manufacturers:

- Include UAA compliance in your product plans.

- Start testing your system with the appropriate Microsoft UAA class driver as soon as possible. Systems must pass logo qualification tests with the Microsoft Vista UAA class driver, not with third-party drivers

- Encourage audio device vendors to build UAA-compliant devices.

- For audio device manufacturers:

- Include UAA compliance in your product plans.

- Start testing your device with the appropriate Microsoft UAA class driver as soon as possible. Audio device solutions must pass logo qualification tests with the Microsoft Vista UAA class driver in addition to the UAA audio driver included with the submission package.

- Contact Microsoft for information about designing UAA-compliant devices and for test tools and pre-release versions of drivers for validating systems and devices..

If you have questions about UAA, please send e-mail to uaa@microsoft.com.

UAA Compatibility Information

- For information about UAA design guidelines for USB audio, IEEE 1394 AV/C, and Intel HD Audio devices on PCs running Windows operating systems, send e-mail to Microsoft at uaa@microsoft.com.

- For information about achieving low-latency audio streaming that is nearly glitch-free, see the white paper titled A Wave Port Driver for Real-Time Audio Streaming at http://www.microsoft.com/whdc/device/audio/.

USB Audio Resources

- All public USB specifications are available from the USB Implementers Forum Web site at http://www.usb.org. The following specifications apply for UAA:

- Universal Serial Bus Device Class Definition for Audio Devices, Release 1.0

- Universal Serial Bus Device Class Definition for Audio Data Formats, Release 1.0

- Universal Serial Bus Device Class Definition for Terminal Types, Release 1.0

- Universal Serial Bus Device Class Definition for MIDI Devices, Release 1.0

IEEE 1394 Audio Resources

- IEEE 1394 AV/C audio specifications are available to members of the 1394 Trade Association at http://www.1394ta.org. The following specifications apply for UAA:

- AV/C Audio Subunit Specification 1.0

- IEEE 61883-1 and IEEE 61883-6 Protocol Specifications (IEC 60958)

- AV/C Digital Interface Command Set General Specification 3.0

- AV/C Stream Format Information Specification 1.0

- AV/C Connection and Compatibility Management Specification 1.0

- Formats for transmitting multichannel PCM audio data to DVD-Audio devices are described in Guideline of Transmission and Control for DVD-Video/Audio through IEEE1394 Bus, Version 1.0, at http://www.dvdforum.org.

HD Audio Resources

- For information about the Intel High Definition Audio specification, see the Intel HD Audio Web site at http://www.intel.com/standards/hdaudio/.

- To ensure compatibility with the UAA function class driver for HD Audio and the HD Audio bus driver, hardware vendors should design their HD Audio devices according to the design guidelines in this paper and in the following documents at http://www.microsoft.com/whdc/device/audio:

- Plug and Play Guidelines for High Definition Audio Devices

- Pin Configuration Guidelines for High Definition Audio Devices

- For access to audio design compliance test tools and pre-release versions of the Microsoft class driver for HD Audio to use through all system design phases, please email uaa@microsoft.com

PCI-SIG Vendors IDs

- Manufacturers of HD Audio devices can obtain vendor IDs from the PCI Special Interest Group at http://www.pcisig.com.

Additional Resources

Windows Driver Kit 
http://www.microsoft.com/whdc/driver/WDK/aboutWDK.mspx

Windows Logo Program 
http://www.microsoft.com/whdc/winlogo/default.mspx