A WINDOWS® RALLY™ WHITE PAPER

Web Services on Devices

December 13, 2006

Abstract

New opportunities for using network-connected devices continue to expand rapidly across consumer, enterprise, and vertical device markets. This expansion is due to device technology advancements including faster and smaller processors, increased computational power, shrinking memory costs, and greater network connectivity.

IP network connectivity provides a platform for rich interoperability experiences between devices, PCs, and Internet services. However, to achieve “USB-like” device growth, networked devices must use technologies that reduce complexity and provide setup, security, control, and cross-network connectivity standards.

Microsoft’s strategy for meeting these challenges is to use Web services. To support Web services on devices, Microsoft has provided native support in  Windows Vista™ for the Devices Profile for Web Services (DPWS) standard. DPWS is an open networking standard that enables industry partners to confidently invest in Web services for devices. DPWS enables network-connected devices to use industry standards–based Web services mechanisms. This provides a common networking solution that supports a wide range of connectivity and messaging options. Together with the other Microsoft Windows® Rally™ technologies, DPWS gives network-connected device manufacturers an extensible architecture that reduces development costs and supports interoperability scenarios that have traditionally been beyond the reach of individual device types.

Device manufacturers that adopt DPWS and Windows Rally technologies can experience the following benefits:

- Lower development costs and long-term investment security. DPWS eliminates the need to create and support custom solutions and proprietary protocols by simplifying the development process. DPWS extends the industry-wide Web services framework to resource-constrained devices, thereby reducing investment risk. Devices can be easily incorporated to support Web services scenarios. DPWS is built on standard Internet protocols including Extensible Markup Language (XML) and Simple Object Access Protocol (SOAP).

- Effortless setup experience and fewer product returns. For customers, the setup experience must be as simple as plugging in a USB device. Windows Rally provides technologies including Plug and Play Extensions (PnP-X) for easy discovery, association, and installation. DPWS can be employed with WS-Discovery to support this critical capability. By using these technologies, manufacturers can increase customer satisfaction “out of the box” and decrease product returns.

- Richer experiences and greater product differentiation. From a user's perspective, DPWS and Windows Rally largely eliminate the experiential difference between devices that are directly connected to PCs and those that are virtually connected over a network and Internet. DPWS allows manufacturers to apply more resources to enhance functionality and improve device integration with applications and Internet services. DPWS allows device manufacturers to increase customer value and expand market opportunities for automation and integration.

- Lower support costs. DPWS and Windows Rally technologies enable easy setup and installation, better security, simplified upgrades, and better access to Internet services, computers, and devices. Customer support calls are lowered due to improved self support. DPWS provides better monitoring and event notification of the status of devices and their current operational state.

The paper discusses three scenarios that implement DPWS together with Windows Rally technologies. This paper also introduces DPWS-related concepts and recommends solutions, emphasizing pertinent design issues for consumer electronics, enterprise products, and vertical markets such as healthcare and fitness.

LICENSE NOTICE. Access to, viewing and implementation of the Windows Rally technologies described in this document is granted under the Microsoft Windows Rally Program License Agreement (“License Agreement”). If you want a license from Microsoft to access, view or implement one or more Licensed Technologies, you must complete the designated information in the License Agreement and return a signed copy to Microsoft. The License Agreement is provided at the end of this document. If the License Agreement is not available with this document, you can download a copy from the Windows Rally Web site at http://www.microsoft.com/rally.

Contents

Introduction	4

Windows Rally Technologies	5

DPWS Technology Overview	6

Devices Profile for Web Services	6

Web Services Description Language	9

Web Services on Devices API	9

WSDAPI Code-Generation Tool	10

Device Communication Flow	10

Phase 1: Web Service Device Discovery	10

Phase 2: Web Services Control and Eventing	11

DPWS Implementation	11

Web Services on Devices Scenarios	12

Enterprise Printers Scenario	13

Enterprise Printers Scenario Description	13

How the Enterprise Printers Technology Works	13

How DPWS Exchanges Messages	15

Fitness and Chronic Healthcare Device Scenario	20

Healthcare Device Scenario Description	20

How the Healthcare Device Scenario Technologies Work	21

In-Home Digital Still Camera Scenario	25

In-Home Camera Scenario Description	25

How the In-Home Camera Scenario Technologies Work	25

Summary of Benefits	30

Lower Development Cost and Long-Term Investment Security	30

Effortless Setup Experience and Fewer Product Returns	31

Richer Experiences and Greater Product Differentiation	32

Lower Support Costs	33

Conclusion	34

Resources	35

Disclaimer

This is a preliminary document and may be changed substantially prior to final commercial release of the software described herein.

The information contained in this document represents the current view of Microsoft Corporation on the issues discussed as of the date of publication. Because Microsoft must respond to changing market conditions, it should not be interpreted to be a commitment on the part of Microsoft, and Microsoft cannot guarantee the accuracy of any information presented after the date of publication.

This White Paper is for informational purposes only. MICROSOFT MAKES NO WARRANTIES, EXPRESS, IMPLIED OR STATUTORY, AS TO THE INFORMATION IN THIS DOCUMENT.

Complying with all applicable copyright laws is the responsibility of the user. Without limiting the rights under copyright, no part of this document may be reproduced, stored in or introduced into a retrieval system, or transmitted in any form or by any means (electronic, mechanical, photocopying, recording, or otherwise), or for any purpose, without the express written permission of Microsoft Corporation.

Microsoft may have patents, patent applications, trademarks, copyrights, or other intellectual property rights covering subject matter in this document. Except as expressly provided in any written license agreement from Microsoft, the furnishing of this document does not give you any license to these patents, trademarks, copyrights, or other intellectual property.

Unless otherwise noted, the example companies, organizations, products, domain names, e-mail addresses, logos, people, places and events depicted herein are fictitious, and no association with any real company, organization, product, domain name, email address, logo, person, place or event is intended or should be inferred.

© 2006 Microsoft Corporation. All rights reserved.

Microsoft, Rally, Visual Studio, Windows, Windows Live, Windows Server, and Windows Vista are either registered trademarks or trademarks of Microsoft Corporation in the United States and/or other countries.

The names of actual companies and products mentioned herein may be the trademarks of their respective owners.

# Introduction

Devices Profile for Web Services (DPWS) is an open networking standard that enables industry partners to confidently invest in Web services for devices). DPWS enables devices to interact with Microsoft® Windows® and other operating systems over Internet Protocol (IP)-based networks.

DPWS is a specification that extends the Web services architecture to resource-constrained, network-connected devices. This specification is built on standard Internet protocols including Extensible Markup Language (XML) and Simple Object Access Protocol (SOAP).

DPWS allows network-connected devices to advertise their functionality and then offer these functions by using Web services over IP-based networks. This allows these devices to communicate with computers and other devices on the network or the Internet by using the Web services framework.

Here are a few of the many uses of DPWS :

- Enterprise devices including printers, fax machines, overhead projectors, and other shared network devices that can be easily discovered and associated and, when selected, are automatically configured for an enterprise scenario.

- Home healthcare monitoring, such as network-connected devices for collecting data on vital signs and exercising such as digital scales, accelerometers, and blood pressure, heart rate, and respiratory rate monitors. Market opportunities also include devices for home chronic-care monitoring such as for people with diabetes and a growing elderly population. Chronic-care scenarios require strong data integration with healthcare provider networks.

- Consumer electronics devices such as cameras that enable roaming and data transfer, for example, to send pictures across the Internet to users’ home computers, to their MSN Spaces site, or even to other devices. Projected uses also include home entertainment centers, MP3 devices, cell phones, and personal digital assistants (PDAs).

- Home control systems that automatically discover and configure lighting, heating, and other systems that monitor energy usage. This includes energy management systems for utilities that reduce peak load consumption. These systems could be monitored and controlled from a home computer or over the Internet.

# Windows Rally Technologies

DPWS is one of several technologies that are supported in the Windows Rally™ technologies for the Windows operating system. Windows Rally provides device manufacturers the best platform for providing effortless, secure, and reliable connectivity between devices and computers, and rich experiences for their customers. These technologies include:

- Web Services on Devices Application Programming Interface (WSDAPI). The Windows operating system provides native support for DPWS through WSDAPI. WSDAPI provides the best end-to-end solution for Web services on devices because it supports both computers using Windows Vista™ and network-connected devices using Windows CE. (WSDAPI for Windows CE support will be provided in 2007.) WSDAPI is discussed in more detail later in this paper.

- Plug and Play Extensions (PnP-X) for Windows. This set of extensions to Plug and Play integrates network-connected devices into the Windows Plug and Play subsystem, making them appear as a local device inside Windows. They provide an installation experience that is similar to attaching a physically connected device. After installation in Windows, PnP-X extends the value of Windows Update to networked devices and other integrated device services.

- Link Layer Topology Discovery (LLTD). This data-link layer (layer-2) protocol enables applications to discover devices and determines the topology of the network. LLTD allows devices to be added to Windows Vista network topology mapping, which is a central graphical view of everything connected to the network. LLTD Quality-of-Service (QoS) extensions can prioritize media streaming for devices, even on networks with limited bandwidth.

- Windows Connect Now. Windows Connect Now technologies enable simple and secure configuration of Wi-Fi networks and provide for the provisioning of wireless devices. Windows Connect Now is a method for transferring wireless home network settings (service set identifier—SSID—and password) to a network-connected device. This can be done by using a USB flash drive through the USB port, by using an Ethernet cable, or in-band by using Wi-Fi and a personal identification number (PIN).

The Windows Rally technologies enable network-connected (IP-enabled) devices to connect over the network or Internet in a manner that is comparable to a physically connected device. Network-connected devices use the network as the “communication bus.”

From a user's perspective, Windows Rally technologies will largely eliminate the experiential difference between using devices that are directly connected to a computer and those that are connected to a network. Typically, a developer can access network-connected devices uniformly by using higher-level publication services and function discovery.

For more information about Windows Rally technologies, visit the Microsoft Windows Rally Web site at: 
	http://www.microsoft.com/whdc/rally/default.mspx

# DPWS Technology Overview

The use of smart devices is expanding at an incredible rate. This is primarily due to advancements in device technologies including faster and smaller processors, increased computational power, shrinking memory costs, and greater network connectivity.

High-growth areas for network-connected devices include enterprise devices, consumer electronics, healthcare, home control, retail, and manufacturing systems. The one common requirement across all of these markets is ease of use, ubiquitous connectivity, and interoperability between devices, computers, and Internet services.

However, network-connected devices are perceived as difficult to use. Network-connected devices and wireless networks have historically been hard to configure. For device manufacturers, the support costs and return rates are often high. In many cases, device manufacturers do not see any differentiation for their products at the networking-layer level.

The best strategy for delivering effortless connectivity and interoperability successfully is to use Web services. With DPWS, smart devices can be easily incorporated to support Web services scenarios that traditionally have been beyond the reach of individual devices, including home, small office, and enterprise scenarios.

Microsoft, IBM, BEA Systems, Intel, and other industry leaders have invested significantly in Web services as the way for applications to communicate with each other.

## Devices Profile for Web Services

Windows Vista provides native support for device Web services through its implementation of DPWS, which offers the best solution for providing Web services connectivity on network-connected devices. It contains a generic architecture and an open interface that allow network-connected devices to communicate with computers, Web services, applications, and other devices over networks and across the Internet.

DPWS defines a limited resource footprint for devices that are resource constrained as compared to desktop computers and commodity servers. This footprint ranges from 100 to 500 KB, depending on the device’s security requirements.

About Network-Connected Devices

A device is considered to be network connected if it provides native support for IP. Network-connected devices are typically built to run on Ethernet or on Wi-Fi networks (802.11). Windows supports a variety of IP device connectivity options including PCI, USB, and IEEE 1394.

IP can be implemented to run on top of other buses, but many solutions include traditional non-IP–based methods for tasks such as discovery and association. For example, the Bluetooth IP personal area network (PAN) can be used only after the device has been discovered and associated by using the Bluetooth stack.

Network-connected devices can program, and be programmed by, computers and Internet-based Web services over a TCP/IP network. This opens up opportunities for automation and integration, and makes it easier to adapt devices to new scenarios and to improve product differentiation.

For example, a network-connected enterprise printer can provide value-added services such as automatically notifying a Windows Vista computer when a new printer toner cartridge is needed.

In addition, Windows-based music storage and playback devices can enable subscribers to connect directly to Internet music stores.

Windows Vista offers a consistent method of discovering and installing devices. With previous version of Windows, the "IP bus" was handled differently from other buses. For example, in Microsoft Windows XP, bus-connected devices appear in Device Manager and network-connected devices are shown in My Network Places.

With Windows Vista all network-connected devices appear in Network Explorer. Devices that support PnP-X can also support the PnP infrastructure and be enumerated in Device Manager with other bus-attached devices.

DPWS is a lightweight subset of Web services that defines a standardized method of interaction with a device class. This profile does not contain all of the requirements that are currently placed on desktop Web services. It prescribes how to use the Web services specifications on devices to enable networking functions:

- Dynamically discovering a Web service

- Sending secure messages to and from a Web service

- Describing a Web service

- Interacting with a Web service on the device by using Web Services Description Language (WSDL)

- Subscribing to, and receiving events from, a Web service

For example, DPWS defines how a specific class of network-connected device uses Web services such as a printer or digital still camera. For more information, see "How DPWS Exchanges Messages" later in this paper.

DPWS calls Web services specifications to enable a base level of interoperability between devices and Web services in core areas. These areas include rich metadata exchange and messaging, in addition to built-in support for discovery, security, and eventing, and cross-Internet support, proxy support, and so on.

DPWS is comprised of a single networking architecture and a set of standards-based communication protocols as seen in Figure 1. These protocols allow devices to contribute to traditional Web services scenarios. The protocols support rich functionality and a variety of service requirements.

Figure 1. DPWS foundation

DPWS is built on common standards for Web services:

- XML

- SOAP

- Hypertext Transport Protocol (HTTP)

- Transmission Control Protocol/Internet Protocol (TCP/IP)

- Message Transmission Optimization Mechanism (MTOM)

- Secure Sockets Layer (SSL) and Transport Layer Security (TLS)–based security

DPWS also supports the WSDL industry standard.

DPWS enables developers to write applications for devices by using common tools that minimize the development cycle and time to delivery. This profile lets device manufacturers focus on improving product differentiation and increasing customer value.

DPWS gives implementers clear guidance on how to address the most common design issues that are involved in improving device interoperability. Unlike other specifications, this profile describes schema, message formats, and other technical details. Devices connect by using an agreed-upon message format, rather than by forcing disparate devices to run identical software.

DPWS can be used by developers who are implementing networking libraries or platform components. The profile is written as a set of numbered normative statements together with a rationale and examples. The profile can also reference functions from other specifications.

DPWS allows network-connected devices to communicate with other devices and computers. For example, a printer typically receives print jobs, but could also send important device status notifications, such as a device event when it is first connected or a servicing event when its printer supplies run low.

DPWS provides a foundation for application protocols that enables device vendors to:

- Concentrate on designing messages for specific device classes, rather than expending resources to define proprietary protocols.

- Develop new messaging patterns, transports, metadata, capabilities, and requirements.

However, DPWS does not define application messages for specific device classes. Application messages can be defined by individual vendors, industry groups, or standards organizations representing a device type. For example, the application messages for printers were co-authored in workshops with Microsoft and leading printer manufacturers.

Value to Device Manufacturers

Device manufacturers currently expend considerable resources developing proprietary protocols to support device implementations. Proprietary solutions typically entail higher costs and limited interoperability and deliver an inconsistent customer experience.

In contrast, DPWS enables device manufacturers to use an exposed programming API, WSDAPI, and an industry-standard contract model (WSDL) to decrease development costs and improve the return on investment (ROI).

The advantages of using DPWS include:

- Common standards. DPWS protocols are built by using well-known Internet standards including XML and SOAP, HTTP, and TCP/IP. This allows developers to use a unified tool set that is transferable across an entire range of implementations.

- Interoperability. DPWS enables network-connected device implementations to share data and use capabilities from other applications. It does not matter how the applications are built, which operating system or platform is used, or what devices are used to access them.

- Scalability. DPWS is scaled down specifically to support resource-constrained devices. This support can be scaled out to support large organizations and diverse geographical locations.

- Security. DPWS supports channel-based security such as TLS with network-connected devices. It can also employ Web services message-based security such as WS-Security. Note that WS-Security is not supported in the first release of Windows Vista.

## Web Services Description Language

DPWS uses the WSDL industry-standard to describe the contract between a class of network-connected devices and other Web services. Individual vendors, industry groups, or standards organizations can define WSDL for a specific class of devices including:

- The public interface to the Web service. This interface allows other applications and resources to write applications to control a specified device class.

- Control messages that occur over the network and the actions and events that the device class supports.

When a client program connects to a network-connected device, the program reads the WDSL file to determine what functions are available on the device.

To use DPWS, the device manufacturer must develop a WSDL file, applications, and a TCP/IP implementation that resides on the device. In common practice, the device manufacturer typically already has a TCP/IP implementation and must create only the WSDL file.

WSDL is often used in combination with SOAP and XML schema to describe the valid format of messages over the network. Any special data types that are used are embedded in the WSDL file in the form of XML schema. The client can then use SOAP to call the functions in the WSDL file.

## Web Services on Devices API

The Windows operating system provides native support for DPWS through WSDAPI, which contains a set of APIs that allow developers to build Web services applications that interact with network-connected devices.

The advantages of using WSDAPI include:

- Providing the best end-to-end solution for Web services on devices because it supports both computers (Windows Vista) and network-connected devices (Windows CE).

- Allowing developers to create components rapidly, thereby reducing time to delivery and improving the user experience. WSDAPI delivers rich metadata exchange and messaging in addition to built-in support for discovery, security, and eventing.

- Creating better integrated control and management applications. For example, WSDAPI can be used to create a media management application that controls video playback devices, audio playback devices, and other types of IP devices through a single integrated interface.

- Enabling a device client or device host implementation. WSDAPI enables the device host to publish to network-connected devices and discover services on a network. Clients can be written in the C++ programming language by using WSDAPI. (Note that WSDAPI executes unmanaged code.)

- Providing the foundation on which particular types of device classes can be built including the device profile and a set of protocols. When used with the WSDAPI code-generation tool, WSDAPI provides an interface for sending service-specific messages.

## WSDAPI Code-Generation Tool

Microsoft provides the WSDAPI code-generation tool for building Web services applications for network-connected devices. This tool can consume a device service interface description based on WSDL and produce proxy and stub code for use with the WSDAPI stack. For clarification, a client application uses a proxy to communicate with the service. Clients usually import a service's metadata to generate proxy code that can be used to invoke the service.

Application programmers can use the proxy and stub code and add the specifics for the device to complete the implementation. The WSDAPI code-generation tool generates code that exposes WSDL as a programming interface that calls into WSDAPI, which simplifies WSDAPI coding.

Software tools for the PC and for devices will be available in the Windows Software Development Kit (SDK) and the Windows Driver Kit (WDK), respectively. For more information, see “Web Services on Devices” on the WHDC Web site.

## Device Communication Flow

To understand what technology goes on the PC and what goes on the device, it is important to understand how the technologies communicate. The device communicates with the PC in two phases.

### Phase 1: Web Service Device Discovery

Before a PC can communicate with a device, it must first locate, or discover, the device. Device discovery can either be active or passive. Active device discovery is when the PC issues a search for devices that match some criteria (that is, all printers). Passive device discovery is when a PC is listening for the device to announce its presence to the network (such as when it is powered on). In either method, after the device is discovered, metadata describing the device and its general functionality is exchanged. Upon receiving the device’s metadata, a second metadata exchange occurs, during which the PC learns about a device’s service. This discovery is detailed in Figure 2, which shows the communication path among the various technologies.

Figure 2. Phase 1: Web services device discovery

### Phase 2: Web Services Control and Eventing

After the discovery process is completed, the application on the PC can control the device’s functionality and subscribe to device events. For example, if the device is a printer, the PC application can send print jobs to the device or query the state of the printer. In addition, if a print job completed, toner is low, or an error occurred such as a paper jam, the device can send an event to the application notifying the user of the situation. As seen in Figure 3, to control the device functionality, the application calls into the proxy code (derived from passing the service’s WSDL through the WSDAPI code-generation tool). The generated code, in turn, calls into WSDAPI, which sends the request through the network by using HTTP or HTTPS. This is received by the device’s DPWS stack and sent to the device application. In the case of a device using a Windows CE operating system, the device’s WSDAPI stack receives this call and sends it through the generated WSDL code to the device’s service.

Figure 3. Phase 2: Web services control and eventing

## DPWS Implementation

To implement DPWS, the device manufacturer implements a service and a DPWS stack on the device. In addition, the manufacturer can provide advanced functionality by developing an application on the PC that communicates with the device. The DPWS stack can be purchased from a third-party provider or created entirely by the manufacturer. Manufacturers that use Windows CE on their device have access to a DPWS stack by using the Windows CE WSDAPI implementation.

# Web Services on Devices Scenarios

DPWS can be used in a wide range of network-connected device scenarios including ones related to enterprise products, vertical markets, and consumer electronics. The device scenarios in the following sections introduce related concepts and solutions and provide sample guidance that emphasizes pertinent design issues for using DPWS.

These device scenarios include:

- Enterprise printers scenario, which describes how DPWS can be used to support network-connected printers either from a central print server or from individual computers.

- Fitness and chronic healthcare devices scenario, which describes how DPWS can be used to support health and fitness and chronic home healthcare device monitoring to improve healthcare management.

- In-home digital still camera scenario, which describes how DPWS can be used to support a digital still camera with an in-home network.

# Enterprise Printers Scenario

This scenario uses the Windows Vista printing system API, which provides built-in support for DPWS for network-connected printers. Note that the Windows Vista printing system API includes the WSDL file. Microsoft developed the WSDL for network-connected printers with the support of major printer vendors.

Windows Vista also contains support that enables independent hardware vendors (IHVs) to develop DPWS printer drivers. In this case, the printer drivers use the Windows Vista printing system API, which calls in to WSDAPI.

## Enterprise Printers Scenario Description

In this scenario, Contoso Corporation wants to improve its enterprise support for network-connected printers so it can provide better security for users and reduce information technology (IT) printer support costs. Contoso’s IT Technical Support wants to remotely monitor the operational status of printers to determine when replacement items such as printer cartridges are needed. Contoso also wants to provide users with better notification of the status of print jobs.

Contoso has decided to deploy heir printers with Windows Vista to provide these advanced capabilities for their enterprise network. When a network-connected printer on an enterprise network uses DPWS, it supports the following advanced capabilities:

- Event notification. The printer can support rich event notification. In a traditional enterprise scenario, the requestor who sends a print job to the printer receives only an indicator when the print server has finished spooling the print job. With DPWS, the printer can return a notification that indicates when the print job has actually completed (true end-of-job notification).

Printer error notifications such as out-of-paper or maintenance conditions, including low toner and paper jams can be sent directly to the IT Help Desk for correction.

- Security. DPWS security provides for access control and a secure connection. Encryption prevents unauthorized persons from intercepting and reading sensitive print jobs during transmission. In addition, DPWS security features enable a printer to authenticate the client computer and track its usage for billing purposes.

- Services. DPWS can provide access to services, for example, a service that automatically reorders ink cartridges when the printer indicates that the ink is running low.

Note that DPWS can be used to support network-attached printers either from a central print server or from individual computers. In either case the printer must be associated with each computer before the requestor can submit print jobs directly.

## How the Enterprise Printers Technology Works

This section describes how the network-connected enterprise printer scenario works. This includes the technologies that are involved and how these technologies perform.

To learn more about the technologies that are described in this section, see "Resources" at the end of this paper.

Figure 4. Enterprise printers scenario

The steps in the enterprise printers scenario include:

1.	DPWS allows printers to be discovered by using WS-Discovery messages, which permit network-connected printers to be located by using Network Explorer or directed discovery.

For a complete description of WS-Discovery and how DPWS message exchanges work, see "How DPWS Exchanges Messages" later in this paper.

In addition, directed discovery enables clients that have the directed discovery URL for a device to bootstrap communication with that device such as when the client and device are separated by a router or firewall.

For DPWS printers, directed discovery communicates the printer URL to the client by using either of two options:

- If the printer uses the URL convention (published in DPWS for printers), directed discovery uses the IP address or hostname of the printer to establish communication. The Add Printer Wizard constructs a URL according to the convention.

- The Add Printer Wizard can also accept any arbitrary URL. The user would typically obtain this URL by printing the printer configuration page from the device.

2.	If PnP-X is implemented on each printer, a PnP-X hardware ID specifies what software must be installed for the device. (Right-clicking Install in Network Explorer indicates which drivers to install for the device.) PnP-X–compliant printers also provide information about the Web services to be exposed (supported) by the device.

On the print server, PnP-X detects each printer’s presence and installs the printer drivers. When the printer leaves the network, WS-Discovery allows PnP-X to manage the printer’s presence. When PnP-X detects the printer’s presence, it loads or unloads device drivers such as when the printer joins or leaves the network.

In addition, note that the Windows Vista printing system API contains WSDL for enterprise printers. WSDL describes the contract between a class of network-connected devices and other Web services. See "Web Services Description Language" earlier in this paper.

3.	Enterprise workstations, PCs, and laptops send print jobs to the print server running Windows Vista. The workstations, PCs, and laptops are not required to be upgraded to Windows Vista.

Note that a PC or laptop that uses Windows Vista can print directly to a network-attached printer because it shows the printer device state.

4.	The print server (spooler) collects the print job and sends it to the printer. The print server interacts with the printers by using DPWS.

## How DPWS Exchanges Messages

DPWS provides the following functionality through the Web services interfaces that it calls:

- Discovery. Discovery messages are the first step in client and device association. WS-Discovery is most commonly used to locate services according to the device type or by the scope into which it has been deployed. WS-Discovery is not used solely for device discovery. In managed networks, WS-Discovery allows for proxy support for discovery. In this case, the device then communicates directly with the proxy rather than broadcasting across the network.

- Security. Windows Vista contains built-in support for channel-based security such as TLS to protect the confidentiality and integrity of messages.

DPWS can also employ Web services message-based security such as WS-Security. Note that WS-Security is not supported in the first release of Windows Vista.

- Description. WS-Transfer Get Metadata provides an extensible mechanism to retrieve service metadata, including message descriptions (WSDL), interservice relationships, and industry-specific metadata.

- Control. DPWS defines how message types are serialized during transmission. However, DPWS does not define control messages for applications. These messages are described by using WSDL for the device class. See "Web Services Description Language" earlier in this paper.

- Eventing. WS-Eventing allows industry-specific applications to define their own event models including notifications, thereby providing a lease model for subscriptions and enabling either the service or the client to cancel a subscription. WSDL defines events for specific device classes.

- Messaging (attachments). MTOM allows large amounts of binary data such as an audio stream to be attached in a message for the efficient transmission of attachments.

Figure 5 details a typical exchange that uses DPWS to describe the messages between a client that has a print request, an enterprise printer, and a print service hosted by the device.

Figure 5. Example of DPWS message exchanges

#### Discovery Messages

- "Who’s there?" – Client Probe (Message 1)

To find the printer, the client sends a multicast Probe message over User Datagram Protocol (UDP). The Probe message indicates that the client is looking for a print service.

In WS-Discovery, the Probe message is used to locate services and to scale to a large number of endpoints. (WS-Discovery can use a discovery proxy to discover devices on other subnets on the network, although this capability is not currently implemented in Windows Vista.)

Note that directed discovery enables clients that have the directed discovery URL for a device to bootstrap communication with that device such as when the client and device are separated by a router or firewall.

For directed discovery, the Probe and Probe Match messages are sent over HTTP to enable clients that have the directed discovery URL for a device to bootstrap communication with that device, even when multicast UDP communication (required for normal WS-Discovery) cannot take place between the client and the device. For example, directed discovery is used when the client and device are separated by a router or firewall.

- "It’s me" – Device Response to Probe (Message 2)

The device listens for Probe messages. In this case, it hosts a print service and responds with a Probe Match message (defined in WS-Discovery) directly to the client. The Probe Match message contains three pieces of information:

- A SOAP-level address for the device, which can be an endpoint reference. The endpoint reference is defined in WS-Addressing as a transport-neutral mechanism for end-to-end addressing of Web services and messages.

- A service’s universally unique identifier (UUID). This requires the client to issue a Resolve query to obtain an endpoint to that particular service.

- The transports where the device can be reached. All devices must expose SOAP over HTTP, but can support other transports, too.

- Security requirements and device capabilities.

In simple ad hoc deployments, devices respond directly to the discovery proxy. In managed deployments, a discovery proxy (WS-Discovery) informs clients not to send multicast requests and instead communicate directly with the discovery proxy to find appropriate services.

Note that these discovery messages are the first step in client and device association such as Plug and Play.

- "Where are you?" – Resolve Unique ID (Message 3)

For an enterprise printer, the Probe Match message identifies a print service’s unique ID. If the Probe Match message does not have an explicit endpoint, then the client must submit a Resolve message to resolve the unique ID to an actual endpoint such as a URL or an IP address.

If the Probe Match message provides an explicit endpoint such as a URL, then the Resolve message is skipped.

- "I’m here" – Response to Resolve (Message 4)

The device listens to the Resolve messages. It responds with a Resolve Match message (defined in WS-Discovery) directly to the client. The Resolve Match message contains the actual endpoint for the unique ID.

#### Security Messages

If the client requires security, the next message in the sequence can be used to set up a secure channel between the client and the device. A secure channel protects the confidentiality and integrity of all subsequent messages between the client and device or the services that the device hosts.

A secure channel with the client can be used only when the device provides an endpoint that uses HTTPS. If the device does not specify a secure channel endpoint, then the client cannot use a secure channel. If the device specifies more than one endpoint, then the client can choose which endpoint to use. The lifetime of a secure channel is implementation dependent; the channel may be created, closed, and then created again when the device wants to send another message to the host.

For authentication, the device uses a device certificate. In managed deployments, a device can have a certificate with a root that the client trusts.

DPWS defines an initial, simple security profile that can be extended to provide greater and more flexible protection and authentication. DPWS also specifies that Discovery messages can be signed.

#### Description Messages

- "Tell me about yourself" – Get Metadata (Message 5)

To discover more about the device and services that it hosts, a client can send a Get Metadata message directly to the device. The Get Metadata message is defined in the WS-Transfer Get Metadata specification to retrieve arbitrary types of metadata that are associated with a service.

- "Here’s what I can do" –  Metadata Response (Message 6)

The device responds with its metadata, either inline within the response message or by including a pointer to the metadata. In addition to information such as the manufacturer of the device, serial number, and firmware version, the metadata includes the endpoint reference for each of the services that the device hosts.

- "Tell me about your service" – Get Metadata from Service (Message 7)

To discover more about the print service, a client can send a Get Metadata message directly to that service.

- "Here is my service" –  Service Metadata Response (Message 8)

The service responds with metadata that contains the URL that points to the WSDL definition for the operations that are supported by the service and printer-specific metadata. For example, this information might describe the printer’s ability to print in color, pages per minute (PPM), number and size of paper trays, and so on.

Strictly speaking, Messages 3 through 6 could be optional. When DPWS is applied to devices for specific markets, standards organizations within those industries could mandate standard services to be hosted, standard capabilities for those services, and even standard names for those services. Although this implies less flexibility, a client that uses the standard functionality would not be required to retrieve device or service metadata.

#### Control Messages

- "Interact with the service" (Messages 9 and 10)

The client sends a message to start a print job and the print service responds. DPWS does not define these application messages. However, individual vendors or standards organizations working in specific industries can define these messages. WSDL describes the definition of these messages.

#### Eventing Messages

- "I want to be kept informed of your status" – Subscribe Request (Message 11)

To receive updates on the status of the print job, or to receive information about the overall status of the print service, the client can send a Subscribe message to the print service. The Subscribe message is defined in WS-Eventing along with messages to renew and cancel a subscription.

- "Here’s your subscription" – Subscribe Response (Message 12)

The Subscribe response message that is defined in WS-Eventing includes an identifier for the subscription and when it will expire. If the client wants status after this expiration, it must renew the subscription.

#### Attachment Messages

- "Here’s an attached task" – "I’ll do the job" (Messages 13 and 14)

The client sends the print job to the print service that is encoding large images as attachments to the message. The response from the print service indicates that the job has been accepted.

Attachments are defined in MTOM as an optimization for transmitting a SOAP message by selectively re-encoding portions of the message while still presenting an XML information set (infoset) to the SOAP application. MTOM enables the efficient inclusion of binary data such as an audio stream to be attached in a message, combining data and control in a single connection.

#### Eventing Messages

- "Something happened" – Application Event (Message 15)

The print service sends an event to the client that informs it, for example, of a change in the print job such as the number of pages printed, paper jam, or job completed. DPWS neither defines nor constrains the format of application events. Specific industries can define general messages as events.

# Fitness and Chronic Healthcare Device Scenario

This scenario describes how DPWS can be used to support health and fitness and chronic home healthcare device monitoring to improve healthcare management.

The focus on preventive wellness, fitness, and home health monitoring is getting worldwide attention as a crucial solution for controlling healthcare costs. This gives healthcare device manufacturers new market opportunities.

Implementing DPWS and related technologies can benefit many devices for home healthcare monitoring, such as devices for collecting data on vital signs and exercise such as digital scales, accelerometers, and blood pressure, heart rate, and respiratory rate monitors. Health clubs are also using network-connected cardio and weight machines to help track exercise efforts.

Market opportunities also include devices for home chronic-care monitoring, such as for people with diabetes and a growing elderly population. For chronic-care scenarios, the integration of the data with healthcare providers is essential.

In this scenario, a consumer, Abby, purchases health and fitness devices, installs them on her home network, and gathers exercise information to monitor her wellness on a home PC and on an online healthcare portal.

## Healthcare Device Scenario Description

Abby wants to maintain her health and fitness. She usually has time to exercise only one to three times a week, alternating between running and cycling. Abby travels on business once a month and exercises in the hotel health club by walking or running on the treadmill. She wants to exercise more often.

To track her exercise efforts, Abby purchases a digital scale, a blood pressure monitor, and an accelerometer to monitor her jogging activity. These devices come with a software application that allows her to track her progress on her home PC and with a membership offer to join the online healthcare portal.

At home, Abby uses the CD that came with the devices to install the software:

- Abby has a wireless network in her home. With her PC running, she turns on each device. The devices use Web services over Wi-Fi and Bluetooth (PAN) to wirelessly communicate with the PC. DPWS can work over any "IP-based bus."

- The next section, "How the Healthcare Device Scenario Technologies Work," describes how Abby installs her health and fitness devices on her wireless home network.

After Abby installs the devices, she tests them:

- She stands on the scale, and her weight appears on the Home Health page on her PC.

- Abby goes to the healthcare portal Web site and registers for the program by using the authorization number that was provided with the membership offer. After registering, she stands on the scale again and watches as her weight appears on the Web site, along with the current date.

- Abby tests the blood pressure device. Then she goes for a jog with the accelerometer clipped to her waist. After returning home, she pushes the Transmit button and the distance traveled is uploaded to her PC and then sent to the healthcare portal.

Over the next month, Abby tracks her weight, blood pressure, and exercise efforts on her home PC and monitors her progress against others on the healthcare portal. One fun feature showed how her fitness measurements compared to other local, national, and international averages for her gender, age, height, and build.

## How the Healthcare Device Scenario Technologies Work

This section describes how Abby’s health and fitness devices are installed and how they perform. This includes the involved technologies, what the device manufacturer implements, and the benefits of using these technologies.

To learn more about the technologies described in this section, see "Resources" at the end of this paper.

Figure 6. Healthcare device scenario

The steps in this scenario include:

1.	Abby adds each healthcare device to the wireless home network.

Technologies involved on the PC:

- Windows Connect Now allows Abby to transfer the service set identifier (SSID) and password for the wireless home network to the device. Windows Connect Now can be implemented by using a USB flash device, Wi-Fi with PIN, or an Ethernet cable.

What the manufacturer implements on the device:

- Windows Connect Now can be supported by using in-band Wi-Fi (not additional hardware support), Ethernet, or a USB flash drive. At a minimum, the device hardware should support in-band Wi-Fi by using a PIN to secure the transaction of wireless settings. For details, see “Configuration Technologies: Windows Connect Now.”

Implementation benefits:

- Implementing these technologies lowers product support costs by making it easy to connect devices to a wireless network.

2.	Each healthcare device is discovered on the network.

Technologies involved on the PC:

Support for PnP-X association:

- Discovery (WS-Discovery) locates the device on the network.

- PnP-X provides a "Plug and Play” like experience for network-connected devices.

- Network Explorer displays a list of the devices that are available on the network and shows each device as it is discovered on the network.

What the manufacturer implements on the device:

These technologies require the device manufacturer to:

- Implement Discovery (WS-Discovery) in compliance with the DPWS specification. The PnP-X metadata extensions must also be added to the description document. For details, see “PnP-X: Plug and Play Extensions for Windows Specification.” For a complete description of how these Web services message exchanges work, see "How DPWS Exchanges Messages" earlier in this paper.

The client and device exchange the following messages:

- The client sends a WS-Discovery Hello message. The Hello message is a multicast UDP message that is sent to everyone on the same subnet.

- The device receives a WS-Discovery Probe message from the Windows Vista PC. The device responds to the Windows Vista PC with a Probe Match message.

- The device receives a WS-Discovery Resolve message from the Windows Visa PC. The device sends a Resolve Match response that contains the transport address of the device.

- The device receives a WS-Transfer Get Metadata request from the Windows Vista PC. The device sends a Get Metadata response with its metadata.

- Implement PnP-X on the device. In this scenario, all devices are PnP-X compliant. Implementing PnP-X support requires a PnP-X hardware ID that specifies what software must be installed for the device. (Right-clicking Install in Network Explorer exposes actions such as “install” for the device.)

Note: The device manufacturer can decide to write the Network Explorer .dll extension that gives the device other options. For example, device manufacturers can customize icons and context menus in Network Explorer to support branding and extra features.

Implementation benefits:

- Implementing these technologies allows the user to discover network devices by using a common experience through Network Explorer in Windows Vista.

3.	Abby selects and installs each healthcare device in Network Explorer.

Technologies involved on the PC:

- PnP-X detects each device’s presence and installs the device drivers.

- Discovery (WS-Discovery) allows PnP-X to manage the device presence. PnP-X detects the device’s presence and loads or unloads device drivers such as when the device leaves or returns to the network.

- Network Explorer displays a list of the devices on the network and shows these devices as each is added to the network.

- Windows Update checks for changes to the drivers and applications (application support is a new feature of Windows Update) at setup and automatically downloads files from the Internet as needed (no user action is required).

- WSDAPI allows Web services applications and drivers to interact with network-connected devices.

What the manufacturer implements for the device:

These technologies require the device manufacturer to:

- Develop an installation CD that contains a driver package for the device. The driver package contains an INF file that matches the hardware ID that was specified in the device metadata. The driver package also contains driver and application files for the device.

- Set up the device to send a Hello message when powered on or joining the network and send a Goodbye message when the device is powered down or leaves the network. This ensures that presence information in PnP-X is maintained correctly.

- Complete the Windows Vista Logo Program submission for eligibility for delivering drivers and other software through Windows Update.

Implementation benefits:

- For the end user, the result is easy installation.

- Network Configuration is a simple as adding a numerical PIN to configure a more secure wireless profile.

- PnP-X persistence is a key benefit for network-connected devices.

- Security for Windows devices is improved by using Windows Update.

- The device manufacturer can reduce support costs and improve customer satisfaction.

4.	Software on the PC collects data from the device, stores a local copy, and then submits the data to the healthcare portal.

Technologies involved:

- WSDL on the device describes the contract between the device class and other Web services including the healthcare portal interface. The application programmer uses WSDL to get a description of the actions and events that the device class can transmit to the portal.

Note that the WSDAPI code-generation tool consumes a device class service description (WSDL) and produces proxy and stub code for use with the WSDAPI stack.

- WS-Eventing allows the device to act as an event source. An application on the PC can subscribe to the event source such as the current heart rate, and the device sends event notifications when the heart rate changes. This is accomplished by the device acting as the Web service and the PC application acting as the client that subscribes to the events on the device’s Web services. For more information, see "How DPWS Exchanges Messages" earlier in this paper.

What the manufacturer implements:

These technologies require the device manufacturer to:

- Develop or apply WDSL for each device class. Individual vendors, industry groups, or standards organizations working in specific industries can define WSDL.

- Implement WDSL on the device.

- Implement an application that aggregates the data from the device and securely uploads it to a Web site such as to the healthcare portal. This can involve writing a driver that PnP-X can install to support this capability.

Implementation benefits:

- Application programmers can use WSDL to get a description of the actions and events that a device class supports.

- Application programmers can import WSDL into development tools such as Microsoft Visual Studio®.

5.	The healthcare portal collects and processes data.

Technologies involved:

- Function Discovery enables the PC to discover devices. Applications can use the Function Discovery API to enumerate the devices.

- The WSDL interface on the device describes the device to the healthcare portal. The application programmer uses WSDL to get a description of the actions and events that the device can transmit to the portal.

- Visual Studio provides developer tools for implementing the Web site and client application. It can be used to import WSDL for the healthcare portal to support applications.

- Windows Communication Foundation (WCF) provides a secure and reliable communication framework for implementing the healthcare portal Web service.

What the manufacturer implements:

These technologies require the device manufacturer to:

- Implement the Windows client application on the device to aggregate data from the device and upload it to the healthcare portal.

- Implement the healthcare portal or use a third-party portal.

Implementation benefits:

- Visual Studio and WCF provide compatible tools for implementing a healthcare portal Web service.

# In-Home Digital Still Camera Scenario

This scenario describes how DPWS can be used to support a digital still camera with an in-home network. Many consumer devices for in-home networks can benefit from the implementation of DPWS and related technologies.

When a digital still camera is integrated with the PC, the camera can capture, store, edit, and display photos on the computer. When the camera is integrated with Web services and connected by using the Internet, the camera can store images on other servers and send images to Internet sites.

A digital still camera can share photos by sending an event to the subscribing devices, with the picture attached. In this scenario, the PC is used as the management interface to configure the device and to set up the subscriptions.

This scenario focuses on using an in-home digital still camera to provide immediate access to services across the Internet. In this scenario, a consumer, Ichiro, purchases the digital still camera, installs it on his home network, and sets up a Web site that stores photos. Then he takes photos and saves them on a PC and to a Web site.

## In-Home Camera Scenario Description

Ichiro is a 30-year-old professional who uses his digital still camera to take photos of his family, at social events with his friends and while on vacation. He often posts his photos to a public Internet share for his friends and family to see and sends photos to people through e-mail.

When Ichiro decided to purchase a new digital still camera, he wanted more mega-pixels and something new and different. After considering various camera connectivity options, he bought a Wi-Fi camera with Windows Vista connectivity. The camera promised to allow him to transfer pictures to his home PC and to Web sites on the Internet.

At home, Ichiro unpacks the camera and finds the one-page instruction sheet that asks him to:

- Run the CD that came with the camera to install the software by using the Windows Vista PC.

- Set up a Microsoft Windows Live™ Spaces site to store his photos by using Microsoft Internet Explorer.

After completing these steps, Ichiro uses his camera to send a test photo to his PC over his home network. He also sends the test photo from the PC to his Windows Live Spaces site, which he verifies by using his PC.

Later Ichiro takes photos of his family by using his new camera. Then he downloads his photos to his My Pictures folder on the Windows Vista PC. A folder is created and his photos are transferred to the folder. The folder’s name is the date and time of the transfer. Then he checks his Windows Live Spaces site and confirms that his photos are available on the Web site.

## How the In-Home Camera Scenario Technologies Work

This section describes how Ichiro’s digital still camera is installed and how it performs. This includes the technologies involved, what the device manufacturer implements, and the benefits of using these technologies.

To learn more about the technologies described in this section, see "Resources" at the end of this paper.

Figure 7. In-home camera scenario

The steps in this scenario include:

1.	Ichiro adds the digital camera to the wireless home network.

Technologies involved on the PC:

- Windows Connect Now allows Ichiro to transfer the SSID and password for the wireless home network to the camera. Windows Connect Now can be implemented by using a USB Flash device, Wi-Fi with PIN, or an Ethernet cable.

What the manufacturer implements on the device:

- Device hardware and software support as needed such as for a wireless access key including Wi-Fi with PIN, USB Flash host port (read/write to a flash memory card), and USB or Ethernet target (cable) port.

Implementation benefits:

- Implementing these technologies lowers product support costs by making it easy to connect devices to a wireless network.

2.	The digital camera is discovered on the network.

Technologies involved on the PC:

Support for the USB flash host port and PnP-X association:

- Discovery (WS-Discovery) locates the camera on the network.

- PnP-X provides a "USB-like" experience for network-connected devices.

- Network Explorer displays a list of the devices that are available on the network and shows each device as it is discovered on the network.

What the manufacturer implements on the device:

These technologies require the device manufacturer to:

- Implement Discovery (WS-Discovery) in compliance with the DPWS specification. For a complete description of how these Web services message exchanges work, see "How DPWS Exchanges Messages" earlier in this paper.

The client and device exchange the following messages:

- The client sends a WS-Discovery Hello message. The Hello message is a multicast UDP message that is sent to everyone on the same subnet.

- The camera receives a WS-Discovery Probe message from the Windows Vista PC. The camera responds to the Windows Vista PC with a Probe Match message.

- The camera receives a WS-Discovery Resolve message from the Windows Visa PC. The camera sends a Resolve Match response that contains the transport address of the device.

- The camera receives a WS-Transfer Get Metadata request from the Windows Vista PC. The camera sends a Get Metadata response with its metadata.

- Implement PnP-X on the device. In this scenario the camera is PnP-X compliant. Implementing PnP-X support requires:

- PnP-X hardware ID that specifies what software must be installed for the camera. (Right-clicking Install in Network Explorer indicates which drivers to install for the device.)

Note: The device manufacturer can decide to write the Network Explorer .dll extension that gives the device other options. For example, device manufacturers can customize icons and context menus in Network Explorer to support branding and extra features.

Implementation benefits:

- Implementing these technologies allows the user to discover network devices by using a common experience through Network Explorer in Windows Vista.

3.	Ichiro selects and installs the digital camera in Network Explorer.

Technologies involved on the PC:

- PnP-X detects the camera’s presence and installs the drivers.

- Discovery (WS-Discovery) allows PnP-X to manage the device presence. PnP-X detects the camera’s presence and loads or unloads device drivers such as when the camera is taken from or returns to the network.

- Network Explorer displays a list of the devices on the network and shows these devices as each is added to the network.

- Windows Update checks for changes to the drivers and applications (application support is a new feature of Windows Update) at setup and automatically downloads files from the Internet as needed (no user action is required).

- WSDAPI allows Web services applications and drivers to interact with network-connected devices.

What the manufacturer implements for the device:

These technologies require the device manufacturer to:

- Develop an installation CD that contains a driver package for the camera. The driver package contains an INF file that matches the hardware ID that was specified in the device metadata. The driver package also contains driver and application files for the camera.

- Set up the camera to send a Hello message when powered on or joining the network and send a Goodbye message when the camera is powered down or leaves the network. This ensures that presence information in PnP-X is maintained correctly.

- Complete the Windows Vista Logo Program submission for eligibility for delivering drivers and other software through Windows Update.

Implementation benefits:

- For the end user, the result is easy installation.

- Network Configuration is a simple as adding a numerical PIN to configure a more secure wireless profile.

- PnP-X persistence is a key benefit for network-connected devices.

- Security for Windows devices is improved by using Windows Update.

- The device manufacturer can reduce support costs and improve customer satisfaction.

4.	Ichiro takes a test photo with his camera and stores a local copy on the PC.

Technologies involved:

- WSDL on the device describes the contract between the device class and other Web services. The application programmer uses WSDL to get a description of the actions and events that the device class can transmit.

Note that the WSDAPI code-generation tool consumes a device class service description (WSDL) and produces proxy and stub code for use with the WSDAPI stack.

- WS-Eventing allows the device to act as an event source. An application on the PC can subscribe to the event source, and the device sends event notifications when the event changes. This is accomplished by the device acting as a Web service and the PC application acting as the client that subscribes to the events on the device’s Web services. For more information, see "How DPWS Exchanges Messages" earlier in this paper.

- MTOM allows large amounts of binary data to be transmitted efficiently with the SOAP message. For example, when the user selects photos from the camera and shares them with the PC, the photos are sent as MTOM attachments.

What the manufacturer implements:

These technologies require the device manufacturer to:

- Develop or apply the WDSL for the device class. Individual vendors, industry groups, or standards organizations working in specific industries can define WSDL.

- Implement WDSL on the device.

- Implement an application that aggregates the data from the device and uploads it to the PC.

Implementation benefits:

Application programmers can:

- Use WSDL to get a description of the actions and events that a device class supports.

- Import WSDL into development tools such as Visual Studio.

5.	Ichiro sends the test photo from his PC to his Windows Live Spaces site.

Ichiro uses his Microsoft Internet Explorer browser to store and look at his photos on the Windows Live Spaces site.

# Summary of Benefits

By adopting DPWS and Windows Rally technologies, network-connected device manufacturers can benefit immensely. These benefits are summarized in this section.

## Lower Development Cost and Long-Term Investment Security

These technologies can provide the manufacturer with improved connectivity, interoperability, and advanced capabilities at a lower cost by supplying:

- Lower product development costs

DPWS is an open networking standard that enables industry partners to securely invest in Web services for devices. DPWS eliminates the need to create and support custom solutions and proprietary protocols. By using industry standard protocols, DPWS simplifies the development of device drivers and software for device implementations. In addition, Windows Vista provides built-in support for DPWS printer drivers. In the enterprise printer scenario, the printer drivers use the Windows Vista printing system API, which calls in to WSDAPI.

- Common, industry-wide solution

Web services are the industry standard for application communication across networks. By providing Web services connectivity to devices, manufacturers can reduce investment risk both for the developer and customer by providing a solution that is supported industry wide.

Extended Web Services to Devices. By using DPWS, the device manufacturer can plug Web services into network-connected devices by using a limited resource footprint. Devices can be easily incorporated to support typical Web services scenarios. DPWS can be used in a wide range of network-connected device scenarios including those for enterprise products, vertical markets, and consumer electronics as demonstrated by the scenarios in this paper.

- Strong, built-in security mechanisms

DPWS contains built-in security mechanisms including channel-based security transports that encrypt information such as SSL and TLS. DPWS can also employ Web services message-based security (WS-Security).

- Well-known Internet standards

DPWS is built on common Internet standards including XML, SOAP, HTTP, and TCP/IP. In addition, MTOM allows large amounts of binary data to be transmitted efficiently by using SOAP, as described in the in-home camera scenario.

- An open-platform solution

By using XML and common developer tools, devices can work across platforms and operating systems, regardless of the programming language. Manufacturers no longer must develop different code for each platform. For example, by using Web services architecture, the WSDL for network-connected printers, developed with the support of major industry groups, enables interoperability through a common XML interface.

- Common developer tools

- Manufacturers can use common interfaces for all systems and developer tools. DPWS uses a generic architecture and open interface that enables developers to write components and applications for devices rapidly by using common tools, thereby minimizing the development cycle and time to delivery.

- Microsoft offers the best end-to-end platform for developers to build connected experiences for Windows devices, computers, servers, services, using well known .NET development tools..

## Effortless Setup Experience and Fewer Product Returns

DPWS and Windows Rally technologies allow device manufacturers to provide customers with effortless setup and increased satisfaction by supplying:

- Effortless discovery and installation

- With DPWS and Windows Rally, the customer’s setup experience with a network-connected device is as simple as a physically connected USB device. This effortless setup ensures that customers can run network-connected devices easily out of the box.

- PnP-X enables easy discovery, association, and driver installation. Finding the IP address is automatic. PnP-X makes devices appear locally connected inside Windows. In all of the scenarios, PnP-X provides a Plug and Play–like experience with network-connected devices. PnP-X–compliant devices provide information about the Web services exposed (supported) by the device.

- PnP-X extends the value of Windows Update to networked devices and other integrated device services.

- Seamless network connectivity

- Network-connected devices connect to computers, other devices, and Internet services by using the network as the communication bus. Network-connected devices appear and act as if each is physically connected. In Windows Vista, Network Explorer displays a list of the devices that are discovered on the network, which provides a common experience to users.

- DPWS allows network-connected devices to advertise their functionality over an IP-based network and then offer these functions by using Web services.

- DPWS enables devices to find and access the Web services of other devices and computers on the network and the Internet.

- Increased customer satisfaction and fewer product returns

- The USB-like setup and simplified driver installation increase user satisfaction out of the box. In the enterprise printer scenario, PnP-X is used on the print server to detect each printer’s presence on the network and to load or unload device drivers, such as when the printer is added to or removed from the network.

- Simplifying user setup increases the likelihood of success, thereby reducing product returns.

## Richer Experiences and Greater Product Differentiation

DPWS and Windows Rally technologies allow the device manufacturer to improve products by providing:

- Rich end-user experiences

- From a user's perspective, DPWS and Windows Rally largely eliminate the experiential difference between using devices directly connected to a computer and those virtually connected over a network or the Internet.

- This increase in functionality and interoperability provides the developer with far more built-in capabilities and offers richer experiences to end users.

- Robust, secure encryption prevents unauthorized persons from accessing or intercepting protected information.

- Network-connected devices can communicate with other devices and computers. For example, DPWS event notification allows printers to send true end-of-job notifications and servicing events to computers.

- Improved product differentiation and customer value

- DPWS allows manufacturers to allocate more resources to enhance functionality and improve device integration with PCs, Internet services, applications, and other devices, rather than developing proprietary protocols and disparate solutions.

- By using the Web services architecture, DPWS allows devices to contribute to Web services scenarios not previously available to devices. For example, in the healthcare device scenario, the application programmer uses WSDL interface to get a description of the actions and events that the device can transmit to the healthcare portal.

- Developers can more easily add new features such as another device or new control messages, thereby improving customer value. Application programmers can use the WSDL interface to get a description of the actions and events that a device class supports and import WSDL into development tools such as Visual Studio.

- New opportunities for automation

- DPWS allows manufacturers and developers to increase customer value and expand market opportunities for automation and integration.

- Devices can more easily adapt to new scenarios. DPWS allows network-connected devices to program, and be programmed by, computers and Internet-based Web services over a TCP/IP network.

- DPWS can enable a device client or device host implementation. By using WSDAPI, a device host can publish to network-connected devices and discover services on a network. This is accomplished when the device acts as a Web service and the PC application acts as the client that subscribes to the events on the device’s Web services.

- Developers can create better integrated control and management applications. For example, WSDAPI can be used to create a media management application (interface) that controls video playback devices, audio playback devices, and other types of IP devices.

## Lower Support Costs

DPWS and Windows Rally technologies lower device manufacturer support costs by providing:

- Easy setup and installation

Effortless setup ensures that customers can run the device easily out of the box. This reduces setup errors and support calls.

- Better security and trustworthy computing

By using built-in channel-based security such as SSL and TLS, DPWS provides better access control, secure connection to the device, and encryption to prevent unwanted intrusion.

- Simplified upgrading

When PnP-X is used, network-connected devices can benefit from Windows Update and other integrated device services. In the enterprise printer scenario, drivers are updated by using Windows Update, which keeps the printer software current without IT staff intervention.

- Better access to Internet services, computers, and devices

DPWS allows devices to connect to and communicate with applications, computers, and other devices over networks and the Internet by using common Web services. Support for devices does not have to be custom built.

- Improved monitoring and diagnostic support

- DPWS provides better monitoring and event notification of the status of devices and their current operational state. For example, in the healthcare device scenario, WS-Eventing allows the device to act as an event source. When an application on the PC subscribes to the event source such as the current heart rate, the device sends event notifications, for example, when the heart rate changes.

- The Windows Rally technology, LLTD, allows for diagnostic self-support for Windows Vista users through a graphical map that shows the connection status of all computers and devices on a network.

- DPWS’s rich event notification can indicate when a servicing event occurs such as when a printer toner cartridge is low.

- Fewer support calls and quicker time to resolution

- Customer support calls are reduced due to effortless installation and the improved self-support provided by DPWS and Windows Rally. The consumer scenarios show how implementing these technologies lowers product support costs by making it easy to connect devices to a wireless network.

- When support is required, technicians can reduce the time it takes to resolve customer issues due to better connectivity and monitoring.

# Conclusion

Device connectivity will usher in the next generation of connected experiences for users and provide tremendous market opportunity for device manufacturers and solution providers. For this to happen, technology simplification and adherence to Internet standards are essential. Web services on devices fulfills these requirements by providing a single end-to-end communication protocol, based on existing Internet standards. The protocol provides ubiquitous and secure connectivity and interoperability between devices, computers, and Internet services.

Microsoft provides native support for Web services on devices in Windows Vista. The Windows Rally technologies maximize your investments in Web services by providing additional capabilities to easily add your devices to the network, provide a Plug and Play experience for them, supply development APIs for your PC-side solutions, and provide management capabilities for greater support.

To take advantage of this opportunity, visit the Windows Rally Web site at http://www.microsoft.com/rally . You can attend an upcoming Windows Rally Summit to learn more about the technologies, download the SDK to evaluate the development tools, or send mail to the Windows Rally team for more information at rally@microsoft.com.

# Resources

For more information about the Microsoft licensing program for Windows Vista enterprise printers, send an e-mail to prninfo@microsoft.com.

Essential Links

- About the Windows Rally Program
http://www.microsoft.com/whdc/rally/aboutrally.mspx

- Configuration Technologies: Windows Connect Now
http://www.microsoft.com/whdc/rally/rallyWcn.mspx

- Discovery Technologies: PnP-X
http://www.microsoft.com/whdc/rally/rallyPnpx.mspx

- Web Services on Devices
http://www.microsoft.com/whdc/rally/Rallywsd.mspx

- LLTD and QoS for Media Experiences 
http://www.microsoft.com/whdc/rally/rallyLltd.mspx

- Specifications and Tools: Windows Rally
http://www.microsoft.com/whdc/rally/rallyspecs.mspx

- A Technical Introduction to the Devices Profile for Web Services [MSDN]
http://msdn.microsoft.com/library/default.asp?url=/library/en-us/dnwebsrv/html/deviceprofile-techoverview.asp

- Web Services Description Language (WSDL) 1.1
http://www.w3.org/TR/wsdl

- PnP-X: Plug and Play Extensions for Windows Specification
http://www.microsoft.com/whdc/Rally/pnpx-spec.mspx

Related Links

- Windows Driver and Hardware Central (WDHC) Web page
http://www.microsoft.com/whdc/default.mspx

- WinHEC 2007
http://www.microsoft.com/whdc/winhec/default.mspx

- Windows Logo Program and Driver Signing
http://www.microsoft.com/whdc/winlogo/default.mspx

- Windows Driver Kit (WDK)
http://www.microsoft.com/whdc/driver/WDK/default.mspx

Presentations

- Device Connectivity: Vision and Evolution [WinHEC 2006; 3.98 MB]
http://download.microsoft.com/download/5/b/9/5b97017b-e28a-4bae-ba48-174cf47d23cd/CON020_WH06.ppt

- Web Services on Devices in Windows Vista [WinHEC 2006; 1.51 MB]
http://download.microsoft.com/download/5/b/9/5b97017b-e28a-4bae-ba48-174cf47d23cd/CON105_WH06.ppt

- Web Services on Devices[WinHEC 2006; 3.09 MB]
http://download.microsoft.com/download/5/b/9/5b97017b-e28a-4bae-ba48-174cf47d23cd/CON106_WH06.PPT

- Plug and Play for Network Connected Devices [WinHEC 2006; 2.59 MB]
http://download.microsoft.com/download/5/b/9/5b97017b-e28a-4bae-ba48-174cf47d23cd/CON074_WH06.ppt#2

- How to Use Windows Connect Now Configuration to Enable Simple Setup for Consumer Wi-Fi Networks [WinHEC 2006; 5.83 MB]
http://download.microsoft.com/download/5/b/9/5b97017b-e28a-4bae-ba48-174cf47d23cd/CON044_WH06.ppt