# Procedure Microsoft Hardware

TITLE: E&D Supplier Audit Policy

Document Number: H00875 Group Ownership: E&D Quality and Compliance Distribution: Internal Only External All External Restricted (If restricted, specify restricted to whom): [Click and Type]

| Revision | Date | ECO | Description | Author / Revised by |
| --- | --- | --- | --- | --- |
| A | 10/11/02 | --- | Initial Release | Tony Anderson |
| B | 2/6/04 | C07269 | Update R & R’s, Added History Table & Distribution to Front Page, Added new assessment processes (ICA, PRA, SMA & TRA) | Raylon Hui |
| C | 3/31/05 | C09240 | Update to reflect the latest Supplier Assessment Policy and business needs. Update to reflect the latest organization and terminology used in audit. New formats of audit forms and templates. | Raylon Hui |
| D | Jun 7, 2005 | C10021 | Revise 6.2.5: Updated Restricted Substance Control Audit Template (to align with latest H00642 rev D requirements). Revise 5.3.2 to exclude Lead Free as independent audit area. Delete 6.2.6: Lead Free Audit Template | Raylon Hui |
| E | Dec 15, 2005 | C12878 | Add Supply Chain Security as the audit area. Revise the scope of application of Product Development (PD) Audit. Update the roles of Audit CA closure. Update QSP audit template (Rev B) with the addition of ESD requirement. Add “SMA audit model” as guideline for SMA frequency and scope definition. Group name update Audit Finding Report Form revision change (A to B) | Raylon Hui |
| F | Jan 26, 2006 | TBD | Addition of Product Configuration audit as one of the audit areas (section 5.3.6, 6.2.9, 6.2.10) Audit Finding Report Form revision change (B to C) – Add SCS as one of the option in audit areas QSP audit templates revision change (B to C) – Q10.2 and Q10.12 for ICA, and Q9.2 and Q9.13 for SMA SCS audit template revision change (A to B) – add overall score calculation and correlate to overall risk ranking Revise section 5.3.3 and 6.2.6: Terminology of CSR change to SCSER Revise section 5.6.2.6: corrective action metrics Revise section 6.3 “Audit Performance Driven Forecast Table” | Raylon Hui |
| G | Mar 14, 2006 | C13735 | Remove SCS audit template | Raylon Hui |
| H | Aug 2, 2006 | TBD | Unify the terminology of “Assessment” as “Audit” to ensure common understanding Update Audit Finding Report form (Rev D) (add PCA as new audit area option and “Audited Product Categories” as part of audit background info on report) Modify 5.4 “Audit Roles and Responsibilities” Modify 5.9 “Audit Findings Definition” Update 6.2.7 Integrate the 6 PD audit template files into 1 Modify 6.1 “Supplier Audit Process Flow” to better reflect the processes nowadays | Raylon Hui |
| J | Dec 6, 2006 | C17079 | 1. Updated E&D logo and group name 2. Rename ‘DPG” as “PCHW” and include “PEG” in section #2.0 (Scope) 3. Include H03562 and Vendor Code of Conduct as references (section #4.0) 4. Modify section #5.3.2 (RSC audit definition and include H03562 as one of the reference spec) 5. Delete audit area section #5.3.3 (Supply Chain Social and Environmental Responsibility) 6. Modify section #5.4.1 (Audit requester R&R) 7. Modify section #5.6.2.2 “Audit Request” 8. Modify section #5.6.2.4(d) adding the use of digital photos to supplement Critical and Major audit NCs details. 9. Add /Modify section #5.6.2.6 “Supplier Response to Audit Finding Non Conformities (all issue types)” 10. Add /Modify section #5.6.2.6.1 “Standard Issue Process - Corrective Action Tracking, Closure and Result Feedback to the Business”, and #5.6.2.6.2 “Exception Issue Process - Corrective Action Tracking, Continuous Improvement, Closure and Result Feedback” 11. Modify CA closure metrics from 20 days to 30 days for Critical NC (section #5.6.2.6.1(d)) 12. Add section #5.6.2.7 “Code Red Issue (potential Stop Production / Stop Shipment criteria)” 13. Add section #5.6.2.8 “Escalation of Persistent Open NC (for all types of NC’s)” 14. Modify section #6.1.1 “Supplier Audit Process Flowchart” (add standard NC closure metrics) 15. Add section #6.1.2 “Audit Non-Conformity Resolution Process Flowchart” 16. Modified “Audit Finding Report Form” (rev G) (section 6.2.2) 17. Added forms: section #6.2.10 “8D CAR form” (rev A), section #6.2.11 “Non-Conformity Exception CA Progress Metrics Form” 18. Delete section #6.3 (Audit Performance Driven Forecast Table) | Raylon Hui |
| K | July 26, 2007 | C20446 | Consistently define and use the term “Supplier Manager” to replace “Vendor Account Manager” in this document. Remove PD audit area and PD audit template. Update QSP and RSC audit template versions Update site links and aliases Modify “Audit Scoring Criteria” and “Supplier Performance Rating” Add Minor NC tracking and closure requirement Change the terminology from “Audit” to “Assessment” for ICA Add Audit Engagement Process with direct suppliers who subcontract work out Add compliance risk assessment process as part of the audit forecast phase of audit process Add monthly audit program metrics report requirement Add recommended auditor labor day requirement | Raylon Hui |
| L | June 2008 | C24736 | - Modified section #5.3.3 to elaborate SEA audit requirements - Addition of new audit area – Technical Capability Assessment (TCA) - Modified section #5.6.2.1b regarding rules of “Audit Forecast” projection - Modified section #5.7 to define “Audit Scoring Methodology” for various Audit Areas - Modified section #5.8 to define “Supplier Audit Performance Rating” systems for various Audit Areas - Added section #5.11 “Supplier Compliance Index (Audit)” - Modify section #5.9.1 to #5.9.4 “Audit Findings Definition”. - Removed audit escalation process section #5.6.2.7, #5.6.2.8, and flowchart #6.1.2, form #6.2.9 - Updated QSP and RSC audit templates, and addition of Packaging RSC audit template section #6.2.3 and #6.2.4 - Updated AFR form section #6.2.2 - Modified section #5.10 “Microsoft audit guidance for Fabless suppliers or suppliers who procure FPP” - Change in payment responsibility for audits section #2.0 | Raylon Hui |

PROPRIETARY NOTICE Information in this document is subject to change without notice. Companies, names, and data used in examples herein are fictitious unless otherwise noted. No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, for any purpose, without the express written permission of Microsoft Corporation. Microsoft may have patents or pending patent applications, trademarks, copyrights, or other intellectual property rights covering subject matter in this document. The furnishing of this document does not give you any license to these patents, trademarks, copyrights, or other intellectual property rights. Published by: Microsoft Corporation Home and Entertainment Division One Microsoft Way Redmond, WA 98052-6399 Telephone (425) 882-8080 © 2000 - 2003 Microsoft Corporation. All rights reserved. Microsoft, MS, MS-DOS, Windows and the Windows logo are either registered trademarks or trademarks of Microsoft Corporation in the United States and/or other countries. MICROSOFT CONFIDENTIAL

- PURPOSE

Supplier audits are conducted to identify the extent to which new and existing suppliers comply with Microsoft requirements and to identify associated risks, strengths, and opportunities that might affect the quality of Microsoft products and services. Audits may be required before supplier business award, during component / product sourcing and manufacturing stages, or when critical quality issues are reported and immediate resolutions are required.

Audits can be used by Microsoft to (a) evaluate business opportunities with a new supplier, (b) determine the capability and competency of a new or existing supplier’s quality system and processes used to manufacture and deliver  products or services, (c) determine a new or existing supplier’s product and operational compliance with Microsoft requirements, (d) provide valuable feedback on a supplier’s product and operational compliance performance during a regular supplier business review, and/or (e) effectively and systematically drive quality issue resolution.

- SCOPE AND COSTS

This procedure applies to all Microsoft Tier 1 and CPA Tier 2 hardware suppliers, managed under the Xbox Console, Xbox Peripherals, PC Hardware (PCHW), Portable Entertainment Device (PEG), and Procurement business groups of Microsoft E&D Division. These guidelines may be used for internal, second party or third party audits.

Starting July 1, 2008, costs for the audits will be paid by the suppliers.  Every 6 months, Microsoft will send an invoice to all suppliers who completed an audit in the previous 6 months. Suppliers must then send a payment to Microsoft for the full audit costs. Suppliers who perform well in one year’s audit may be allowed to conduct audits every other year, and not annually.  This decision will be made after each year’s audits, starting after the FY09 audits are complete.  [Note: audit scheduling and execution will continue to be managed by the Microsoft Audit Team].

Audits will be charged to the supplier based on the fees below:

| Audit Area | Cost (USD) |
| --- | --- |
| QSP | 1300 |
| RSC | 1000 |
| SEA ICA | 2000 |
| SEA SMA | 3000 |
| SCS | 1500 |
| PCA | 1200 |
| 1st verification audit (Corrective Action Audit) | 600 |
| 2nd verification audit (Corrective Action Audit) | 1200 |
| 3rd verification audit (Corrective Action Audit) | 2000 |

- DEFINITIONS

Audit - Systematic, independent and documented process for obtaining performance evidence, and for objectively evaluating, and determining the extent to which the audit criteria are fulfilled

Audit Criteria - A set of pre-determined requirements against which audit evidence is compared

Audit Evidence – Verifiable records, referenced facts, corroborated statements or other information relevant to the system, process, product or operation being audited. Evidence may be qualitative or quantitative

Audit Findings – The determination of the extent to which the audit criteria are fulfilled

Audit Conclusion - An audit team’s considered determination of an audit’s outcome

Audit Team - One or more auditors performing an audit.  An audit team may include other support roles such as outsourced auditors, subject matter experts (SMEs), trainees, and observers

Audit Team Leader – A qualified auditor who has been appointed to lead an audit team

Auditor - A qualified person appointed to conduct audits

Auditee - An entity or organization being audited

Audit Program – An audit or set of audits planned to achieve a specific objective within a specified timeframe

Audit Plan – A description of who, what, where, when, and how an audit will be conducted per the audit program

Audit Requester – An organization or person who initiates an audit request for a particular supplier

Audit Scope – A description of the extent and boundary of an audit. The scope typically defines the applicable audit criteria, physical audit locations, organizational coverage, audit activities, processes affected, and the audit duration

Auditor Qualification - Combination of personal attributes, education, training, work and auditing experience, criteria knowledge, and other attributes required for an auditor to be able to effectively conduct audits

E&D Quality and Compliance –The Microsoft Entertainment and Device Division Quality and Compliance group

Fabless Supplier  – a supplier that does not conduct the manufacturing themselves, but hires a third party to do the manufacturing

Non-Conformity – The failure to meet specified requirements

Observation – A comment on a situation that the audited quality or compliance system, process or operation is not optimal

Objective Evidence - Qualitative or quantitative information, records, or facts

Subject Matter Expert (SME) – An audit team member who has specific knowledge on the technology or information as needed by the system, process, product or operation being audited

Supplier Manager  – The person responsible and accountable for ensuring their managed suppliers comply with Microsoft requirements. This includes the Vendor Account Managers of Tier 1 suppliers and the Procurement Managers of Tier 2 suppliers.

- REFERENCES

- ANSI / ISO 9000:2000 Quality Management System

- H00594, H00642, H02446, H01514,  H02050, H02216, Vendor Code of Conduct

- PROCEDURE

| Responsibility | Number | Action |
| --- | --- | --- |
| General General General Auditors Auditors Auditors Auditors Auditors Supplier Manager Audit PM SQE Supplier Manager Audit PM Audit Requester Auditors Audit Program Manager Supplier Manager Supplier Auditors Auditors Auditors Auditors Auditors 3rd party auditors Audit PM Audit Manager Audit PM Audit requestor Supplier Manager Audit Requestor Audit PM Audit PM Audit Requester Audit PM Auditors Audit PM Auditors Auditors Audit PM Audit Manager Supplier Manager Subject Matter Expert (SME) Supplier Manager Audit PM Supplier Manager Other SME-as appropriate General General General | 5.1 5.2 5.2.1 5.2.2 5.2.3 5.3 5.3.1 5.3.2 5.3.3 5.3.4 5.3.5 5.3.6 5.4 5.4.1 5.4.2 5.4.3 5.4.4 5.4.5 5.5 5.5.1 5.5.1.1 5.5.1.2 5.5.1.3 5.5.1.4 5.6 5.6.1 5.6.2 5.6.3 5.6.4 5.6.5 5.6.6 5.6.7 5.7 5.7.1 5.7.2 5.8 5.8.1 5.8.2 5.8.3 5.8.4 5.8.5 5.9 5.9.1 5.9.2 5.9.3 5.9.4 | Audit Objectives Audits are generally initiated for one or more of the following purposes: - To evaluate a potential supplier where there is a desire to establish a contractual relationship. - To verify that the supplier quality or compliance system, process, product, or operation continues to meet Microsoft’s requirements and effectively be implemented, and to ensure applicable terms of the supplier’s contract with Microsoft are met. - To ensure the supplier quality or compliance system and processes are fully capable of manufacturing and shipping Microsoft products. - To ensure critical quality and compliance issues are effectively addressed and resolved. Audit Categories There are three types of audits covered by this policy: Initial Capability Assessment (ICA) a) Performed during sourcing b) Focused on identifying gaps, risks, and strengths of suppliers in complying with Microsoft requirements - Performed as a standard comprehensive assessment on various audit areas Sustaining Maintenance Audit (SMA) - Performed regularly to validate the sustaining compliance of supplier quality and product/operational compliance systems with Microsoft requirements. - Performed on an audit frequency and scope that are mainly determined and driven by the past audit results, supplier improvement progress, and identified supplier compliance risk - Used in supplier performance scorecards, business reviews and awards, sourcing decisions, etc. Tactical Resolution Audit (TRA) - Performed as needed to address specific concerns - Triggered by escalated quality or compliance issues Integrated Audit Program – Audit Areas The Integrated Audit Program is to improve supplier audit effectiveness while reducing overall cost, minimizing audit randomization, and accelerating supplier performance capability. Audit areas under the Integrated Audit Program include: Quality System and Process (QSP) - This is an audit to determine a supplier’s gaps, ability or adherence to meet Microsoft’s expectations on maintaining an effective quality management system (ISO and QS9000 based) and capable manufacturing processes run by supplier’s manufacturing facilities to supply quality products and /or services for Microsoft. - The audits are typically conducted by the Audit Team or 3rd party auditors. Restricted Substance Control (RSC) - This is an audit to ensure that the supplier is capable of producing products that adhere to Microsoft’s specifications restricting the use of certain substances in its hardware products and to ascertain gaps in the supplier management of compliance that could lead to risk for Microsoft. These substances and associated testing and disclosure requirements are cited in the following Microsoft specifications: - "Restricted Substances for Hardware Products” (H00594) - “Restricted Substance Control System” (H00642) - “Microsoft Analytical Lab Approval Process” (H02446) - “Lead Free Product Requirements for PC-Hardware and Xbox” (H01514), - The objective is to: - Avoid usage of restricted substances in the make-up or manufacture of Microsoft products. - Identify areas of risk (of containing restricted substances) and priorities - The audits are typically conducted by the Audit Team, 3rd party auditors or SME. Social and Environmental Accountability (SEA) - Microsoft has a code of conduct and requires companies to comply to certain ethical standards. - In addition to Microsoft’s VendorCode of Conduct and Social & Environmental Accountability Compliance Specification (H02050), Microsoft as a member company of EICC /GeSI, follows the audit codes established and maintained by EICC /GeSI to conduct SEA audits in various SEA aspects, including - Labor and Ethics - Occupational Health and Safety - Environment - Labor and Ethics Management System - Environmental, Health and Safety Management System - The audits are typically conducted by the 3rd party auditors. Supply Chain Security (SCS) - Authorized suppliers are responsible for all aspects of site security for the purpose of safeguarding Microsoft consigned components and/or product as well as information regarding cargo routing of shipments containing these elements. Contact with, and access to, Microsoft consigned components and/or product as well as information regarding cargo routing of shipments containing these elements by employees, temporary staff, and visitors must be strictly limited as set forth in H02216 “Microsoft Supply Chain Security Policies and Requirements”. - The audit is typically conducted by a 3rd party auditor. - A Supply Chain Security audit is applicable to all Tier 1 suppliers, and is required in the initial capability assessment (ICA) and sustaining maintenance audit (SMA). - A supplier self assessment is not required in this audit area. The audit template is not provided to suppliers based on Microsoft corporate supply chain security policy. Product Configuration Audit (PCA) The Product Configuration Audit (PCA) is required only for existing T1 suppliers and normally aligned with the annual Sustaining Maintenance Audit (SMA) schedule as one of the audit areas. PCA consists of 2 stages, namely (a) BOM to BOM comparison, and (b) Physical Configuration Audit. - BOM to BOM Comparison A BOM to BOM comparison audit is typically an off site activity to check and see if the manufacturer has the same BOM on file that Microsoft has in Agile. However, as 100% of review of all BOM documents would be time prohibitive, a sampling plan is used. For sample size recommendations, refer to section 5.3.6 (c). Normally, a Microsoft employee is the preferred person to conduct the audit. The following are the recommended steps to plan for a BOM to BOM Comparison Audit: - Once the SMA audit schedule of a supplier is fixed, request the Microsoft Supplier Manager to provide the BOM list for products manufactured by the supplier. - Randomly select sample BOMs (based on above sampling plan) from the list and request the Supplier Manager to get both Microsoft BOMs (Agile) and corresponding supplier BOMs. - With the use of the “BOM to BOM Checklist” (Appendix 6.2.5), compare info of sampled parts (based on above sampling plan) for both existence and accuracy, and complete the checklist form. - Any non-conformities shall be reported via Audit Finding Report form (see Appendix 6.2.2) and follow the normal audit process for supplier corrective actions. - Physical Configuration Audit A Physical Configuration Audit is typically an on site activity to examine each Microsoft part number (MPN) by reviewing a representative of the product and determine that the related design documentation matches the deliverables. Normally a Microsoft employee or 3rd party auditor is the appropriate person to conduct the audit. For sample size recommendations, refer to section 5.3.6 (c). In addition to Physical Configuration Audit as an on-site physical check activity, the following criteria shall be considered before deciding samples to take: - Product build plan during the days of audit that could better ensure the auditor having necessary physical materials available for audit / verification. - High risk programs, especially a new launch. The Supplier Manager is the right person to be consulted. - Programs recently having an engineering change. The Supplier Manager is the right person to be consulted. In addition, the following are the recommended steps to plan for a Physical Configuration Audit: - Once the SMA audit schedule is fixed, request the Supplier Manager to provide relevant supplier build plan for Microsoft products during the audit days. - Select sample product BOMs to audit based on above considerations (i.e. criteria (i) to (iii)). - Prepare the Physical Configuration Audit agenda and integrate with the master SMA agenda via the Audit PM. - With the use of the “Physical Configuration Audit Checklist” (Appendix 6.2.6) and based on the sampled MPN, verify the materials are actually used with correct configurations (i.e. right specification, quantity, and approved vendor list (AVL)), and complete the checklist accordingly (Hints for auditors: Walk the production lines, parts kit areas and raw material warehouse storage and examine the material tags/ labels for the selected MPNs). - Any non-conformity shall be reported via Audit Finding Report form (see Appendix 6.2.2) and follow the normal audit process for supplier corrective actions. c) Sample size requirements (for auditor reference only) - For BOM sampling – 1 to 3 BOMs to be sampled from all available BOMs (skus) of MS products in same manufacturer, depending on the time available or allowed. - Component sampling (in same BOM) – 10% of the total number of components within the same BOM, or no fewer than 100 components to be sampled from the same BOM. Auditor shall take the upper quantity whenever applicable. Technical Capability Assessment (TCA) - A TCA is a commodity specific technical quality assessment of a supplier’s capability and readiness to produce quality commodity parts for Microsoft. Each commodity category is assigned by an SQE. - A TCA is normally a part of a supplier’s ICA or SMA audit. - The SQE conducts the TCA audit at the supplier’s facility, distributes the reports, and works with the supplier to close the corrective actions. Audit Roles and Responsibilities - Audit Requester: Responsible and accountable for providing audit program details, including objectives, urgency status, product and supplier specifics, proposed schedule and scope - Audit Team: Responsible for conducting the audit and preparing the audit report. - Audit PM: Responsible and accountable for driving audit forecast, audit schedule, audit resource, ensuring the audit program is consistently and effectively followed, driving for supplier’s self assessment results and supportive documents prior to audit, preparing and communicating audit agenda, communicating audit results, providing on-site audit support (if necessary), on-going audit process and tools management and improvement, managing outsourced auditors and audit CA closure process. - Supplier Manager Responsible and accountable for site visit approvals. Contributor to scheduling and planning. Normally the Supplier Manager initiates an audit request via the supplier audit system. - Supplier: Responsible and accountable for conducting and reporting self assessment, and providing supportive documents (factory and product) prior to audit. Implements corrective actions to resolve non-compliance gaps. Principles of Auditing Effective and productive auditing is based on the principles below: - Independence: Freedom from inappropriate associations, bias, or other situations that interfere with objective audit; - Objective Evidence: The rational basis for reaching factual audit conclusions based on defined criteria; - Due Care: Reasonable and professional care taken in all matters related to the audit; - Ethical Conduct: Adherence to the established code of ethics governing audit activities (e.g. The American Society for Quality’s Code of Ethics); - Fair Presentation: Truthful, accurate, and factual reporting of specific system or operation noncompliance based on the specified requirements and/or standards. These principles, when applied, define a framework for conducting the audit. For auditors, they serve as a checklist to help assure audit integrity. - Audits are objective, systematic, and independent and produce information on which the suppliers’ management can act to improve supplier quality and compliance. - Audits have objectives – they are authorized, planned, and managed using defined methods and techniques. - Audit team members are competent, unbiased, and free from any conflict of interest. - The relationship between the stakeholders, audit team, auditee, and client is professional and ethical. - Audit conclusions and audit evidence are factual, relevant, accurate, reliable, and complete. Auditors working independently from one another shall reach similar conclusions. - An audit is conducted during a finite time period. It is not possible to assess all aspects of all operations. Audit evidence relies on a sample of information. Auditor Qualification Microsoft auditors, or delegated 3rd party auditors (subcontractors), shall possess or acquire the capabilities, awareness, and knowledge /experience to effectively perform audits: Capabilities - To determine logical audit scope, collect audit evidence, determine audit findings, and develop audit conclusions. - To communicate effectively orally and in writing: - Open Questioning Technique: - To fully understand the supplier’s systems / processes. - To verify the process owner’s understanding and compliance with process requirements. - Closed Questioning Technique: - To narrow the scope of audit - To confirm the auditee’s thinking or actions - To confirm audit findings - Report Writing Skill - Clear, accurate and factual statements - To be objective at all times - To remain focused despite distractions - To be patient and calm at all times - To be diplomatic and professional at all times - To respect the auditee’s personnel and be sensitive to cultural differences - To show appropriate empathy, but not sympathy, for the auditee’s situations - To reach unbiased conclusions Awareness - To display a quality-oriented attitude in sync with Microsoft’s values - To keep the audit within the scope and to avoid irrelevant activities or evidence - To focus on appropriate procedures and practices to be audited - To manage time effectively - To be alert to sensitive or confidential evidence f) To immediately communicate with the audit team, the Audit PM, and /or the Supplier Manager (or as appropriate the auditee) on critical or abnormal issues that must be instantly addressed (e.g. Critical Tolerance non-compliance, lack of support from auditee or supplier’s management). Knowledge and Expertise - Basic technical knowledge at a general level in engineering, manufacturing, business operation, quality engineering and management, particularly in computer peripheral or electronics manufacturing areas, (including but not limited to): - Critical process / product parameters used in : - SMT process - PCB assembly process - Component manufacturing process and testing (e.g. cable, connector, DC fan, power supply, etc.) - Box build assembly and testing. - Process control concept and tools application (e.g. Control Chart application in Statistical Process Control). - Restricted Substance Control system requirements (e.g. RS compliance traceability) External Audit support - If subcontracted audit work is necessary, all 3rd party auditors shall be appointed and qualified by MS Audit PM before the subcontractor is authorized to conduct audits for MS. - Subject Matter Experts (SMEs) from MS functional groups shall provide appropriate level of support and contributions to the audit programs that require their expertise. Audit Process A detailed audit process flowchart can be found in appendix (item 6.1). Generally, an audit is divided into 6 phases: Audit Forecast, Audit Request, Audit Planning, Conducting the Audit, Audit Reporting, and Corrective Action Tracking and Closure. Audit Forecast - For initial capability assessment of new Tier 2 suppliers, relevant new audit requests as well as the annual projected audit forecast collected from the Supplier Manager, are risk assessed, before logged down for continuous tracking and scheduling by the Audit PM. For Tier 1 suppliers, ICAs always occur. - For sustaining maintenance audit of existing suppliers, the Audit PM is responsible for projecting relevant audit forecast in the following fiscal year, based on the supplier’s risk, past audit performance, and improvement. The prepared SMA audit forecast is then sent to relevant Supplier Manager for review and approval. The Audit PM maintains and updates the master audit forecast for subsequent audit planning and scheduling. (Remarks: A well performing supplier in the previous audit may have relevant audit areas skipped for the following year. However, this rule does NOT apply to Tier 1 suppliers as well as those defined “High Risk” Tier 2 suppliers) - Monthly e-mail communication of the upcoming months’ audit forecast and schedule shall be sent by the Audit PM to the Supplier Manager. Audit Request - Audit requests originate by either: - An ICA which is triggered by a RFQ/RFP and final candidate down-selection. Assessments occur on or before final negotiation, OR - An SMA whose frequency is determined based on past audit performance, corrective action status, and any identified supplier compliance risks, OR - A TRA which is triggered by a critical quality or noncompliance issue in a supplier that requires an audit to identify the gaps and drive for resolution. - Who is audited? - MS conducts audits for suppliers for which we have contracts. For all others we rely on the T1, who has the contract with them, to conduct the audit. - Who requests an audit? - Supplier Manager - Procurement Managers (CPA) - The quality and/or compliance organization if deemed an audit is needed - Dev community if deemed an audit is needed Audit Planning - The Audit PM requests the supplier to provide the factory background, and product and process specific documents (e.g. Product spec and MCP), via the “Supplier Factory Survey Form” (see Appendix 6.2.1), in time for auditor review at least one week prior to the audit. - Based on the agreed audit scope, relevant audit templates are prepared and sent to the supplier for self assessment by Audit PM. All supplier self assessment reports are completed and submitted to the Audit PM /Auditors for review at least one week prior to the audit. Conducting the Audit - At least one auditor within the audit team shall be knowledgeable about each audit discipline. - Auditors shall always fulfill the “Auditor Qualification” requirements as mentioned on section 5.5. - Prior to conducting the audit, each auditor shall be familiar with the audit requirements, including the audit objectives, audit templates and criteria. They shall understand the supplier’s past performance by reviewing previous audit findings and any corrective actions taken. - Auditors will conduct on-site audit according to the applicable audit template criteria. They shall capture accurate audit evidence, and assess and evaluate the risks and impacts resulting from the observed non-conformities by “Category” in accordance with the definitions provided by Microsoft. All categorized “Critical” and “Major” non-conformities should have relevant digital photos to support and detail the non-conformities (In order to maintain the audit report having reasonable file size, the recommended photo resolution is around 1152X864, or photo size around 100-200K bytes per photo). Relevant photos must be treated as Microsoft Confidential information by all parties. Audit Reporting - All audit results, including audit scores and detailed non-conformity descriptions shall be recorded by the auditors in respective “Scorecards” (i.e. Audit templates) and “Audit Finding Report“(Audit Findings Report Form) respectively. - All audit reports shall be sent to the Audit PM for review. - The Audit PM shall communicate audit results to the Supplier Manager and Supplier. - Audit program executive summary reports are sent monthly by the Audit Manager to the directors and group managers of supplier management. Reported monthly audit program metrics include the monthly audit results and supplier corrective action progress (CA closure). Supplier Response to Audit Non-Conformities (all issue types) - Refer to #6.1.1 “Supplier Audit Process Flowchart”. - For all non-conformities identified in an audit, the supplier shall identify the root cause, determine and begin implementing effective corrective actions to become fully compliant with Microsoft requirements. All audit non-conformities (NC) and corrective actions (CA) shall be documented in the “Audit Finding Report” format (Audit Finding Report Form) and sent to the Audit PM and Supplier Manager for review. - For each Critical and Major NC, relevant CA shall be addressed and responded using the “8D CAR Form” (see appendix 6.2.7) to address and provide a comprehensive improvement plan. Relevant 8D CAR forms shall be attached as supplement to the defined column of Audit Finding Report Form. - If an issue requires significant resources and time to implement, or is a significant challenge to address, the “8D CAR Form” should specifically state key objectives and milestones for completion that define the continuous improvement plan to ultimately become fully compliant with Microsoft Requirements. - The Audit PM (or Microsoft Compliance Team member) shall review the Corrective Action Plan with auditors, as necessary, and - Consult appropriate Microsoft SME, to “approve” any “Exception Issue” solution processes: - Enter a Product Studio bug to track and manage progress and closure of non-conformity. - In the AFR under Microsoft Responsibility, enter Bug # and set “Closure Type” = “Exception, <BUG #>”. - Section 5.6.2.6.2 then applies. - Follow the Standard Issue Process (Section 5.6.2.6.1) for any remaining non-conformities Standard Issue Process - Corrective Action Tracking, Closure and Result Feedback to the Business - Refer to #6.1.1 “Supplier Audit Process Flowchart” - The Audit PM is responsible and accountable for scheduling follow up audits to verify the Corrective Actions taken by the suppliers. Verification is normally conducted via on-site verification, but could be via document review, or tele-conference call if considered sufficient for verification and closure. Outsourced auditor resources may be used as deemed necessary. - The Audit PM continuously tracks and validates the corrective action closure status of all audit non-conformities, based on the defined closure metrics for different non-conformity categories. - Metrics for tracking and driving CA closure: - Target timeframe to close “Critical” NC: <30 days from the audit report issue date. - Target timeframe to close “Major” NC: < 60 days from the audit report issue date. - Target timeframe to close “Minor” NC: < 90 days from the audit report issue date. - Microsoft allows a maximum of 2 corrective action verification audits. Suppliers failing to close all “open” NCs are quarantined and escalated to the Microsoft SME and Supplier Manager for follow up. - Audit result and corrective action closure both contribute to be part of the supplier compliance performance in the regular Supplier Business Review event. - Audit process of Supply Chain Security audit, including the non-conformity definition, reporting, corrective action tracking and closure requirements are different and are defined in separate audit process document maintained by the Microsoft Corporate Supply Chain Security Group. Exception Issues An “Exception” may occur if MS agrees, due to business requirements, to treat an NC as an exception. These are only allowed with approval from Microsoft. Continuous Improvement Issues An issue may be placed in the Continuous Improvement Process (CIP), only with Microsoft approval. In this situation, the - Supplier focuses on the discovery of the root cause issue, implementation of remediation solutions and establishing targets and milestones that define the progressive steps to achieve a sustainable compliance solution. - Supplier communicates the status to the commitments (targets & milestones) of their CA Plan via monthly (or other approved frequency of reporting) through the CIP progress template by sending these to the Audit team alias (audit_t@microsoft.com) and the SME, if appropriate. - The Audit PM (or delegate) may verify the CIP progress made by the suppliers through a Validation audit. Verification may be conducted via on-site verification, document review, or tele-conference if considered sufficient. Outsourced auditor resources may be used as deemed necessary. - When the CIP is fully completed, the Audit PM will verify this via a Validation Audit, call or document review. Code Red Issue (Potential Stop Production / Stop Shipment) A Code Red can be issued for 2 reasons: 1. Identification of a Critical or Zero Tolerance Issue – these are identified within the audit process and are issues that are weighted the most due to their importance. The Supplier will be notified of these issues immediately when noted during the audit. l 2. Failed Audit: When a supplier scores below the acceptable criteria outlined in Section 5.9.4. Both of the above lead to escalation within Microsoft and within the Supplier’s Management. Immediate action is needed from the supplier at this time. - A stop production or stop ship decision may result from this process until the Critical issues or failed audit has been addressed.. Audit Scoring Methodology Audit scoring either follows a “5 levels of compliance” scenario, or follows a “Yes /No” scenario scoring system, among different audit areas. Quality System and Process (QSP) and Restricted Substance Control (RSC) Scoring criteria of each audit question is generally specified in 5 levels of compliance (scores from “1” showing “Needs Improvement High” to “5” showing “Exceeding Microsoft Requirement”) to help auditors score properly. Each score (compliance level) is written with a specific description of supplier performance expectation that enables the supplier to attain that score. This process helps reduc auditor variation and promotes performance consistency. Microsoft “minimum acceptable standard” is a score of “4” meaning “Meets Microsoft Requirement”. (Each audit question may also have the “Not Applicable” (NA) score possibility when a particular requirement does not apply to a certain business model, non-related product category or manufacturing process, or because no information is available for audit due to an audit in a very early project phase). Each audit question is color coded based on their assigned weightings developed from their relative importance (values) and risks towards Microsoft. These color coding definitions assist auditors to justify the classification of non-conformities when they find violations to audit questions with different color coding. Refer to the attached audit templates in section #6.2.3 and #6.2.4 for details. Social and Environmental Accountability (SEA), Supply Chain Security (SCS), Product Configuration Audit (PCA) and Technical Capability Assessment (TCA) “Yes /No” or “Meet requirement /Not Meet requirement” scenarios apply to these audit areas, and the overall audit results are determined based on the number of “Yes” (“Meet requirement”) and “No” (“Not Meet requirement”) gained from all audit questions, Supplier Audit Performance Rating Among the various audit areas, different supplier audit performance rating systems shall apply based on the business needs. Quality System and Process (QSP) and Restricted Substance Control (RSC) Supplier audit performance rating (i.e. the overall audit result) is evaluated based on a percentage of the total possible score (compliance percentage), and the number and level of non-conformities in the audit. The overall rating of an audit shall be displayed in the “Audit Result Summary” page of the audit template (Remarks: See the audit templates for how the individual questions are weighted: red has the highest weight (making red issues “critical” issues), purple a medium weight, and yellow the lightest weight). Overall audit results generally fall into one of the 3 categories Pass (Green): 0 Red Nonconformity, AND <=5 Purple Nonconformity, AND Compliance Percentage > 75% Conditional (Yellow): 0 Red Nonconformity, AND >5 Purple Nonconformity OR Compliance Percentage >60% and <75% Fail (Red): >1 Red Nonconformity OR Compliance Percentage <60% Refer to the attached audit templates in section #6.2.3 and #6.2.4 for details. Social and Environmental Accountability (SEA) – EICC audit template The supplier audit performance rating (i.e. the overall audit result) is a consideration of critical, major and minor non conformances as reflected in the Audit Finding Reports (AFRs). Priority action should follow the sequence of critical, major, and then minor. The final score (i.e. overall audit result) at the back of the Audit Questionnaire is used as reference by Microsoft. An overall supplier social and environmental performance assessment is calculated separately and considers the critical, major and minor findings as an input together with an assessment of Management System capabilities. Audit template format is NOT provided in this document due to confidentiality reason. Supply Chain Security (SCS) Supplier audit performance rating is based on a percentage of the total possible score. Suppliers will fall into one of the 3 Compliance Risk categories Urgent – <55% High – 56% to 65% Medium – 66% to 75% Low - >75% Audit template format is NOT provided in this document in order to align with the Microsoft Corporate Supply Chain Security Policy requirement. Product Configuration Audit (PCA) Supplier audit performance rating is based on a percentage of the total possible score. Suppliers will fall into one of the 3 categories Pass – 85% to 100% Conditional – 75% to 84% Fail – 0% to 74% Technical Capability Assessment (TCA) Supplier audit performance rating is based on a percentage of the total possible score. 0% is assigned as the overall audit score if any non-compliant (i.e. a “No”) “must have” issues are found during the audit. Suppliers will fall into one of the 2 categories Pass – 85% to 100% Fail – 0% to 84% Audit Findings Definitions (Generic) A Critical Tolerance Non-Conformity is: - The violation of the high weighed (marked “red”) questions, which is the failure to meet a critical requirement which Microsoft does not tolerate non-compliance. These are the audit elements which Microsoft require compliance if the supplier is to establish or maintain business relationship with Microsoft. These issues require the in-advance agreement between the auditors and Microsoft Audit PM (or Supplier Manager) before the audit closing meeting. A Major Non-Conformity is, and can be a combination of: - The violation of the medium weighed (marked “purple”) questions, which causes or potentially causes significant impact towards Microsoft (e.g. probable shipment of a non-conforming product, a condition that may result in failure, or materially reduce the usability of the products or services for their intended purpose, etc.). OR, - The absence or breakdown of a system to meet Microsoft’s regulatory stated requirements. (Hints: >50% violation of the samples can call for a major non-conformity. >90% violation of the samples can call for a critical non-conformity as necessary). OR, - A number of minor non-conformities against one requirement (which when combined can represent the breakdown of a management system) can be considered a major non-conformity). A Minor Non-Conformity is, and can be a combination of: - The violation of the low weighed (marked “yellow”) questions, which normally reflects the gaps of the management system or processes (e.g. unclear or non-specific definition of procedural requirements, etc.), or ineffective execution of a management system (e.g. inadequate operator training, inaccessible work instruction, etc.), OR - NOT causing significant impact towards Microsoft (e.g. probable shipment of a non-conforming product, a condition that may result in failure, or materially reduce the usability of the products or services for their intended purpose, etc.). An Observation is NOT a violation of Microsoft or regulatory requirements. It is a recommendation for supplier’s continuous improvement. |
| Audit PM Supplier Supplier Manager General | 5.10 5.10.1 5.10.2 5.10.3 5.10.4 5.10.5 5.10.6 5.11 | Microsoft Audit Guidance for Fabless Suppliers and Suppliers Who Procure FPP # Purpose Microsoft’s policy is that all Microsoft direct suppliers manage and audit, when necessary, their supply base to ensure high quality and compliant components are used in Microsoft products. This section sets expectations and requirements for Microsoft fabless suppliers (suppliers who do not conduct the manufacturing themselves, but hire a 3rd party to conduct the manufacturing) and suppliers who procure FPP. # Scope This procedure applies to all Microsoft E&D hardware product and component direct suppliers (e.g., suppliers who are under contract to Microsoft). # Policy and Requirements The quality and compliance programs that this audit specification supports are outlined in the individual Quality and Compliance specifications, which are listed in the table in section #6.2.10. The individual quality and compliance specs that support these programs are also attached to Microsoft’s direct suppliers’ contracts. # Procedure Microsoft fabless suppliers and suppliers who procure FPP for Microsoft, who conduct an audit on these facilities, need to meet the following requirements: - The Microsoft direct supplier must conduct the audit of their sub-tier supplier(s) using the Microsoft audit templates. These templates are provided in the appendices . - The Microsoft direct supplier must conduct the audit using a Microsoft approved 3rd party audit company. Current approved 3rd party audit companies are: - Intertek Testing Services Shenzhen Limited - Contact (Person name /Title /e-mail address): - K.Y. Cheung / Senior CSR Manager / Kamyuen.Cheung@intertek.com - Janet Kan /Customer Service Manager / Janet.Kan@intertek.com - Address: 5/F, M-Space, Bldg. A, Nanhai Ave. South, Shekou, Nanshan District, Shenzhen, China, 518067 - Website: http://www.intertek-labtest.com - The audit results will be shared with Microsoft, by the Microsoft direct supplier, right after the audit occurs. The Microsoft direct suppliers have the overall ownership and responsibility for auditing their suppliers, including program planning, scheduling, payment, logistics arrangement, communication, audit work execution, audit result reporting, and audit non-conformity closure, while keeping the Microsoft Supplier Manager and Microsoft Audit Program Manager (audit_t@microsoft.com ) informed (including audit result sharing). # Reporting The Microsoft direct supplier must submit the status, the Audit Finding Report, and progress of issue closure to the Microsoft Supplier Manager and Microsoft Audit Program Manager (audit_t@microsoft.com ). Microsoft must be informed when all issues noted in the audit have been closed. # Microsoft contacts for audit support For any questions of this audit process, please e-mail Microsoft E&D audit team at audit_t@microsoft.com for assistance. For questions specific to any of the Microsoft quality or compliance programs, please see the following: - For questions on the Microsoft environmental compliance program, you can email ecteam@microsoft.com - For questions specific to the Microsoft quality program, you can email marklink@microsoft.com or hotsang@microsoft.com. - For questions on the Microsoft SEA program, you can email dharder@microsoft.com or lichong@microsoft.com. Supplier Compliance Index (Audit) – SCI Supplier’s Compliance Index (Audit) of each supplier, via their respective initial /annual (ICA /SMA) audit results as well as their follow on improvement performance, is continuously tracked and reported by Microsoft Audit Team, and used in the regular supplier performance and business review. A supplier’s SCI (Audit) is generally contributed by: - The initial audit score (percentage) - The Number and Level of Non-Conformities (NCs) found in the initial audit - The Number of NCs that can be closed in a CA verification audit - The Number of CA verification audits required to close all NCs A supplier’s SCI (Audit) generally falls into one of below 4 ranking: - 5 – Exceeds Compliance Standards - 4 – Above Average Compliance - 3 – Acceptable Compliance - 2 –Deficient Compliance - 1 – Defective Compliance |

6.0	APPENDIX

6.1	Supplier Audit Process Flowchart

  - Audit Forms and Templates

    - Supplier Factory Survey Form

    - Audit Findings Report Form

    - Audit Template (Quality System and Processes)

    - Audit Template (Restricted Substance Control)

    - Audit Template (BOM to BOM Checklist)

    - Audit Template (Physical Configuration Audit Checklist)

    - Audit Template (Technical Capability Assessment)

Each commodity category has its own audit checklist, which are released via Agile.  Please contact the SQE for your commodity type for an example.

    - Audit Template (Social and Environmental Accountability - EICC) and related audit documents

Not provided in this specification, as the templates are industry templates, owned by the EICC.

    - 8D CAR Form

    - Regulatory specification matrix for Microsoft auditors

    - Auditor labor day requirement guideline

| Auditor Labor day requirement |  |
| --- | --- |
| Quality & Compliance Audit Area | Audit Prep and Labor Days |
| Quality System and Processes (QSP) | 3 |
| Restricted Substance Control (RSC) | 2 |
| Supply Chain Security (SCS) | 2 |
| Product Configuration Audit (PCA) | 2 |
| Social and Environmental Accountability – Facility with <1000 employees. ICA only | 3 |
| Social and Environmental Accountability – Facility with >1000 and <8000 employees. ICA only | 4 |
| Social and Environmental Accountability – Facility with >8,000 employees. ICA only | 6 |
| Social and Environmental Accountability – Facility with <1000 employees. SMA only | 5 |
| Social and Environmental Accountability – Facility with >1000 and <8000 employees. SMA only | 6 |
| Social and Environmental Accountability – Facility with >8,000 employees. SMA only | 8 |

* SEA Sustaining Maintenance Audits shall be conducted using the EICC SEA audit template. SEA Initial Capability Assessments shall be conducted using the SEA ICA template.